| Project | IEEE 802.21 MIHO <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | SPARQL query example and usage of RDF schema |  |
| Date Submitted | September 20th, 2006 |  |
| Source(s) | Kenichi Taniuchi, Yoshihiro Ohba, Subir Das |  |
| Re: | 21-06-0728-01-0000-SPARQL_query_example |  |
| Abstract | This document contains SPARQL query example and usage of RDF schema. |  |
| Purpose |  |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.21. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

# Introduction

## This document contains example of SARQL [SPARQL] query and usage of RDF [RDF] basic schema [RDFS] .

# RDF example

This RDF example is created based on the example network in Appendix of 21-06-0674-02-0000-IE_TLV_Representation. The format of the RDF is Turtle [Turtle]. Turtle is an extension of Notation 3 (N3) [N3].  Turtle is a language which is a compact and readable syntax describing only RDF graphs.

@prefix : <http://www.mih.org/2006/09/rdf-basic-schema#> .

<http://informationserver/network1>     :cost  [

:currency "USD";

:service-info "1";

:unit "2";

:value "5" ];

:network-type "23";

:operator-identifier  [

:operator-name "red-mobile";

:operator-namespace "0" ];

:poa  [

:poa-address  [

:address "aaaaaaaaaaaa";

:address-type "6" ];

:poa-capabilities "004a";

:poa-channel-range  [

:high-channel-range "470";

:low-channel-range "450" ];

:poa-data-rate "50";

:poa-location  [

:geospatial-location-information  [

:altitude "0f00";

:altitude-resolution "30";

:altitude-type "1";

:datum "1";

:latitude "04dcc1fc8";

:latitude-resolution "21";

:longitude "f65ecf031";

:longitude-resolution "20" ];

:method "0" ];

:poa-mac-type "CSMA-CA";

:poa-subnet-information  [

:prefix-length "16";

:subnet-address  [

:address "c0230000";

:address-type "1" ] ] ],

[

:poa-address  [

:address "bbbbbbbbbbbb";

:address-type "6" ];

:poa-capabilities "004c";

:poa-channel-range  [

:high-channel-range "430";

:low-channel-range "410" ];

:poa-data-rate "50";

:poa-location  [

:geospatial-location-information  [

:altitude "0f00";

:altitude-resolution "30";

:altitude-type "1";

:datum "1";

:latitude "04dcc1fcf";

:latitude-resolution "21";

:longitude "f65ecf030";

:longitude-resolution "20" ];

:method "0" ];

:poa-mac-type "";

:poa-subnet-information  [

:prefix-length "16";

:subnet-address  [

:address "c01f0000";

:address-type "1" ] ] ];

:qos "class1/class2/class3";

:roaming-partner  [

:operator-name "red-partner1";

:operator-namespace "0" ],

[

:operator-name "red-partner2";

:operator-namespace "0" ];

:security "strong-cipher" .

<http://informationserver/network2>     :cost  [

:currency "USD";

:service-info "1";

:unit "2";

:value "3" ];

:network-type "27";

:operator-identifier  [

:operator-name "red-mobile";

:operator-namespace "0" ];

:poa  [

:poa-address  [

:address "cccccccccccc";

:address-type "6" ];

:poa-capabilities "00b7";

:poa-channel-range  [

:high-channel-range "470";

:low-channel-range "450" ];

:poa-data-rate "80";

:poa-location  [

:geospatial-location-information  [

:altitude "6700";

:altitude-resolution "30";

:altitude-type "2";

:datum "1";

:latitude "053c1f751";

:latitude-resolution "21";

:longitude "f50ba5b97";

:longitude-resolution "20" ];

:method "0" ];

:poa-mac-type "802.16";

:poa-subnet-information  [

:prefix-length "16";

:subnet-address  [

:address "c0c90000";

:address-type "1" ] ] ];

:qos "class1";

:security "strong-cipher" .

<http://informationserver/network3>     :cost  [

:currency "USD";

:service-info "1";

:unit "2";

:value "5" ];

:network-type "27";

:operator-identifier  [

:operator-name "blue-mobile";

:operator-namespace "0" ];

:poa  [

:poa-address  [

:address "dddddddddddd";

:address-type "6" ];

:poa-capabilities "00b1";

:poa-channel-range  [

:high-channel-range "470";

:low-channel-range "450" ];

:poa-data-rate "95";

:poa-location  [

:geospatial-location-information  [

:altitude "6800";

:altitude-resolution "30";

:altitude-type "2";

:datum "1";

:latitude "053c1f751";

:latitude-resolution "21";

:longitude "f50ba5b97";

:longitude-resolution "20" ];

:method "0" ];

:poa-mac-type "802.16";

:poa-subnet-information  [

:prefix-length "48";

:subnet-address  [

:address "2002522d94d100000000000000000000";

:address-type "2" ] ] ];

:qos "class1/class2";

:roaming-partner  [

:operator-name "blue-partner1";

:operator-namespace "0" ];

:security "open" .

Figure 1 - Graphic representation of example network (Network 1)

Figure 2 - Graphic representation of example network (Network 2 and 3)

# SPARQL query example

There are 5 example queries which are same queries as Appendix of 21-06-0674-02-0000-IE_TLV_repsesentation.

## Example query 1

IS client is asking for all neighboring network information.

The IS client sends an IS Query Parameters TLV which includes SPARQL query as an Information Request..

The actual SPARQL query is below.

DESCRIBE ?x

WHERE {?x ?y ?z}

IS server sends back an IS Query Response TLV which also includes SPARQL query response as an Information Response.

The SPARQL query response is below. The data format is Turtle.

@prefix : <http://www.mih.org/2006/09/rdf-basic-schema#> .

<http://informationserver/network1>     :cost  [

:currency "USD";

:service-info "1";

:unit "2";

:value "5" ];

:network-type "23";

:operator-identifier  [

:operator-name "red-mobile";

:operator-namespace "0" ];

:poa  [

:poa-address  [

:address "aaaaaaaaaaaa";

:address-type "6" ];

:poa-capabilities "004a";

:poa-channel-range  [

:high-channel-range "470";

:low-channel-range "450" ];

:poa-data-rate "50";

:poa-location  [

:geospatial-location-information  [

:altitude "0f00";

:altitude-resolution "30";

:altitude-type "1";

:datum "1";

:latitude "04dcc1fc8";

:latitude-resolution "21";

:longitude "f65ecf031";

:longitude-resolution "20" ];

:method "0" ];

:poa-mac-type "CSMA-CA";

:poa-subnet-information  [

:prefix-length "16";

:subnet-address  [

:address "c0230000";

:address-type "1" ] ] ],

[

:poa-address  [

:address "bbbbbbbbbbbb";

:address-type "6" ];

:poa-capabilities "004c";

:poa-channel-range  [

:high-channel-range "430";

:low-channel-range "410" ];

:poa-data-rate "50";

:poa-location  [

:geospatial-location-information  [

:altitude "0f00";

:altitude-resolution "30";

:altitude-type "1";

:datum "1";

:latitude "04dcc1fcf";

:latitude-resolution "21";

:longitude "f65ecf030";

:longitude-resolution "20" ];

:method "0" ];

:poa-mac-type "";

:poa-subnet-information  [

:prefix-length "16";

:subnet-address  [

:address "c01f0000";

:address-type "1" ] ] ];

:qos "class1/class2/class3";

:roaming-partner  [

:operator-name "red-partner1";

:operator-namespace "0" ],

[

:operator-name "red-partner2";

:operator-namespace "0" ];

:security "strong-cipher" .

<http://informationserver/network2>     :cost  [

:currency "USD";

:service-info "1";

:unit "2";

:value "3" ];

:network-type "27";

:operator-identifier  [

:operator-name "red-mobile";

:operator-namespace "0" ];

:poa  [

:poa-address  [

:address "cccccccccccc";

:address-type "6" ];

:poa-capabilities "00b7";

:poa-channel-range  [

:high-channel-range "470";

:low-channel-range "450" ];

:poa-data-rate "80";

:poa-location  [

:geospatial-location-information  [

:altitude "6700";

:altitude-resolution "30";

:altitude-type "2";

:datum "1";

:latitude "053c1f751";

:latitude-resolution "21";

:longitude "f50ba5b97";

:longitude-resolution "20" ];

:method "0" ];

:poa-mac-type "802.16";

:poa-subnet-information  [

:prefix-length "16";

:subnet-address  [

:address "c0c90000";

:address-type "1" ] ] ];

:qos "class1";

:security "strong-cipher" .

<http://informationserver/network3>     :cost  [

:currency "USD";

:service-info "1";

:unit "2";

:value "5" ];

:network-type "27";

:operator-identifier  [

:operator-name "blue-mobile";

:operator-namespace "0" ];

:poa  [

:poa-address  [

:address "dddddddddddd";

:address-type "6" ];

:poa-capabilities "00b1";

:poa-channel-range  [

:high-channel-range "470";

:low-channel-range "450" ];

:poa-data-rate "95";

:poa-location  [

:geospatial-location-information  [

:altitude "6800";

:altitude-resolution "30";

:altitude-type "2";

:datum "1";

:latitude "053c1f751";

:latitude-resolution "21";

:longitude "f50ba5b97";

:longitude-resolution "20" ];

:method "0" ];

:poa-mac-type "802.16";

:poa-subnet-information  [

:prefix-length "48";

:subnet-address  [

:address "2002522d94d100000000000000000000";

:address-type "2" ] ] ];

:qos "class1/class2";

:roaming-partner  [

:operator-name "blue-partner1";

:operator-namespace "0" ];

:security "open" .

## Example query 2

IS client is asking for only UMTS neighboring network information.

SPARQL query:

PREFIX mihbasic: <http://www.mih.org/2006/09/rdf-basic-schema#>

DESCRIBE ?x

WHERE {?x mihbasic:network-type "23"}

SPARQL query response (Turtle format):

@prefix : <http://www.mih.org/2006/09/rdf-basic-schema#> .

<http://informationserver/network1>     :cost  [

:currency "USD";

:service-info "1";

:unit "2";

:value "5" ];

:network-type "23";

:operator-identifier  [

:operator-name "red-mobile";

:operator-namespace "0" ];

:poa  [

:poa-address  [

:address "aaaaaaaaaaaa";

:address-type "6" ];

:poa-capabilities "004a";

:poa-channel-range  [

:high-channel-range "470";

:low-channel-range "450" ];

:poa-data-rate "50";

:poa-location  [

:geospatial-location-information  [

:altitude "0f00";

:altitude-resolution "30";

:altitude-type "1";

:datum "1";

:latitude "04dcc1fc8";

:latitude-resolution "21";

:longitude "f65ecf031";

:longitude-resolution "20" ];

:method "0" ];

:poa-mac-type "CSMA-CA";

:poa-subnet-information  [

:prefix-length "16";

:subnet-address  [

:address "c0230000";

:address-type "1" ] ] ],

[

:poa-address  [

:address "bbbbbbbbbbbb";

:address-type "6" ];

:poa-capabilities "004c";

:poa-channel-range  [

:high-channel-range "430";

:low-channel-range "410" ];

:poa-data-rate "50";

:poa-location  [

:geospatial-location-information  [

:altitude "0f00";

:altitude-resolution "30";

:altitude-type "1";

:datum "1";

:latitude "04dcc1fcf";

:latitude-resolution "21";

:longitude "f65ecf030";

:longitude-resolution "20" ];

:method "0" ];

:poa-mac-type "";

:poa-subnet-information  [

:prefix-length "16";

:subnet-address  [

:address "c01f0000";

:address-type "1" ] ] ];

:qos "class1/class2/class3";

:roaming-partner  [

:operator-name "red-partner1";

:operator-namespace "0" ],

[

:operator-name "red-partner2";

:operator-namespace "0" ];

:security "strong-cipher" .

## Example query 3

IS client is asking for only UMTS neighboring network information and only wanting the return of “General Network” information.

SPARQL query:

PREFIX mihbasic: <http://www.mih.org/2006/09/rdf-basic-schema#>

SELECT ?opid ?partners ?currency ?unit ?value ?servinfo ?security ?qos

WHERE {?x1 mihbasic:network-type "23" .

?x1 mihbasic:operator-identifier ?x2 .

?x2 mihbasic:operator-name ?opid .

?x1 mihbasic:roaming-partner ?x3 .

?x3 mihbasic:operator-name ?partners .

?x1 mihbasic:cost ?x4 .

?x4 mihbasic:currency ?currency .

?x4 mihbasic:unit ?unit .

?x4 mihbasic:value ?value .

?x4 mihbasic:service-info ?servinfo .

?x1 mihbasic:security ?security .

?x1 mihbasic:qos ?qos }

SPARQL query response (XML format) [RESULT]:

<?xml version="1.0"?>

<sparql

xmlns:rdf="http://www.w3.org/1999/02/22-rdf-syntax-ns#"

xmlns:xs="http://www.w3.org/2001/XMLSchema#"

xmlns="http://www.w3.org/2005/sparql-results#" >

<head>

<variable name="opid"/>

<variable name="partners"/>

<variable name="currency"/>

<variable name="unit"/>

<variable name="value"/>

<variable name="servinfo"/>

<variable name="security"/>

<variable name="qos"/>

</head>

<results ordered="false" distinct="false">

<result>

<binding name="opid">

<literal>red-mobile</literal>

</binding>

<binding name="partners">

<literal>red-partner1</literal>

</binding>

<binding name="currency">

<literal>USD</literal>

</binding>

<binding name="unit">

<literal>2</literal>

</binding>

<binding name="value">

<literal>5</literal>

</binding>

<binding name="servinfo">

<literal>1</literal>

</binding>

<binding name="security">

<literal>strong-cipher</literal>

</binding>

<binding name="qos">

<literal>class1/class2/class3</literal>

</binding>

</result>

<result>

<binding name="opid">

<literal>red-mobile</literal>

</binding>

<binding name="partners">

<literal>red-partner2</literal>

</binding>

<binding name="currency">

<literal>USD</literal>

</binding>

<binding name="unit">

<literal>2</literal>

</binding>

<binding name="value">

<literal>5</literal>

</binding>

<binding name="servinfo">

<literal>1</literal>

</binding>

<binding name="security">

<literal>strong-cipher</literal>

</binding>

<binding name="qos">

<literal>class1/class2/class3</literal>

</binding>

</result>

</results>

</sparql>

| opid | partners | currency | unit | value | servinfo | security | qos |
| --- | --- | --- | --- | --- | --- | --- | --- |
| red-mobile | red-partner1 | USD | 2 | 5 | 1 | strong-cipher | class1/class2/class3 |
| red-mobile | red-partner2 | USD | 2 | 5 | 1 | strong-cipher | class1/class2/class3 |

## Example query 4

IS client is asking for PoA information owned by operator “blue-mobile”.

SPARQL query:

PREFIX mihbasic: <http://www.mih.org/2006/09/rdf-basic-schema#>

DESCRIBE ?y

WHERE {?x mihbasic:poa ?y .

?x mihbasic:operator-identifier ?z .

?z mihbasic:operator-name "blue-mobile"}

SPARQL query response (Turtle format):

@prefix : <http://www.mih.org/2006/09/rdf-basic-schema#> .

[      :poa-address  [

:address "dddddddddddd";

:address-type "6" ];

:poa-capabilities "00b1";

:poa-channel-range  [

:high-channel-range "470";

:low-channel-range "450" ];

:poa-data-rate "95";

:poa-location  [

:geospatial-location-information  [

:altitude "6800";

:altitude-resolution "30";

:altitude-type "2";

:datum "1";

:latitude "053c1f751";

:latitude-resolution "21";

:longitude "f50ba5b97";

:longitude-resolution "20" ];

:method "0" ];

:poa-mac-type "802.16";

:poa-subnet-information  [

:prefix-length "48";

:subnet-address  [

:address "2002522d94d100000000000000000000";

:address-type "2" ] ] ].

## Example query 5

IS client is asking for only subnet information of PoAs of operator “red-mobile”.

SPARQL query:

PREFIX mihbasic: <http://www.mih.org/2006/09/rdf-basic-schema#>

SELECT ?prefix_length ?type ?address

WHERE {?x1 mihbasic:operator-identifier ?x2 .

?x2 mihbasic:operator-name "red-mobile" .

?x1 mihbasic:poa ?x3 .

?x3 mihbasic:poa-subnet-information ?x4 .

?x4 mihbasic:prefix-length ?prefix_length .

?x4 mihbasic:subnet-address ?x5 .

?x5 mihbasic:address-type ?type .

?x5 mihbasic:address ?address }

SPARQL query response (XML format):

<?xml version="1.0"?>

<sparql

xmlns:rdf="http://www.w3.org/1999/02/22-rdf-syntax-ns#"

xmlns:xs="http://www.w3.org/2001/XMLSchema#"

xmlns="http://www.w3.org/2005/sparql-results#" >

<head>

<variable name="prefix_length"/>

<variable name="type"/>

<variable name="address"/>

</head>

<results ordered="false" distinct="false">

<result>

<binding name="prefix_length">

<literal>16</literal>

</binding>

<binding name="type">

<literal>1</literal>

</binding>

<binding name="address">

<literal>c0c90000</literal>

</binding>

</result>

<result>

<binding name="prefix_length">

<literal>16</literal>

</binding>

<binding name="type">

<literal>1</literal>

</binding>

<binding name="address">

<literal>c01f0000</literal>

</binding>

</result>

<result>

<binding name="prefix_length">

<literal>16</literal>

</binding>

<binding name="type">

<literal>1</literal>

</binding>

<binding name="address">

<literal>c0230000</literal>

</binding>

</result>

</results>

</sparql>

| prefix_length | type | address |
| --- | --- | --- |
| 16 | 16 | 16 |
| 1 | 1 | 1 |
| c0c90000 | c01f0000 | c0230000 |

## Example query 6

IS client is asking for PoAs, excluding those with DATA_RATE larger or equial 75.

SPARQL query:

PREFIX mihbasic: <http://www.mih.org/2006/09/rdf-basic-schema#>

PREFIX xsd: <http://www.w3.org/2001/XMLSchema#>

DESCRIBE ?y

WHERE {?x mihbasic:poa ?y .

?y mihbasic:poa-data-rate ?z .

FILTER ( xsd:unsignedInt(?z) >= 75 ) }

SPARQL query response (Turtle format):

@prefix : <http://www.mih.org/2006/09/rdf-basic-schema#> .

[      :poa-address  [

:address "cccccccccccc";

:address-type "6" ];

:poa-capabilities "00b7";

:poa-channel-range  [

:high-channel-range "470";

:low-channel-range "450" ];

:poa-data-rate "80";

:poa-location  [

:geospatial-location-information  [

:altitude "6700";

:altitude-resolution "30";

:altitude-type "2";

:datum "1";

:latitude "053c1f751";

:latitude-resolution "21";

:longitude "f50ba5b97";

:longitude-resolution "20" ];

:method "0" ];

:poa-mac-type "802.16";

:poa-subnet-information  [

:prefix-length "16";

:subnet-address  [

:address "c0c90000";

:address-type "1" ] ] ].

[      :poa-address  [

:address "dddddddddddd";

:address-type "6" ];

:poa-capabilities "00b1";

:poa-channel-range  [

:high-channel-range "470";

:low-channel-range "450" ];

:poa-data-rate "95";

:poa-location  [

:geospatial-location-information  [

:altitude "6800";

:altitude-resolution "30";

:altitude-type "2";

:datum "1";

:latitude "053c1f751";

:latitude-resolution "21";

:longitude "f50ba5b97";

:longitude-resolution "20" ];

:method "0" ];

:poa-mac-type "802.16";

:poa-subnet-information  [

:prefix-length "48";

:subnet-address  [

:address "2002522d94d100000000000000000000";

:address-type "2" ] ] ].

## Useful SPARQL query

### An example of Mobile Node’s current location based query

A Mobile node knows the current location is (N38.89868 (0x04dcc1fc8) , W77.03723 (0xf65ecf031)  ) using GPS and so on.

An approximately 100m square, of which the

Mobile node is in center is shown using the change of 19th bit of latitude and longitude. The 19th bit is a resolution of almost 100m.

SPARQL query for retrieving 802.11 AP BSSID in 100m square of current location of the Mobile is below:

PREFIX mihbasic: <http://www.mih.org/2006/09/rdf-basic-schema#>

PREFIX xsd:      <http://www.w3.org/2001/XMLSchema#>

SELECT ?address

WHERE {?x1 mihbasic:poa-address ?x2 .

?x2 mihbasic:address ?address .

?x1 mihbasic:poa-location ?x3 .

?x3 mihbasic:geospatial-location-information ?x4 .

?x4 mihbasic:latitude ?latitude .

FILTER ((xsd:unsignedLong(?latitude) < 4dcc9fc8 ) &&

(xsd:unsignedLong(?latitude) > 4dcb9fc8 ) ) .

?x4 mihbasic:longitude ?longitude .

FILTER ((xsd:unsignedLong(?longitude) < f65ed7031 ) &&

(xsd:unsignedLong(?longitude) > f65ec7031 ) ) .

}

Figure 3 the serching area of the location based query

### An example of asking query

If a Mobile node needs yes or no question, SPARQL has ASK form. Example below is for asking whether the current network’s one of the roaming partner is “red-partner” or not.

PREFIX mihbasic: <http://www.mih.org/2006/09/rdf-basic-schema#>

PREFIX xsd:      <http://www.w3.org/2001/XMLSchema#>

ASK {?x1 mihbasic:poa ?x2.

?x2 mihbasic:poa-address ?x3 .

?x3 mihbasic:address “aabbccddeeff” .

?x1 mihbasic:roaming-partner ?x4 .

?x4 mihbasic:operator-name “red-partner”

}

### An example of a query using LIMIT option

If a mobile node can assume that query response might be large data size, LIMIT is useful. If used, limit number of answer is only returned. Example below is that a Mobile node needs only 5 of address of 802.11 AP around 100m square from Mibile node’s current location.

PREFIX mihbasic: <http://www.mih.org/2006/09/rdf-basic-schema#>

PREFIX xsd:      <http://www.w3.org/2001/XMLSchema#>

SELECT ?address

WHERE {?x1 mihbasic:poa-address ?x2 .

?x2 mihbasic:address ?address .

?x1 mihbasic:poa-location ?x3 .

?x3 mihbasic:geospatial-location-information ?x4 .

?x4 mihbasic:latitude ?latitude .

FILTER ((xsd:unsignedLong(?latitude) < 4dcc9fc8 ) &&

(xsd:unsignedLong(?latitude) > 4dcb9fc8 ) ) .

?x4 mihbasic:longitude ?longitude .

FILTER ((xsd:unsignedLong(?longitude) < f65ed7031 ) &&

(xsd:unsignedLong(?longitude) > f65ec7031 ) ) .

}

LIMIT 5

# References (This section is not part of proposed text.)

## [SPARQL]

SPARQL Query Language for RDF,  E. Prud'hommeaux,  A. Seaborne, Editors.  World Wide Web Consortium.  http://www.w3.org/TR/rdf-sparql-query/

## [RDF]

RDF/XML Syntax Specification, Dave Beckett, Editor, W3C Recommendation. http://www.w3.org/TR/rdf-syntax-grammar/

## [RDFS]

RDF Vocabulary Description Language 1.0:RDF Schema, Dan Brickley, R.V. Guha, Editors. W3C Recommendation.  http://www.w3.org/TR/rdf-schema/

## [Turtle]

Turtle – Terse RDF Triple Language, Dave Beckett. http://www.dajobe.org/2004/01/turtle/

## [N3]

Notation 3.  http://www.w3.org/DesignIssues/Notation3

## [RESULT]

SPARQL Query Results XML Format, D. Beckett, Editor, W3C Working Draft. http://www.w3.org/TR/rdf-sparql-XMLres/

# Comparison (This section is not part of proposed text.)

## Comparison table

The table below shows suppurted query type which is used in 6 examples for each method (SPARQL query, TLV query type in contribution 21-06-0674-02-0000-IE_TLV_representation, and TLV query type in IEEE P802.21/D01.09)

|  | SPARQL | 0674 | D01.09 |
| --- | --- | --- | --- |
| Query1 | O | O | O |
| Query2 | O | O | X |
| Query3 | O | O | X |
| Query4 | O | O | X |
| Query5 | O | O | X |
| Query6 | O | O | X |

## Comparison of the data size

The table below shows the data size for each query and query response of TLV type query and SPARQL type query.  The target of the size is Value of InfoQueryParameters. (The details are in 21-06-0728-0000-SPARQL_query_example.ppt)

| Byte | SPARQL | 0674 |
| --- | --- | --- |
| Query1 | 62 | 5 |
| Query2 | 147 | 45 |
| Query3 | 566 | 78 |
| Query4 | 215 | 64 |
| Query5 | 433 | 39 |
| Query6 | 257 | 64 |
| Query response 1 | 3292 | 724 |
| Query response 2 | 1589 | 341 |
| Query response 3 | 633 | 130 |
| Query response 4 | 665 | 119 |
| Query response 5 | 945 | 44 |
| Query response 6 | 1223 | 220 |