| Project | IEEE 802.21 Media Independent Handover Services <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | LB#1 Issues 5,6 (21-06-0731-00-0000-LB-Issues-5-6) |  |
| Date Submitted | September 2006 |  |
| Source(s) | Srinivas Sreemanthula, Vivek Gupta |  |
| Ref: |  |  |
| Abstract | This contribution provides resolutions for the issues. |  |
| Purpose | Discuss and adopt in the draft. |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.21. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

# Remedy 1: Update Table-6 as follows

Please refer to section 6.3.5 for representation of each of these IEs.

Table-6

| No | Name of Information Element | Description | Representation in Section |
| --- | --- | --- | --- |
| General Information Elements |  |  |  |
| 1.1 | TYPE_IE_NETWORK_TYPE | Link types of the networks that are available in a given geographical area | 6.3.5.2.1 |
| 1.2 | TYPE_IE_OPER ATOR_IDENTIFI ER | Network Operator The operator of the core network. | 6.3.5.2.2 |
| Access Network specific information For each (Link Type and Operator) combination following information may be provided |  |  |  |
| 2.1 | TYPE_IE_NETWORK_IDENTIFIER | Identifier for the access network. | 6.3.5.3.1 |
| 2.2 | TYPE_IE_ROAMING_PARTNERS | Roaming Partners. Operators with which the current network operator has direct roaming agreements. | 6.3.5.3.2 |
| 2.3 | TYPE_IE_COST | Cost. Indication of cost for service or network usage | 6.3.5.3.3 |
| 2.4 | TYPE_IE_NETWORK_STANDARDS | Access Network specific applicable revisions to the base access network standard | 6.3.5.3.4 |
| PoA specific information For each PoA of each (Link Type + Operator) following IEs may be provided |  |  |  |
| 3.1 | TYPE_IE_POA_ ADDRESS | MAC Address of PoA | 6.3.5.4.1 |
| 3.2 | TYPE_IE_POA_L OCATION | Location of PoA. Geographical location of a given PoA. Multiple location types are supported including coordinate-based location information and civic address | 6.3.5.4.2 |
| 3.3 | TYPE_IE_POA_DATA_RATE | Data Rate. The minimum and maximum value of data rate supported by the link layer of a given PoA | 6.3.5.4.3 |
| 3.4 | TYPE_IE_POA_CHANNEL_RANGE | Channel Range/Parameters Spectrum range supported by the Channel for that PoA | 6.3.5.4.4 |
| Higher layer services/information per PoA |  |  |  |
| 4.1 | TYPE_IE_POA_SUBNET_INFORMATION | Information about subnets supported by a typical PoA | 6.3.5.5.1 |
| 4.2 | TYPE_IE_POA_ CAPABILITIES | Bitmap of PoA capabilities | 6.3.5.5.2 |
| Other Information Elements |  |  |  |
| 5.1 | Vendor Specific IEs | Vendor Specific Services | These may be defined by individual vendors. |

## Remedy 2: Delete the following sub-sections (Draft D01.09..pg 55-59)

6.3.5.3.3 : Security of Network

6.3.5.3.4 : QoS of Network

6.3.5.4.3 : PoA MAC Type

6.3.5.4.7 : PoA PHY Type

## Remedy 3: Add the following sub-sections

6.3.5.3.1: Network Identifier (for Access Network + Operator)

| Type | Description | Length | Value |
| --- | --- | --- | --- |
| TYPE_IE_NETWORK_IDENTIFIER | Identifier of the Access Network | Variable | Access network specific identifier. This is used to uniquely identify the access network. As an example for 802.11 (WLAN) networks this is the ESSID. |

6.3.5.3.4: Network Standards (for Access Network + Operator)

| Type | Description | Length | Value |
| --- | --- | --- | --- |
| TYPE_IE_NETWORK_STANDARDS | Access Network specific applicable revisions to the base access network standard | Variable | This is the list of revisions applicable for the access network standard. For 802 networks this is the list of specific revisions applicable for that access network. For cellular networks this represents the release version of the standrad that is appliacble. |

## Remedy 4: Move section on IE Containers (6.3.5.1) after the definition of different IEs as specified in sections 6.3.5.2/3/4/5

## Remedy 5: Update the section on IE Container definition (current section 6.3.5.1) to reflect the changes in Remedy 1 and Remedy 2

- Remove Network Security IE from TYPE_Container_Network

- Remove QoS IE from TYPE_Container_Network

- Add Network Identifier to TYPE_Container_Network

- Add Network Standards to TYPE_Container_Network

- Remove PoA MAC Type IE from TYPE_Container_POA

- Remove  PoA PHY from TYPE_Container_POA

## Remedy 6: Update the example on TLV Representation in section 6.3.6 to reflect changes in Remedy 1, 2 and 5