| Title | Rules for Inclusion IDs: LB Issue- #53: Comment #s 754 and 755 |  |
| --- | --- | --- |
| Date Submitted | September 2006 |  |
| Source(s) | Subir Das and Yoshihiro Ohba |  |
| Ref: |  |  |
| Abstract | This contribution provides resolutions for the issues. |  |
| Purpose | Discuss and adopt in the draft. |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.21. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

Comment # 754

Whether to include MIHID/SID  is not apparently defined for the different message types.  Is it optional for all of them

Suggested Remedy:

Replace Sections 8.3.3.1 ,  8.3.3.2 and 8.3.3.3  with the following:

      - MIHF ID

Media Independent Handover Function Identifier (MIHF ID) is an identifier that is required to uniquely identify MIHF end points for delivering the MIH services that are registered for it. As described in earlier section xxx, MIH registration enables two MIH entities to create an MIH pairing based on the policies configured in the UE or after capability discovery mechanism. MIH network entities may not be able to provide any MIH services to the UE without this signaling from UE. Use of MIHF ID also enables the MIHF protocol to be transport agnostic. MIHF ID may be assigned to the MIHF during configuration process. MIHF ID is used during MIH registration and all messages that are required to identify the end points that are currently not in a session.

| Type | Length | Value |
| --- | --- | --- |
| TYPE_MIHF_ID | Variable | MIHF ID of the sender from where the MIHF frame originated |

MIHF ID Value:

- MIHF-ID shall be invariant even if the device obtains different interfaces or other components in its registration lifetime.

- For example, MIHF-ID may be a  FQDN or NAI of the sender of the registration request.

8.3.3.2 Session ID

Session Identifier (Session ID) is assigned during a session establishment for remote registration. Each remote  registration needs to be associated with the MIHF that keeps the registration information for particular MIH service.

| Type | Length | Value |
| --- | --- | --- |
| TYPE_SESSION_ID | 4 octets | A unique identifier generated by the source and destination and valid for a session |

Session ID Value:

A Session ID is a 32-bit random number that is unique within the pair MIHFs of the source and destination of session request. The Session ID is created jointly by the source and destination MIHF peers at the time of registration to avoid conflict at both ends. The source MIHF chooses the first 2 octets and the destination MIHF chooses the last 2 octets. The Session ID is valid only during the lifetime of the registration. The Session ID is used in all MIH messages  that are related to Event Service and also all messages that make use of MIH registration.

8.3.3.3 Transaction ID

Transaction Identifier (Transaction ID) is an identifier that is used with every MIH initiator and its response message. This is also required to match each request, response or indication message and its acknowledgment. This Identifier shall be created at the node initiating the transaction and it is carried over within the fixed header part of the MIHF frame. In order to handle the case of duplicate transaction identifiers at the responding node, this identifier is used in conjunction with the Session Identifier.

Transaction ID Value:

Transaction ID is a 16 bit long identifier that is unique to a session between the sender and the receiver. For example, this could be an integer that starts from zero (0) and incremented by one (1)(modulo 2^16) every time a new Transaction ID is generated.

Comment # 755

No format for the synch information is defined,  nor are any rules provide on when to include it or how to interpret it.  In particular, there is an implicit assumption of time-of-day synchronization between devices.

Suggested Remedy:

Pl. refer to 21-06-0726-00-0000-……document.  There is no need for Synchronization information and name space definition for IDs is proposed.