| Title | Rules for Inclusion IDs: LB Issue- #56 and #57: Comment #s 769 and 770 |  |
| --- | --- | --- |
| Date Submitted | September 2006 |  |
| Source(s) | Subir Das and Vivek Gupta |  |
| Ref: |  |  |
| Abstract | This contribution provides resolutions for the issues. |  |
| Purpose | Discuss and adopt in the draft. |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.21. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

Comment # 769

It is not  clear what is the difference between ‘common’ and ‘other’ encodings and where the boundary between two number spaces lies.

Discussion:

There is no need for two encoding schemes. Table 20 and 21 should be merged into one and the encoding should start at 1. As and when new parameter name appears, WG needs to make sure that it does not exist and then should add a new type value for the new parameter from the reserved field.

Suggested Remedy:

Replace Section 8.4 and subsequent Subsections with the following.

Note to Editor: Pl. change the type numbers in other sections as appropriate.

8.4 MIH TLV Encodings

The following TLV encodings shall be used for parameters in MIH Protocol Messages. The Type field shall be one octet. The format of the Length field shall be per the “definite form” of ITU-T X.690. Specifically if the actual length of the Value field is less than or equal to 127 octets, then:

- The length of the Length field shall be one octet

- The MSB of the Length field shall be set to 0, and

- The other 7 bits of the Length field shall be used to indicate the actual value of the Value field in octets.

If the length of the Value field is more than 127 octets, then:

- The length of the Length field shall be one octet more than what is catually used to indicate the length of the Value field in octets

- The MSB of the first octet of the length field shall be set to 1.

- The other 7 bits of the first octet of the length field shall be used to indicate the number of additional octets of of the Length field (i.e. excluding the first octet), and

- The remaining octets (i.e. excluding the first octet) of the Length field shall be used to indicate the actual length of the Value field.

TLV type values shall be unique. TLV encodings start at 1 and subsequent values are assigned in ascending prder. . TLV Type values are assigned in accordance with the following rules:

- Common encodings start at 255, subsequent values are assigned in descending order.

- All other encodings start at 1, subsequent values are assigned in ascending order.

8.4.1  Common TLV Encodings

Common TLV fields and their associated Type codes are presented in Table  below .

Table xx: Type values for Common TLV encodings

| Type | Parameter Name |
| --- | --- |
| 1 | Event List |
| 2 | Command List |
| 3 | Transport List |
| 4 | IS Query Type List |
| 5 | Mobile Node MAC Address |
| 6 | PoA MAC Address |
| 7 | Access Router MAC Address |
| 8 | Link Type |
| 9 | IP Address |
| 10 | Time Interval |
| 11 | Status |
| 12 | Current Link Identifier |
| 13 | Newt Link Identifier |
| 14 | IP Renewal Flag |
| 15 | Mobilty Management Protocol List |
| 16 | Link Down Reason Code |
| 17 | Confidence Level |
| 18 | Unique Identifier |
| 19 | Capability Flag |
| 20 | Number of Parameters |
| 21 | Link Quality Parameter Type |
| 22 | Parameter Old Value |
| 23 | Parameter New Value |
| 24 | Link Status Parameters Type |
| 25 | Handover Mode |
| 26 | Link Action |
| 27 | Link Configure Parameters Type |
| 28 | PoA List |
| 29 | Link Parameter Threshold |
| 30 | Handover Status |
| 31 | Abort Reason |
| 32 | Link Resource Parameters Type |
| 33 | Resource Retention Status |
| 34 | Link Status Parameters Response |
| 35 | Link Configure Parameter Value |
| 36 | IS Query Parameters |
| 37 | IS Query Response |
| 38 | Link Resource Parameters Values |
| 39 - 255 | Reserved |

| Type | Parameter Name |
| --- | --- |
| 255 | Event List |
| 254 | Command List |
| 253 | Transport List |
| 252 | IS Query Type List |
| 251 | Mobile Node MAC Address |
| 250 | PoA MAC Address |
| 249 | Access Router MAC Address |
| 248 | Link Type |
| 247 | IP Address |
| 246 | Time Interval |
| 245 | Status |
| 244 | Current Link Identifier |
| 243 | New Link Identifier |

8.4.1.1 Event List

This parameter specifies the list of events that may be carried in a message. Each event is represented by an event code and occupies one octet.

| Type | Length | Value |
| --- | --- | --- |
| 255 1 | Variable | Array of event codes formatted as messaged-id: Event Code1, Event Code2, Event Code3,.... |

8.4.1.2 Command List

This parameter specifies the list of commands that may be carried in a message. Each command is represented by a command code and occupies one octet.

| Type | Length | Value |
| --- | --- | --- |
| 254 2 | Variable | Array of command codes formatted as messaged-id: Command Code1, Command Code2, Command Code3,.... |

8.4.2.2 Transport List

This parameter specifies the list of transport options for different MIH services.

| Type | Length | Value |
| --- | --- | --- |
| 253 3 | 4 | Octet 1 specifies the transport option for the event service. Octet 2 specifies the transport option for the command service. Octet 3specifies the transport option for the information service. For each bit location a value of ‘1’ indicates that the transport option is supported. Bit #0: L2 Bit #1: L3 or higher layer protocol Bit #2~7: Reserved |

8.4.1.4 IS Query Type List

This parameter specifies the list of IS query types that may be carried in a message.

| Type | Length | Value |
| --- | --- | --- |
| 252 4 | 1 | For each bit location a value of ‘1’ indicates that the Query type is supported. Bit #0: TLV Bit #1: RDF_DATA Bit #2: RDF_SCHEMA_URL Bit #3: RDF_SCHEMA Bit #4~7: Reserved |

8.4.1.5 Mobile Node MAC Address

This parameter specifies the MAC Address of the mobile node.

| Type | Length | Value |
| --- | --- | --- |
| 251 5 | Variable | MAC Address of Mobile Node. The length of address may depend on media specific technology. |

8.4.1.6 PoA MAC Address

This parameter specifies the MAC Address of the Point of Attachment.

| Type | Length | Value |
| --- | --- | --- |
| 250 6 | Variable | MAC Address of PoA. The length of address may depend on media specific technology. |

8.4.1.7 Access Router MAC Address

This parameter specifies the MAC Address of the Access Router.

| Type | Length | Value |
| --- | --- | --- |
| 249 7 | Variable | MAC Address of Access Router. The length of address may depend on media specific technology. |

8.4.1.8 Link Type

This parameter specifies the network type.

| Type | Length | Value |
| --- | --- | --- |
| 248 8 | 4 | See Table . |

8.4.1.9 IP Address

This parameter specifies the IP Address of any entity (MN, HA, etc.). The address can be IPV4 or IPv6 format.

| Type | Length | Value |
| --- | --- | --- |
| 247 9 | Variable | IP Address (IPv4 or IPv6) |

8.4.1.10 Time Interval

This parameter specifies the duration of time interval in milli-seconds.

| Type | Length | Value |
| --- | --- | --- |
| 246 10 | 2 | Time interval in milli-seconds |

8.4.1.11 Status

This parameter specifies the status of an operation.

| Type | Length | Value |
| --- | --- | --- |
| 245 11 | 1 | Status of operation 0: Success 1: Failure 2: Rejected |

8.4.1.12 Link Identifier

The following TLVs specify the identifier of a link.

| Type | Length | Value |
| --- | --- | --- |
| 244 12 | Variable | An identifier of the current link. The value field consists of the following TLVs - Link Type - Mobile Node MAC Address - PoA MAC Address (optional) |
| 243 13 | Variable | An identifier of a new link. The value field consists of the follow ing TLVs - Link Type - Mobile Node MAC Address - PoA MAC Address (optional) |

    - Other TLV Encodings

Other TLV fields and their associated Type codes are presented in Table  .

- Type values for other TLV encodings

| Type | Parameter Name |
| --- | --- |
| 1 | IP Renewal Flag |
| 2 | Mobilty Management Protocol List |
| 3 | Link Down Reason Code |
| 4 | Confidence Level |
| 5 | Unique Identifier |
| 6 | Capability Flag |
| 7 | Number of Parameters |
| 8 | Link Quality Parameter Type |
| 9 | Parameter Old Value |
| 10 | Parameter New Value |
| 11 | Link Status Parameters Type |
| 12 | Handover Mode |
| 13 | Link Action |
| 14 | Link Configure Parameters Type |
| 15 | PoA List |
| 16 | Link Parameter Threshold |
| 17 | Handover Status |
| 18 | Abort Reason |
| 19 | Link Resource Parameters Type |
| 20 | Resource Retention Status |
| 21 | Link Status Parameters Response |
| 22 | Link Configure Parameter Value |
| 23 | IS Query Parameters |
| 24 | IS Query Response |
| 25 | Link Resource Parameters Values |

8.4.1.13 IP Renewal Flag

This parameter specifies whether IP Address has to be renewed or not

| Type | Length | Value |
| --- | --- | --- |
| 1 14 | 1 | Indicates whether the MN shall change IP address when at the new PoA. 0: Change in IP Address required 1: Change in IP Address NOT required |

.

8.4.1.14 Mobility Management Protocol List

This parameter specifies the list of Mobility Management Protocols that may be supported by a PoA.

| Type | Length | Value |
| --- | --- | --- |
| 2 15 | 1 | Indicates type of Mobility Management Protocol supported by the new PoA 0: Mobile IPv4 with FA (FA-CoA) 1: Mobile IPv4 without FA (Co-located CoA) 2: Mobile IPv6 3: Mobile IPv6 with DHCPv6 4-7: Reserved. |

8.4.1.15 Link Down Reason Code

This parameter specifies the reasons for a Link Down event indication.

| Type | Length | Value |
| --- | --- | --- |
| 3 16 | 1 | Reason for why the link went down 0: RC_EXPLICIT_DISCONNECT 1: RC_PACKET_TIMEOUT 2: RC_FAIL_NORESOURCE 3-127: Reserved 128~255: RC_VENDOR_SPECIFIC |

Note: Meaning of Reason code presented above is as follows:

- RC_EXPLICIT_DISCONNECT: The link is down because of explicit disconnect procedures initiated either by client or network

- RC_PACKET_TIMEOUT: The link is down because no acknowledgements were received for transmitted packets within the specified time limit.

- RC_FAIL_NORESOURCE: The link is down because there were no resources to maintain the connection

- [4] RC_VENDOR_SPECIFIC: Vendor Specific reason code.

8.4.1.16 Confidence Level

This parameter specifies the confidence level as a percentage.

| Type | Length | Value |
| --- | --- | --- |
| 4 17 | 1 | Confidence level as a percentage (0-100) |

8.4.1.17 Unique Identifier

This parameter specifies a unique identifier.

| Type | Length | Value |
| --- | --- | --- |
| 5 18 | 2 | Unique Integer Identifier |

8.4.1.18 Capability Flag

This parameter specifies the MIH Capability Flag.

| Type | Length | Value |
| --- | --- | --- |
| 6 19 | 1 | 0: MIH Capability NOT supported 1: MIH Capability Supported |

8.4.1.19 Number of Parameters

This parameter specifies the number of parameters that may be included in a list.

| Type | Length | Value |
| --- | --- | --- |
| 7 20 | 1 | Number of paramaeters that may be included in a list (0-255) |

8.4.1.20 Link Quality Parameter Type

This specifies the type of link parameter.

| Type | Length | Value |
| --- | --- | --- |
| 8 21 | 2 | First Octet: Type of link. 0: Generic Other valid values are defined in Table . Second Octet: Type of Parameter (definition is specific to the Type of Link) For Generic Link (for example): 0: Speed 1: Signal Strength 2: Bit Error Rate 3: Frame Loss Rate before Retransmission ... |

8.4.1.21 Parameter Old Value

This parameter specifies the old value of any link layer parameter.

| Type | Length | Value |
| --- | --- | --- |
| 9 22 | Variable | Old value of a link layer parameter depending on parameter type |

8.4.1.22 Parameter New Value

This parameter specifies the new value of any link layer parameter.

| Type | Length | Value |
| --- | --- | --- |
| 10 23 | Variable | New value of a link layer parameter depending on parameter type |

8.4.1.23 Link Status Parameters Type

This parameter specifies the set of link layer parameters for which status may be requested.

| Type | Length | Value |
| --- | --- | --- |
| 11 24 | 4 | Set of link parameters for which status is requested Bit #0: Network Types Bit #1: Device Information Bit #2: Operation Mode Bit #3: Channel Identifier Bit #4: Channel Quality Bit #5: Link Speed Bit #6: Battery Level Bit #7~31: Reserved |

8.4.1.24 Handover Mode

This parameter specifies the mode of handover.

| Type | Length | Value |
| --- | --- | --- |
| 12 25 | 1 | 0: Make before Break 1: Break before Make |

8.4.1.25 Link Action

This parameter specifies the action to take on link connectivity during handover.

| Type | Length | Value |
| --- | --- | --- |
| 13 26 | 4 | Specifies the suggested action on link during handover. Combina tion of actions is permissible. Bit #0: LINK_DISCONNECT Bit #1:LINK_LOW_POWER Bit #2:LINK_POWER_DOWN Bit #3:LINK_NO_ACTION Bit #4:LINK_RESOURCE_RETAIN Bit #5:DATA_FORWARDING_REQUEST Bit #6:BI_CASTING_REQUEST Bit #7:HANDOVER_CANCEL Bit #8-31: Reserved |

8.4.1.26 Link Configure Parameters Type

This specifies the set of link parameters that may be configured for handover.

| Type | Length | Value |
| --- | --- | --- |
| 14 27 | 1 | Set of link parameters which may be used to configure the link Bit #0: Operation Mode Bit #1: Transmitter Status Bit #2: Current Address Bit #3~31: Reserved |

8.4.1.27 PoA List

This specifies the list of PoAs for a particular type of network.

| Type | Length | Value |
| --- | --- | --- |
| 15 28 | Variable | MAC Address of list of PoAs. Each MAC Address is of same length. |

8.4.1.28 Link Parameter Threshold

This specifies the threshold value of a link layer parameter.

| Type | Length | Value |
| --- | --- | --- |
| 16 29 | Variable | Specifies the threshold value of a link layer parameter. The value depends on the parameter under consideration. |

8.4.1.29 Handover Status

This specifies the status of handover request.

| Type | Length | Value |
| --- | --- | --- |
| 17 30 | 1 | Status of handover request: 0: Initiate Handover 1: Abort Handover |

8.4.1.30  Abort Reason

This specifies the reason for aborting handover.

| Type | Length | Value |
| --- | --- | --- |
| 18 31 | 1 | Reason for aborting handover: 0: Abort handover 1: Accept handover |

8.4.1.31 Link Resource Parameters Type

This specifies the set of link parameters that may be queried for resources during handover.

| Type | Length | Value |
| --- | --- | --- |
| 19 32 | 1 | Set of link parameters which may be queried for resources during handover: Bit #0: Maximum Bandwidth Bit #1-31: Reserved |

8.4.1.32 Resource Retention

This specifies the resource retention request.

| Type | Length | Value |
| --- | --- | --- |
| 20 33 | 1 | 0: Retain resources 1: Release resources |

8.4.1.33 Link Status Parameters Response

This parameter specifies the response to request to get the status of link layer parameters, see Section  for details of link status parameter definition.

| Type | Length | Value |
| --- | --- | --- |
| 21 34 | Variable | Status of different link layer parameters |

8.4.1.34 Link Configure Parameter Value

This specifies the configuration values for a set of link parameters

| Type | Length | Value |
| --- | --- | --- |
| 22 35 | Variable | Values for a set of link parameters that need to be configured |

8.4.1.35 IS Query Parameters

This specifies the parameters to be specified for IS query

| Type | Length | Value |
| --- | --- | --- |
| 23 36 | Variable | List of query specific parameters depending on value of InfoQue ryType: 1: TLV specific parameters 2: One SPARQL Query parameter 3: Nothing is carried 4: zero or one Extended Schema URL parameter |

.

Table XX: SPARQL Query Parameter Format

| Type | Length | Value |
| --- | --- | --- |
| TYPE_MIME_TLV | Variable | An octet string for a MIME type to specify a format of SPARQL Query. When this TLV is not contained, MIME type "application/sparql- query" is used. |
| SPARQL_QUERY_VALUE | Variable | An octet string of the content of the SPARQL Query |

Table XX: Extended Schema URL Parameter Format

| Type | Length | Value |
| --- | --- | --- |
| TYPE_EXTENDED_SCHE MA_URL | Variable | URL of an Extended Schema encoded in octet string |

8.4.1.36 IS Query Response

This specifies the parameters to be specified for IS query

| Type | Length | Value |
| --- | --- | --- |
| 24 37 | Variable | List of response depending on value of InfoQueryType: 1: TLV specific response parameters 2: One SPARQL Query response parameter 3: One Extended Schema URL parameter 4: Extended Schema Content |

Table XX: SPARQL Query Response Parameter Format

| Type | Length | Value |
| --- | --- | --- |
| TYPE_MIME_TLV | Variable | An octet string for a MIME type to specify a format of SPARQL Query Result. When this TLV is not contained, MIME type "application/sparql-results+xml" is used. |
| SPARQL_QUERY_RESULT S_TLV | Variable | A value that is formatted with the MIME type specified in the MIME_Type_TLV. When the MIME type is "application/sparql-results+xml" this field contains XML text for SPARQL Query Results |

Table XX: Extended Schema Content TLV Format

| Type | Length | Value |
| --- | --- | --- |
| TYPE_MIME_TLV | Variable | A MIME Type TLV to specify a format of the Schema_Data. When this TLV is not contained, MIME type "application/ rdf+xml" is used. |
| TYPE_EXTENDED_S CHEMA | Variable | This field contains Extended Schema stored in the requested URL. |

8.4.1.37 Link Resource Parameter Values

This specifies the response due to query of resources

| Type | Length | Value |
| --- | --- | --- |
| 25 38 | Variable | List of response depending on set of Resource Parameters |

Comment # 770

It is entirely unclear how protocol extensibility is handled for these various number spaces. For  example, if a new information element is defined, how is the receiver to determine what to do with unknown types (drop, ignore, reject message etc.)

Discussion:

With the above name space definition, it adds the protocol extensibility. If new information elements are defined WG needs to add a new type value. With regards to receiver behavior, error codes need to be defined to handle the unknown types. However, it will depend upon the services. For example, for Information Service, error codes such as ‘information element not defined’ or ‘information not available’ may be defined and added. A separate section on “Error Codes’ may be defined and added as an appendix in newer version of the draft.  WG should solicit such contributions.

Suggested Remedy:

Add the following text at the end of Section 8.4.

Currently, there are 38  type values defined for existing  parameters  name. Values from 39-255 are reserved for future extension.

.