IEEE P802.21/D01.09
Draft IEEE Standard for Local and Metropolitan Area Networks:
Media Independent Handover Services

| Normative Text Proposal handling of LinkSwitch command towards Link Layers |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-September-11 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Ulises Olvera | InterDigital Communications | 100, rue Sherebrook, Ouest, 10e étage, Montreal, QC, H3A 3G4 | +1514-904-6258 | Ulises.Olvera-Hernandez@InterDigital.com |

- Abstract

This document contains a normative text proposal in support of link layer switch commands that can be mapped to existing link layer technologies’ operations.

Notice: This document has been prepared to assist IEEE 802.21. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.21.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

# Problem Statement

## Background

The current 802.21 specification provides a uniform, well defined set of primitives that can be used by an MIH User to avail of Command, Event and Information services. The MIHF receives MIH primitives from the user and determines whether or not one or more link layer primitives should be triggered in order to satisfy the request from the user.

Likewise the MIHF may generate MIH primitives toward the user upon receipt of one or more link layer primitives coming from link layers.

Furthermore, 802.21 link layer primitives may be mapped directly or indirectly into existing primitives supported by other link layer technologies.

In this regard the link layer primitives described by 802.21 serves as a “blue print” of what is expected from the link layer technology and therefore a mapping can be established between these “blue print” primitives and one or more existing primitives within a relevant link layer technology such as 802.11, 802.16 or even 3GPP/3GPP2.

Several examples exist depicting such behavior. Thus, MIH_Link_Detected is characterized as a Link Detected, MIH_Ling_Going_Down is characterized as a Link Going Down an so on.

Problem Description

The current draft supports a number of MIH commands that might cause a service flow to switch from one link layer connections to the other. However although there are enough MIH primitives that can do this, there does not exist any 802.21 link layer counter part or “blue print” as it is the case with other commands, events and information services.

Furthermore the current specification makes references to link commands that should be used by the MIHF when it receives MIH commands affecting link layer connection. Note that these link layer commands are not defined in the specification, however primitives exhibiting similar behavior may be found in existing link layer technologies. An example of these references can be found in section 7.4.7.1.3 “Effect on Receipt

Proposed Solution

We propose the introduction of a new set of primitives that serve as a “blueprint” for the specification of behavior that is expected from link layers when the primitive requesting the manipulation of link layer connection or interfaces is issued. It does not constrain any specific implementation as the primitives are abstract representations. Therefore any primitive that is already specified within specific link layer technologies that satisfy the functionality specified in this primitive can be mapped to the Link_Switch primitive introduced in the following sections.

### New Primitives

We propose that a link layer command be introduced as a means to characterize the switch of link layer connection from the MIHF towards link layers.

We propose that this link layer command be based on the exiting MIH Switch Command and that normative text be introduced as follows:

In page 87, after line 60 insert the following:

7.3.22 Link_Switch.request

7.3.22.1 Function

This primitive is used by the MIHF to request actions on a link layer connection that enable optimal handling of link layer resources for the purpose of handover operations. The link layer connection can be order, e.g., to remain active, to shut down or come up active and to remain in stand-by mode

7.3.22.2 Semantics of Service Primitve

The parameter of the service primitive are as follows:

Link_Switch.Request (

Link Identifier

LinkAction

)

The following table details the possible actions that can be requested from the link layer connection.

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Link Actions | Link Action | Any valid value of Link Action | Specifies the suggested action on link during handover. Combination of the below choices are allowed. Bit #0: LINK_DISCONNECT Bit #1: LINK_LOW_POWER Bit #2: LINK_POWER_DOWN Bit #3: LINK_NO_ACTION Bit #4: LINK_RESOURCE_RETAIN Bit #5: DATA_FORWARDING_REQUEST Bit #6: BI_CASTING_REQUEST Bit #7: HANDOVER_CANCEL Bit #8-31: Reserved |

7.3.22.3 When generated

The MIHF generates this primitive upon request from the MIH User to perform an action on a pre-define link layer connection.

7.3.22.4 Effect on receipt

Upon receipt of this primitive, the link layer technology supporting the current link layer connections performs the action specified by the LinkAction IE in according with procedures specified within the relevant standards organization

7.3.22 Link_Switch.confirm

7.3.22.1 Function

This primitive is used by link layer technologies to provide an indication of the result of the actions executed on the current link layer connection. The

7.3.22.2 Semantics of Service Primitve

The parameter of the service primitive are as follows:

Link_Switch.confirm (

ResultCode

)

The following table details the possible actions that can be requested from the link layer connection.

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| ResultCode | Enumarate | Success Failure Rejection | Result of the actions specified in the Link_Switch.request primitive |

7.3.22.3 When generated

The link layer technology generates this primitive to communicate the result of the actions executed on the link layer connection

7.3.22.4 Effect on receipt

Upon receipt of this primitive, the MIHF determines the relevant MIH command that needs to be used to provide and indication to the MIH user of the actions performed on the current link layer connection

### Modification to existing Primitives

In section 7.4.7.1.1 “Function” Modified the existing text as follows:

Upon receipt of the MIH Switch command,.MIHF may use Link_Switch to order the relevant link layer technology to perform an action on the link or links specified by the LinkIdentifier. The actions that need to be performed are signal through the LinkAction information element..

In section 7.4.7.1.2 “Semantics of Service Primitive” remove the “HandoverMode” indicator. This indicator is not required. The MIHF does not have the means or information to decide when a link should be kept alive and for how long. This behavior is more appropriate from a HO policy entity. An MIH User with this characteristics can request actions on link connection when the action is required.