| Title | LB Issue- #: 8 Comments: 384, 396, 400 |  |
| --- | --- | --- |
| Date Submitted | September 2006 |  |
| Source(s) | Vivek Gupta |  |
| Ref: |  |  |
| Abstract | This contribution provides resolutions for LB-#1 Issue regarding Extensibility of IE Name Space |  |
| Purpose | Discuss and adopt in the draft. |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.21. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

Comment 384:

Include a description for following entries in Table-8:

Core 802.21 Header specific IEs:

Core 802.21 specific IEs

Vendor specific IEs:

14 bit ID:

Reserved for other SDOs:

Reserved for IETF:

Comment 396:

How is extensibility managed for the Information Elements Name Space? What specification needs to be provided to allocate a new Information Element? Who manages the Identifier space?

Behavior in the case of receiving an unknown identifier must also be defined.

Remedy 1:

Update section 6.3.4 as follows:

“

The IEEE 802.21 Information Elements are assigned identifiers (IDs) as follows. Each Information element ID is a 32 bit value. The IEs may be defined by the 802.21 specification and by other SDOs (such as 802.11, 802.16) as well. The name space for these IEs is divided so that each SDO can define its own set of IEs in its own range. Please refer to Tbale-8 for more details.

The 802.21 specific Information Elements (Header and other Core 802.21 ) are assigned identifiers as per the 802.21 specification. Please refer to Table-31 in Annex-3 for more details. Vendors may specify their own IEs using the name space allocated to them. They can use the 16 bit vendor specific OUI and combine it with additional 14 bits allocated to them as part of their individual IE identifiers.

Other SDOs may use the name space allocated to them in defining these SDO specific IEs as specified in table-8. A set of IE name space range is also reserved for development and testing. These should not be used in released products. Allocation of additional IE name space and any revisions to this assignment would be handled by future revisions of the 802.21 specification.

Functional entities should discard any IE identifier that they are not aware of.

“