| Project | IEEE 802.21 MIHO <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | Network Standards IE |  |
| Date Submitted | October 23, 2006 |  |
| Source(s) | Yoshihiro Ohba, Subir Das and Srinivas Sreemanthula |  |
| Re: | IEEE 802.21 Session #17 in November 2006 |  |
| Abstract | This document proposes text for Network Standards IE |  |
| Purpose | LB#1a comment resolution |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.21. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

- Proposal for D02-00

Change Section 6.3.5.2.4:

| Type | Description | Length | Value |
| --- | --- | --- | --- |
| TYPE_IE_NETWORK_STA NDARDS | Access Network specific applicable revisions to the base access network standard | Variable | This is the list of revisions applicable for the access network standard. For 802 networks this is the list of specific revisions applicable for that access network. For cellular networks this represents the release version of the standrad that is appliacble. This field contains a list of standard names where each standard name consists of case-sensitive alphanumeric characters delimited by character ‘&’, e.g., “a&b&g&i&r” for an IEEE 802.11 access network that supports 802.11a, 802.11b, 802.11g, 802.11i and 802.11r standards. The format and semantics of each standard name is defined by each media-specific WG or SDO. |

[2] Add one optional Network Standards IE to TYPE_CONTAINER_POA TLV.  (This is because different PoAs in the same network type may support different set of standards.)