| Project | IEEE 802.21 MIHO <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | Network Standards IE |  |
| Date Submitted | November 15, 2006 |  |
| Source(s) | Yoshihiro Ohba, Subir Das, Srinivas Sreemanthula, Ronny Kim and Simon Barber |  |
| Re: | IEEE 802.21 Session #17 in November 2006 |  |
| Abstract | This document proposes text for Network Standards IE |  |
| Purpose | LB#1a comment resolution |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.21. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

- Proposal for D02-00

Change Section 6.3.5.2.4:

| Type | Description | Length | Value |
| --- | --- | --- | --- |
| TYPE_IE_NETWORK_STA NDARDS | Access Network specific applicable revisions to the base access network standard | 8 octets | This is the list of revisions applicable for the access network standard. For 802 networks this is the list of specific revisions applicable for that access network. For cellular networks this represents the release version of the standrad that is appliacble. This field contains a bitmap in which each bit corresponds to a standard of the specific Network Type. The bitmap is defined as follows. For Network Type = Wireless – IEEE 802.11: (PHY/MAC Features) Bit 0: Aceess point (AP) Bit 1: Independent station (not an AP) Bit 2: FHSS PHY for 2.4GHz band Bit 3: DSSS PHY for 2.4GHz band Bit 4: IR PHY Bit 5: OFDM PHY for 5GHz band Bit 6: High-speed PHY Bit 7: Multidomain operation capability implemented Bit 8: Extended Rate PHY (ERP) Bit 9: Spectrum management operation supported Bit 10: Regulatory class capability implemented Bit 11: QoS Supported Bit 12-15: Rserved (Security Features) Bit 16: WPA - Enterprise Bit 17: WPA - Personal Bit 18: WPA2 - Enterprise Bit 19: WPA2 - Personal Bit 20-31: Reserved (QoS Features) Bit 32: WMM Bit 33: WMM Power Save Bit 34-47: Reserved (Other Features) Bit 48-: CWG-RF Bit 49-63: Reserved: For Network Type = Wireless – IEEE 802.16: Bit 0: 802.16-2001 Bit 1: 802.16c-2002 Bit 2: 802.16a-2003 Bit 3: 802.16-2004 Bit 4: 802.16e-2005 Bit 5: 802.16g-2007 Bit 6-63: Reserved For Newtork Type = CDMA2000: Bit 0: CDMA-2000-Rev-0 Bit 1: Rev-A Bit 2: Rev-B Bit 3: Rev-C Bit 4: Rev-D (?) Bit 5-63: Reserved For Network Type = UMTS Bit 0: GSM Bit 1: GPRS Bit 2: EDGE Bit 3: Rel-99 Bit 4: Rel-4 Bit 5: Rel-5 (w/ HSDPA) Bit 6: Rel-6 (w/ HSUPA) Bit 7: Rel-7 (MIMO/OFDM) Bit 8: Rel-8 Bit 9-63: Reserved |

[2] Add one optional Network Standards IE to TYPE_CONTAINER_POA TLV.  (This is because different PoAs in the same network type may support different set of standards.)