| Project | IEEE 802.21 MIHO <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | PoA IP Address IE |  |
| Date Submitted | November 15, 2006 |  |
| Source(s) | Yoshihiro Ohba and Subir Das |  |
| Re: | IEEE 802.21 Session #17 in November 2006 |  |
| Abstract | This document proposes text for PoA IP Address IE |  |
| Purpose | LB#1a comment resolution |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.21. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

- Proposal for D02-00

[1] Add the following entry to PoA specific information in Table 6:

| 4.X | TYPE_IE_POA_IP_ADDRESS | IP Address of PoA. | 6.5.5.3.X |
| --- | --- | --- | --- |

[2] Insert the following Section:

6.3.5.3.X PoA IP Address

| Type | Description | Length | Value |
| --- | --- | --- | --- |
| TYPE_IE_POA_IP_ADDRESS | IP Address of PoA | Variable | IP Address (4-octet for IPv4 or 16-octet for IPv6). This IE is used only for a PoA that has an IP address. |

[3] Add "TYPE_IE_POA_IP_ADDRESS" to PoA Information in Figure 20.

[4] Add PoA IP Address IE #1, …, #k in TYPE_CONTAINER_POA structure in Section 6.3.5.4

[5] Add TYPE_IE_POA_IP_ADDRESS” in MIIS_TLV_REPORT_TEMPLATE structure in Section 6.3.6.1.

[6] Update the Basic Schema to include a corresponding Class and Property.