| Title | MIH Protocol States |  |
| --- | --- | --- |
| Date Submitted | October 2006 |  |
| Source(s) | Xiaoyu Liu, Jeong-Jae Won |  |
| Ref: |  |  |
| Abstract | This contribution provides suggested MIH Protocol States |  |
| Purpose | Discuss and adopt in the draft. |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.21. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

Introduction

Follow the discussions on MIH state transition diagrams and propose texts for MIH Protocol States to improve the interoperability. The proposed states capture the basic MIH protocol services:

- MIHF Discovery

- MIHF Capability Discovery

- MIH Remote Registration

- MIH Message Exchange

The proposed state transition diagram also clarifies the MIH message exchanges mechanism between MIH peer entities.

Remedy 1:

Insert below text on page 154 of draft D02.00 just before section 8.3.1

Figure xx - MIH Protocol States in Mobile Node

The MIHF in the Mobile Node can be in one of four states – MIH-LOCAL, MIH-DEREGISTERED, MIH-REGISTERED, and MIH-TRANSACTION.

Upon configuration of the MIH functional entity, e.g., after MIHF is initialized or reset, the MIHF in the MN enters MIH-LOCAL state by default. In MIH-LOCAL state, local MIH services are allowed, however the mobile node can not communicate with remote peer MIHF entities. The MN initiates the MIHF Discovery procedure as specified in section 6.4.2 in the MIH-LOCAL state.

In MIH-DEREGISTERED state, the MN has discovered a peer remote MIHF entity; however, the MN has not registered successfully with the peer remote MIHF. The MN initiates the MIH Registration procedure to establish a session as specified in section 6.4.4 in the MIH-DEREGISTERED state. MIHF may perform MIH Capability Discovery procedures in MIH-DEREGISTERED state.

The MIHF in MN enters MIH-REGISTERED state after successfully registering with a peer MIHF according to section 6.4.4. In this state the MN has established a session with corresponding MIHF. While in MIH-REGISTERED, a Session ID has been created and may be used in all MIH messages exchanged between peer MIHF entities. However, there is no MIH transaction in the MIH-REGISTERED state.

The MIHF enters the MIH-TRANSACTION state when the message exchange between peer MIHF entities occurs. In case of L3 transport, the MIH-NET SAP opens the MIH data transport, i.e. message exchange between the two entities is enabled. For connectionless-mode data transfer sessions, MIHF enters MIH-TRANSACTION when MIHF receives an MIH data request; for connection-oriented data transfer sessions, MIHF enters MIH-TRANSACTION when it opens a connection for data transfer. When the MIHF finishes the data transfer for connectionless-mode sessions, or closes the connection for connection-oriented sessions, it enters back to MIH-REGISTERED state.

The MIHF enters MIH-DEREGISTRATION from MIH-REGISTERED or MIH-TRANSACTION state after deregistration procedure is performed.

The MIHF may enter MIH-LOCAL state from any of the other MIH states on loss of MIHF capability indications. MIHF remote communication is terminated.