| Project | IEEE 802.21 MIHO <http://www.ieee802.org/21/> |
| --- | --- |
| Title | Revised Basic Schema |
| Date Submitted | July 13th, 2007 |
| Source(s) | Kenichi Taniuchi, Yoshihiro Ohba, Subir Das |
| Re: | 21-07-00006-07-0000-Basic_Schema |
| Abstract | This document contains revised definition of Basic Schema and Extended Schema example. |
| Purpose | Letter Ballot 1e Recirculation |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.21. |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |

# Introduction

## This document contains revised definition of Basic Schema that follows IEEE P802.21/D06.00 with related modified text and describes how to create Extended Schema using an example.

# Graphical Representation of Basic Schema

## Suggested modified text

Updated Figure 1,2 involves modification of the explaining text at line 1, page 225 in the IEEE P802.21/D06.00 as below:

Figure XX and YY show an example UML representation of the IEEE 802.21 MIIS basic schema. A dotted arrow indicates range of a property. An attribute indicates a property of a class. A class which starts from TYPE_IE is a subclass of Information_Element class.

Figure XX UML Representation of Basic Schema

Figure YY UML Representation of Basic Schema

# Basic Schema Definition

<?xml version="1.0"?>

<!DOCTYPE rdf:RDF [

<!ENTITY rdf "http://www.w3.org/1999/02/22-rdf-syntax-ns#">

<!ENTITY rdfs "http://www.w3.org/2000/01/rdf-schema#">

<!ENTITY mihbasic "URL_TO_BE_ASSIGNED">

<!ENTITY owl "http://www.w3.org/2002/07/owl#">

<!ENTITY xsd "http://www.w3.org/2001/XMLSchema#">

]>

<rdf:RDF xmlns:rdf="&rdf;" xmlns:rdfs="&rdfs;"

xmlns:mihbasic="&mihbasic;" xml:base="&mihbasic;"

xmlns:owl="&owl;" xmlns:xsd="&xsd;">

<owl:Ontology rdf:about="">

<rdfs:label>

Basic Schema for IEEE 802.21 Information Service

</rdfs:label>

<owl:versionInfo>1.0</owl:versionInfo>

</owl:Ontology>

<owl:DatatypeProperty rdf:ID="ie_type_identifier">

<rdfs:subPropertyOf rdf:resource="&rdfs;label"/>

<rdfs:range rdf:resource="&xsd;hexBinary"/>

<rdfs:comment>

A type identifier values for Information Elements.

</rdfs:comment>

</owl:DatatypeProperty>

<owl:ObjectProperty rdf:ID="type_ie_container_list_of_network">

<mihbasic:ie_type_identifier>10000300</mihbasic:ie_type_identifier>

<rdfs:range rdf:resource="#LIST_OF_NETWORKS"/>

</owl:ObjectProperty>

<owl:Class rdf:ID="LIST_OF_NETWORKS">

<rdfs:subClassOf>

<owl:Restriction>

<owl:onProperty rdf:resource="#type_ie_container_network"/>

<owl:cardinality rdf:datatype="&xsd;nonNegativeInteger">1

</owl:cardinality>

</owl:Restriction>

</rdfs:subClassOf>

</owl:Class>

<owl:ObjectProperty rdf:ID="type_ie_container_network">

<mihbasic:ie_type_identifier>10000301</mihbasic:ie_type_identifier>

<rdfs:domain rdf:resource="#LIST_OF_NETWORKS"/>

<rdfs:range rdf:resource="#NETWORK"/>

<rdfs:comment>

This class contains General Information depicting and Access

Network Specific Information.

</rdfs:comment>

</owl:ObjectProperty>

<owl:Class rdf:ID="NETWORK">

<rdfs:subClassOf>

<owl:Restriction>

<owl:onProperty rdf:resource="#type_ie_network_type"/>

<owl:cardinality rdf:datatype="&xsd;nonNegativeInteger">1

</owl:cardinality>

</owl:Restriction>

</rdfs:subClassOf>

<rdfs:subClassOf>

<owl:Restriction>

<owl:onProperty rdf:resource="#type_ie_operator_identifier"/>

<owl:cardinality rdf:datatype="&xsd;nonNegativeInteger">1

</owl:cardinality>

</owl:Restriction>

</rdfs:subClassOf>

<rdfs:subClassOf>

<owl:Restriction>

<owl:onProperty rdf:resource="#type_ie_container_poa"/>

<owl:minCardinality rdf:datatype="&xsd;nonNegativeInteger">1

</owl:minCardinality>

</owl:Restriction>

</rdfs:subClassOf>

</owl:Class>

<owl:ObjectProperty rdf:ID="type_ie_network_type">

<mihbasic:ie_type_identifier>0x10000000</mihbasic:ie_type_identifier>

<rdfs:domain rdf:resource="#NETWORK"/>

<rdfs:range rdf:resource="#NETWORK_TYPE"/>

</owl:ObjectProperty>

<owl:Class rdf:ID="NETWORK_TYPE">

<rdfs:subClassOf>

<owl:Restriction>

<owl:onProperty rdf:resource="#link_type"/>

<owl:cardinality rdf:datatype="&xsd;nonNegativeInteger">1

</owl:cardinality>

</owl:Restriction>

</rdfs:subClassOf>

</owl:Class>

<owl:DatatypeProperty rdf:ID="link_type">

<rdfs:domain rdf:resource="#NETWORK_TYPE"/>

<rdfs:range rdf:resource="&xsd;unsignedInt"/>

<rdfs:comment>

Link type of a network. The following values are assigned:

1: Wireless - GSM

2: Wireless - GPRS

3: Wireless - EDGE

15: Ethernet

18: Wireless - Other

19: Wireless - IEEE 802.11

22: Wireless - CDMA2000

23: Wireless - UMTS

24: Wireless - cdma-2000-HRPD

27: Wireless - IEEE 802.16

28: Wireless - IEEE 802.20

29: Wireless - IEEE 802.22

</rdfs:comment>

</owl:DatatypeProperty>

<owl:ObjectProperty rdf:ID="revision">

<rdfs:subPropertyOf rdf:resource="#bit_number"/>

<rdfs:domain rdf:resource="#NETWORK_TYPE"/>

<rdfs:comment>

The range of #bit_number is 0-63.

</rdfs:comment>

</owl:ObjectProperty>

<owl:ObjectProperty rdf:ID="type_ie_operator_identifier">

<mihbasic:ie_type_identifier>0x10000001</mihbasic:ie_type_identifier>

<rdfs:domain rdf:resource="#NETWORK"/>

<rdfs:range rdf:resource="#OPERATOR_ID"/>

</owl:ObjectProperty>

<owl:Class rdf:ID="OPERATOR_ID">

<rdfs:subClassOf>

<owl:Restriction>

<owl:onProperty rdf:resource="#operator_namespace"/>

<owl:cardinality rdf:datatype="&xsd;nonNegativeInteger">1

</owl:cardinality>

</owl:Restriction>

</rdfs:subClassOf>

<rdfs:subClassOf>

<owl:Restriction>

<owl:onProperty rdf:resource="#operator_name"/>

<owl:cardinality rdf:datatype="&xsd;nonNegativeInteger">1

</owl:cardinality>

</owl:Restriction>

</rdfs:subClassOf>

</owl:Class>

<owl:DatatypeProperty rdf:ID="operator_namespace">

<rdfs:domain rdf:resource="#OPERATOR_ID"/>

<rdfs:range rdf:resource="&xsd;unsignedByte"/>

<rdfs:comment>

A value of Operator Type:

0: GSM/UMTS

1: CDMA

2: REALM

3: ITU-T/TSB

</rdfs:comment>

</owl:DatatypeProperty>

<owl:DatatypeProperty rdf:ID="operator_name">

<rdfs:domain rdf:resource="#OPERATOR_ID"/>

<rdfs:range rdf:resource="&xsd;string"/>

<rdfs:comment>

The value is a non NULL terminated

string whose length shall not exceed 253 octets.

</rdfs:comment>

</owl:DatatypeProperty>

<owl:DatatypeProperty rdf:ID="type_ie_service_provider_identifier">

<mihbasic:ie_type_identifier>0x10000002</mihbasic:ie_type_identifier>

<rdfs:domain rdf:resource="#NETWORK"/>

<rdfs:range rdf:resource="&xsd;string"/>

<rdfs:comment>

A non-NULL terminated string whose length shall not exceed 253 octets.

</rdfs:comment>

</owl:DatatypeProperty>

<owl:DatatypeProperty rdf:ID="type_ie_access_network_identifier">

<mihbasic:ie_type_identifier>0x10000100</mihbasic:ie_type_identifier>

<rdfs:domain rdf:resource="#NETWORK"/>

<rdfs:range rdf:resource="&xsd;string"/>

<rdfs:comment>

A non-NULL terminated string whose length shall not exceed 253 octets.

</rdfs:comment>

</owl:DatatypeProperty>

<owl:DatatypeProperty rdf:ID="type_ie_access_network_aux_id">

<mihbasic:ie_type_identifier>0x10000101</mihbasic:ie_type_identifier>

<rdfs:domain rdf:resource="#NETWORK"/>

<rdfs:range rdf:resource="&xsd;string"/>

<rdfs:comment>

It is SSID if network type is IEEE 802.11.

</rdfs:comment>

</owl:DatatypeProperty>

<owl:ObjectProperty rdf:ID="type_ie_roaming_partner">

<mihbasic:ie_type_identifier>0x10000102</mihbasic:ie_type_identifier>

<rdfs:domain rdf:resource="#NETWORK"/>

<rdfs:range rdf:resource="#OPERATOR_ID"/>

</owl:ObjectProperty>

<owl:ObjectProperty rdf:ID="type_ie_cost">

<mihbasic:ie_type_identifier>0x10000103</mihbasic:ie_type_identifier>

<rdfs:domain rdf:resource="#NETWORK"/>

<rdfs:range rdf:resource="#COST"/>

</owl:ObjectProperty>

<owl:Class rdf:ID="COST">

<rdfs:subClassOf>

<owl:Restriction>

<owl:onProperty rdf:resource="#cost_unit"/>

<owl:cardinality rdf:datatype="&xsd;nonNegativeInteger">1

</owl:cardinality>

</owl:Restriction>

</rdfs:subClassOf>

<rdfs:subClassOf>

<owl:Restriction>

<owl:onProperty rdf:resource="#cost_value"/>

<owl:cardinality rdf:datatype="&xsd;nonNegativeInteger">1

</owl:cardinality>

</owl:Restriction>

</rdfs:subClassOf>

<rdfs:subClassOf>

<owl:Restriction>

<owl:onProperty rdf:resource="#cost_currency"/>

<owl:cardinality rdf:datatype="&xsd;nonNegativeInteger">1

</owl:cardinality>

</owl:Restriction>

</rdfs:subClassOf>

</owl:Class>

<owl:DatatypeProperty rdf:ID="cost_unit">

<rdfs:domain rdf:resource="#COST"/>

<rdfs:range rdf:resource="&xsd;unsignedByte"/>

<rdfs:comment>

The unit of the cost:

0: second

1: minute

2: hours

3: day

4: week

5: month

6: year

7: free

8: flat rate

9-255: Reserved

</rdfs:comment>

</owl:DatatypeProperty>

<owl:DatatypeProperty rdf:ID="cost_value">

<rdfs:domain rdf:resource="#COST"/>

<rdfs:range rdf:resource="&xsd;double"/>

<rdfs:comment>

The cost value in Currency/Unit

</rdfs:comment>

</owl:DatatypeProperty>

<owl:DatatypeProperty rdf:ID="cost_currency">

<rdfs:domain rdf:resource="#COST"/>

<rdfs:range rdf:resource="&xsd;string"/>

<rdfs:comment>

A three-letter currency code(e.g. "USD") specified by

ISO 4217.

</rdfs:comment>

</owl:DatatypeProperty>

<owl:DatatypeProperty rdf:ID="bit_number">

<rdfs:range rdf:resource="&xsd;unsignedByte"/>

<rdfs:comment>

This property represents a bit number that has

the value as true.

</rdfs:comment>

</owl:DatatypeProperty>

<owl:ObjectProperty rdf:ID="type_ie_network_security">

<mihbasic:ie_type_identifier>0x10000105</mihbasic:ie_type_identifier>

<rdfs:domain rdf:resource="#NETWORK"/>

<rdfs:range rdf:resource="#NETWORK_SECURITY"/>

</owl:ObjectProperty>

<owl:Class rdf:ID="NETWORK_SECURITY">

</owl:Class>

<owl:ObjectProperty rdf:ID="type_ie_network_qos">

<mihbasic:ie_type_identifier>0x10000106</mihbasic:ie_type_identifier>

<rdfs:domain rdf:resource="#NETWORK"/>

<rdfs:range rdf:resource="#QOS_LIST"/>

</owl:ObjectProperty>

<owl:Class rdf:ID="QOS_LIST">

</owl:Class>

<owl:DatatypeProperty rdf:ID="num_qos_types">

<rdfs:domain rdf:resource="#QOS_LIST"/>

<rdfs:range rdf:resource="&xsd;unsignedByte"/>

</owl:DatatypeProperty>

<owl:DatatypeProperty rdf:ID="throughput">

<rdfs:domain rdf:resource="#QOS_LIST"/>

<rdfs:range rdf:resource="&xsd;unsignedInt"/>

</owl:DatatypeProperty>

<owl:DatatypeProperty rdf:ID="packet_error_rate">

<rdfs:domain rdf:resource="#QOS_LIST"/>

<rdfs:range rdf:resource="&xsd;unsignedShort"/>

</owl:DatatypeProperty>

<owl:Class rdf:ID="COS">

<rdfs:subClassOf>

<owl:Restriction>

<owl:onProperty rdf:resource="#cos_id"/>

<owl:cardinality rdf:datatype="&xsd;nonNegativeInteger">1

</owl:cardinality>

</owl:Restriction>

</rdfs:subClassOf>

<rdfs:subClassOf>

<owl:Restriction>

<owl:onProperty rdf:resource="#cos_value"/>

<owl:cardinality rdf:datatype="&xsd;nonNegativeInteger">1

</owl:cardinality>

</owl:Restriction>

</rdfs:subClassOf>

</owl:Class>

<owl:DatatypeProperty rdf:ID="cos_id">

<rdfs:domain rdf:resource="#COS"/>

<rdfs:range rdf:resource="&xsd;unsignedShort"/>

<rdfs:comment>

A type to represent a class of service identifier.

</rdfs:comment>

</owl:DatatypeProperty>

<owl:DatatypeProperty rdf:ID="cos_value">

<rdfs:domain rdf:resource="#COS"/>

<rdfs:range rdf:resource="&xsd;unsignedShort"/>

</owl:DatatypeProperty>

<owl:ObjectProperty rdf:ID="cos_min_packet_transfer_delay">

<rdfs:domain rdf:resource="#QOS_LIST"/>

<rdfs:range rdf:resource="#COS"/>

</owl:ObjectProperty>

<owl:ObjectProperty rdf:ID="cos_avg_packet_transfer_delay">

<rdfs:domain rdf:resource="#QOS_LIST"/>

<rdfs:range rdf:resource="#COS"/>

</owl:ObjectProperty>

<owl:ObjectProperty rdf:ID="cos_max_packet_transfer_delay">

<rdfs:domain rdf:resource="#QOS_LIST"/>

<rdfs:range rdf:resource="#COS"/>

</owl:ObjectProperty>

<owl:ObjectProperty rdf:ID="cos_packet_transfer_delay_jitter">

<rdfs:domain rdf:resource="#QOS_LIST"/>

<rdfs:range rdf:resource="#COS"/>

</owl:ObjectProperty>

<owl:ObjectProperty rdf:ID="cos_packet_loss_rate">

<rdfs:domain rdf:resource="#QOS_LIST"/>

<rdfs:range rdf:resource="#COS"/>

</owl:ObjectProperty>

<owl:DatatypeProperty rdf:ID="type_ie_network_data_rate">

<mihbasic:ie_type_identifier>0x10000107</mihbasic:ie_type_identifier>

<rdfs:domain rdf:resource="#NETWORK"/>

<rdfs:range rdf:resource="&xsd;unsignedInt"/>

</owl:DatatypeProperty>

<owl:ObjectProperty rdf:ID="type_ie_network_ip_config_methods">

<mihbasic:ie_type_identifier>0x10000108</mihbasic:ie_type_identifier>

<rdfs:subPropertyOf rdf:resource="#bit_number"/>

<rdfs:domain rdf:resource="#NETWORK"/>

<rdfs:comment>

The range of #bit_number is 0-31.

</rdfs:comment>

</owl:ObjectProperty>

<owl:ObjectProperty rdf:ID="type_ie_network_capabilities">

<mihbasic:ie_type_identifier>0x10000109</mihbasic:ie_type_identifier>

<rdfs:subPropertyOf rdf:resource="#bit_number"/>

<rdfs:domain rdf:resource="#NETWORK"/>

<rdfs:comment>

The range of #bit_number is 0-31.

</rdfs:comment>

</owl:ObjectProperty>

<owl:DatatypeProperty rdf:ID="type_ie_supported_lcp">

<mihbasic:ie_type_identifier>0x1000010A</mihbasic:ie_type_identifier>

<rdfs:domain rdf:resource="#NETWORK"/>

<rdfs:range rdf:resource="&xsd;unsignedByte"/>

</owl:DatatypeProperty>

<owl:ObjectProperty rdf:ID="type_ie_container_poa">

<mihbasic:ie_type_identifier>0x10000302</mihbasic:ie_type_identifier>

<rdfs:domain rdf:resource="#NETWORK"/>

<rdfs:range rdf:resource="#POA"/>

</owl:ObjectProperty>

<owl:Class rdf:ID="POA">

<rdfs:subClassOf>

<owl:Restriction>

<owl:onProperty rdf:resource="#type_ie_poa_mac_address"/>

<owl:cardinality rdf:datatype="&xsd;nonNegativeInteger">1

</owl:cardinality>

</owl:Restriction>

</rdfs:subClassOf>

<rdfs:subClassOf>

<owl:Restriction>

<owl:onProperty rdf:resource="#type_ie_poa_location"/>

<owl:cardinality rdf:datatype="&xsd;nonNegativeInteger">1

</owl:cardinality>

</owl:Restriction>

</rdfs:subClassOf>

<rdfs:subClassOf>

<owl:Restriction>

<owl:onProperty rdf:resource="#type_ie_poa_channel_range"/>

<owl:cardinality rdf:datatype="&xsd;nonNegativeInteger">1

</owl:cardinality>

</owl:Restriction>

</rdfs:subClassOf>

<rdfs:subClassOf>

<owl:Restriction>

<owl:onProperty rdf:resource="#type_ie_poa_system_information"/>

<owl:cardinality rdf:datatype="&xsd;nonNegativeInteger">1

</owl:cardinality>

</owl:Restriction>

</rdfs:subClassOf>

<rdfs:subClassOf>

<owl:Restriction>

<owl:onProperty rdf:resource="#type_ie_poa_subnet_information"/>

<owl:minCardinality rdf:datatype="&xsd;nonNegativeInteger">1

</owl:minCardinality>

</owl:Restriction>

</rdfs:subClassOf>

<rdfs:comment>

This class contains all the information depicting a PoA.

</rdfs:comment>

</owl:Class>

<owl:ObjectProperty rdf:ID="type_ie_poa_mac_address">

<mihbasic:ie_type_identifier>0x10000302</mihbasic:ie_type_identifier>

<rdfs:domain rdf:resource="#POA"/>

<rdfs:range rdf:resource="#TRANSPORT_ADDRESS"/>

</owl:ObjectProperty>

<owl:ObjectProperty rdf:ID="type_ie_poa_location">

<mihbasic:ie_type_identifier>0x10000201</mihbasic:ie_type_identifier>

<rdfs:domain rdf:resource="#POA"/>

<rdfs:range rdf:resource="#LOCATION"/>

</owl:ObjectProperty>

<owl:Class rdf:ID="LOCATION">

</owl:Class>

<owl:Class rdf:ID="BINARY_GEO_LOCATION">

<rdfs:subClassOf rdf:resource="#LOCATION"/>

<rdfs:comment>

This class has properties that represent geographic coordinate.

The format is based on the Location Configuration Information (LCI)

defined in RFC 3825.

</rdfs:comment>

</owl:Class>

<owl:DatatypeProperty rdf:ID="la_res">

<rdfs:domain rdf:resource="#BINARY_GEO_LOCATION"/>

<rdfs:range rdf:resource="&xsd;unsignedByte"/>

</owl:DatatypeProperty>

<owl:DatatypeProperty rdf:ID="latitude">

<rdfs:domain rdf:resource="#BINARY_GEO_LOCATION"/>

<rdfs:range rdf:resource="&xsd;double"/>

</owl:DatatypeProperty>

<owl:DatatypeProperty rdf:ID="lo_res">

<rdfs:domain rdf:resource="#BINARY_GEO_LOCATION"/>

<rdfs:range rdf:resource="&xsd;unsignedByte"/>

</owl:DatatypeProperty>

<owl:DatatypeProperty rdf:ID="longitude">

<rdfs:domain rdf:resource="#BINARY_GEO_LOCATION"/>

<rdfs:range rdf:resource="&xsd;double"/>

</owl:DatatypeProperty>

<owl:DatatypeProperty rdf:ID="altitude_type">

<rdfs:domain rdf:resource="#BINARY_GEO_LOCATION"/>

<rdfs:range rdf:resource="&xsd;unsignedByte"/>

<rdfs:comment>

Following codes are defined:

1: Meters: in 2s-complement fixed-point 22-bit integer part with

8-bit fraction. If AT = 1, an AltRes value 0.0 would indicate

unknown altitude.  The most precise Altitude would have an AltRes

value of 30.  Many values of AltRes would obscure any variation

due to vertical datum differences.

2: Floors: in 2s-complement fixed-point 22-bit integer part with

8-bit fraction. AT = 2 for Floors enables representing altitude in

a form more relevant in buildings which have different

floor-to-floor dimensions.

</rdfs:comment>

</owl:DatatypeProperty>

<owl:DatatypeProperty rdf:ID="alt_res">

<rdfs:domain rdf:resource="#BINARY_GEO_LOCATION"/>

<rdfs:range rdf:resource="&xsd;unsignedByte"/>

<rdfs:comment>

Altitude resolution.  6 bits indicating the number of valid bits

in the altitude.  Values above 30 (decimal) are undefined and

reserved.

</rdfs:comment>

</owl:DatatypeProperty>

<owl:DatatypeProperty rdf:ID="altitude">

<rdfs:domain rdf:resource="#BINARY_GEO_LOCATION"/>

<rdfs:range rdf:resource="&xsd;double"/>

</owl:DatatypeProperty>

<owl:DatatypeProperty rdf:ID="datum">

<rdfs:domain rdf:resource="#BINARY_GEO_LOCATION"/>

<rdfs:range rdf:resource="&xsd;unsignedByte"/>

<rdfs:comment>

Following codes are defined:

1: WGS

2: NAD 83 (with associated vertical datum for North American

vertical datum for 1998)

3: NAD 83 (with associated vertical datum for Mean Lower Low Water

(MLLW))

</rdfs:comment>

</owl:DatatypeProperty>

<owl:DatatypeProperty rdf:ID="geo_lo_method">

<rdfs:domain rdf:resource="#BINARY_GEO_LOCATION"/>

<rdfs:range rdf:resource="&xsd;unsignedByte"/>

<rdfs:comment>

Way location information was derived or discovered:

0: GPS

1: Assisted GPS

2: Manual

3: Provided by DHCP

4: Triangulation

5-255: Reserved

</rdfs:comment>

</owl:DatatypeProperty>

<owl:Class rdf:ID="XML_GEO_LOCATION">

<rdfs:subClassOf rdf:resource="#LOCATION"/>

</owl:Class>

<owl:DatatypeProperty rdf:ID="xml_geo_location">

<rdfs:domain rdf:resource="#XML_GEO_LOCATION"/>

<rdfs:range rdf:resource="&xsd;String"/>

<rdfs:comment>

Geo address elements as described in RFC4119.

</rdfs:comment>

</owl:DatatypeProperty>

<owl:Class rdf:ID="BINARY_CIVIC_LOCATION">

<rdfs:subClassOf rdf:resource="#LOCATION"/>

<rdfs:comment>

This class has properties that represent civic address.

The format is defined in IETF RFC 4676.

</rdfs:comment>

</owl:Class>

<owl:DatatypeProperty rdf:ID="civic_country_code">

<rdfs:domain rdf:resource="#BINARY_CIVIC_LOCATION"/>

<rdfs:range rdf:resource="&xsd;string"/>

<rdfs:comment>

Two-letter ISO 3166 country code in capital ASCII letters.

</rdfs:comment>

</owl:DatatypeProperty>

<owl:ObjectProperty rdf:ID="civic_addr_elem">

<rdfs:domain rdf:resource="#BINARY_CIVIC_LOCATION"/>

<rdfs:range rdf:resource="#CIVIC_ADDR_ELEM"/>

<rdfs:comment>

This property contains the civic address elements.

The format of the civic address elements is described

in Section 3.4 of IETF RFC 4676 with a TLV pair

(whereby the Type and Length fields are one octet long).

</rdfs:comment>

</owl:ObjectProperty>

<owl:Class rdf:ID="CIVIC_ADDR_ELEM">

<rdfs:subClassOf>

<owl:Restriction>

<owl:onProperty rdf:resource="#catype"/>

<owl:cardinality rdf:datatype="&xsd;nonNegativeInteger">1

</owl:cardinality>

</owl:Restriction>

</rdfs:subClassOf>

<rdfs:subClassOf>

<owl:Restriction>

<owl:onProperty rdf:resource="#cavalue"/>

<owl:cardinality rdf:datatype="&xsd;nonNegativeInteger">1

</owl:cardinality>

</owl:Restriction>

</rdfs:subClassOf>

</owl:Class>

<owl:DatatypeProperty rdf:ID="catype">

<rdfs:domain rdf:resource="#CIVIC_ADDR_ELEM"/>

<rdfs:range rdf:resource="&xsd;unsignedByte"/>

<rdfs:comment>

A one-octet descriptor of the data civic address value.

</rdfs:comment>

</owl:DatatypeProperty>

<owl:DatatypeProperty rdf:ID="cavalue">

<rdfs:domain rdf:resource="#CIVIC_ADDR_ELEM"/>

<rdfs:range rdf:resource="&xsd;string"/>

<rdfs:comment>

The civic address value.

</rdfs:comment>

</owl:DatatypeProperty>

<owl:Class rdf:ID="XML_CIVIC_LOCATION">

<rdfs:subClassOf rdf:resource="#LOCATION"/>

</owl:Class>

<owl:DatatypeProperty rdf:ID="xml_civic_location">

<rdfs:domain rdf:resource="#XML_CIVIC_LOCATION"/>

<rdfs:range rdf:resource="&xsd;String"/>

<rdfs:comment>

Geo address elements as described in RFC4119.

</rdfs:comment>

</owl:DatatypeProperty>

<owl:ObjectProperty rdf:ID="type_ie_poa_channel_range">

<mihbasic:ie_type_identifier>0x10000202</mihbasic:ie_type_identifier>

<rdfs:domain rdf:resource="#POA"/>

<rdfs:range rdf:resource="#CHANNEL_RANGE"/>

</owl:ObjectProperty>

<owl:Class rdf:ID="CHANNEL_RANGE">

<rdfs:subClassOf>

<owl:Restriction>

<owl:onProperty rdf:resource="#low_channel_range"/>

<owl:cardinality rdf:datatype="&xsd;nonNegativeInteger">1

</owl:cardinality>

</owl:Restriction>

</rdfs:subClassOf>

<rdfs:subClassOf>

<owl:Restriction>

<owl:onProperty rdf:resource="#high_channel_range"/>

<owl:cardinality rdf:datatype="&xsd;nonNegativeInteger">1

</owl:cardinality>

</owl:Restriction>

</rdfs:subClassOf>

</owl:Class>

<owl:DatatypeProperty rdf:ID="low_channel_range">

<rdfs:domain rdf:resource="#CHANNEL_RANGE"/>

<rdfs:range rdf:resource="&xsd;unsignedInt"/>

<rdfs:comment>

Lowest channel frequency in MHz

</rdfs:comment>

</owl:DatatypeProperty>

<owl:DatatypeProperty rdf:ID="high_channel_range">

<rdfs:domain rdf:resource="#CHANNEL_RANGE"/>

<rdfs:range rdf:resource="&xsd;unsignedInt"/>

<rdfs:comment>

Highest channel frequency in MHz

</rdfs:comment>

</owl:DatatypeProperty>

<owl:ObjectProperty rdf:ID="type_ie_poa_system_information">

<mihbasic:ie_type_identifier>0x10000203</mihbasic:ie_type_identifier>

<rdfs:domain rdf:resource="#POA"/>

<rdfs:range rdf:resource="#SYSTEM_INFORMATION"/>

</owl:ObjectProperty>

<owl:Class rdf:ID="SYSTEM_INFORMATION">

</owl:Class>

<owl:ObjectProperty rdf:ID="type_ie_poa_subnet_information">

<mihbasic:ie_type_identifier>0x10000204</mihbasic:ie_type_identifier>

<rdfs:domain rdf:resource="#POA"/>

<rdfs:range rdf:resource="#IP_SUBNET_INFO"/>

</owl:ObjectProperty>

<owl:Class rdf:ID="IP_SUBNET_INFO">

<rdfs:subClassOf>

<owl:Restriction>

<owl:onProperty rdf:resource="#subnet_address"/>

<owl:cardinality rdf:datatype="&xsd;nonNegativeInteger">1

</owl:cardinality>

</owl:Restriction>

</rdfs:subClassOf>

<rdfs:subClassOf>

<owl:Restriction>

<owl:onProperty rdf:resource="#ip_prefix_len"/>

<owl:cardinality rdf:datatype="&xsd;nonNegativeInteger">1

</owl:cardinality>

</owl:Restriction>

</rdfs:subClassOf>

</owl:Class>

<owl:ObjectProperty rdf:ID="subnet_address">

<rdfs:domain rdf:resource="#IP_SUBNET_INFO"/>

<rdfs:range rdf:resource="#TRANSPORT_ADDRESS"/>

<rdfs:comment>

An IP address of the PoA encoded as Address base type defined in

RFC3588. The first 2-octet contains AddressType, which may be

either 1 (IPv4) or 2 (IPv6). If AddressType==1, the subnet_address

property contains a 4-octet IPv4 address. If AddressType==2, the

subnet_address property contains a 16-octet IPv6 address.

</rdfs:comment>

</owl:ObjectProperty>

<owl:DatatypeProperty rdf:ID="ip_prefix_len">

<rdfs:domain rdf:resource="#IP_SUBNET_INFO"/>

<rdfs:range rdf:resource="&xsd;unsignedByte"/>

<rdfs:comment>

The bit length of the prefix of the subnet to which subnet_address

property belongs. The prefix_length is less than or equal to 32

for IPv4 subnet and less than or equal to 128 for IPv6 subnet.

</rdfs:comment>

</owl:DatatypeProperty>

<owl:ObjectProperty rdf:ID="type_ie_poa_ip_address">

<mihbasic:ie_type_identifier>0x10000205</mihbasic:ie_type_identifier>

<rdfs:domain rdf:resource="#POA"/>

<rdfs:range rdf:resource="#TRANSPORT_ADDRESS"/>

</owl:ObjectProperty>

<owl:Class rdf:ID="TRANSPORT_ADDRESS">

<rdfs:subClassOf>

<owl:Restriction>

<owl:onProperty rdf:resource="#address_type"/>

<owl:cardinality rdf:datatype="&xsd;nonNegativeInteger">1

</owl:cardinality>

</owl:Restriction>

</rdfs:subClassOf>

<rdfs:subClassOf>

<owl:Restriction>

<owl:onProperty rdf:resource="#address_value"/>

<owl:cardinality rdf:datatype="&xsd;nonNegativeInteger">1

</owl:cardinality>

</owl:Restriction>

</rdfs:subClassOf>

</owl:Class>

<owl:DatatypeProperty rdf:ID="address_type">

<rdfs:domain rdf:resource="#TRANSPORT_ADDRESS"/>

<rdfs:range rdf:resource="&xsd;unsignedShort"/>

<rdfs:comment>

An Address Family defined in

http://www.iana.org/assignments/address-family-numbers.

</rdfs:comment>

</owl:DatatypeProperty>

<owl:DatatypeProperty rdf:ID="address_value">

<rdfs:domain rdf:resource="#TRANSPORT_ADDRESS"/>

<rdfs:range rdf:resource="&xsd;hexBinary"/>

<rdfs:comment>

An address value specific to address_type.

</rdfs:comment>

</owl:DatatypeProperty>

</rdf:RDF>

# How to create an Extended Schema

In this subclause, how to create an extended schema is described. How to create “IP address of Mobile IP home agent” is used as an example.

## Define extended schema

It is possible to support an Extended Schema without defining an additional IEs or TLVs (including vendor specific ones) other than that are defined in this specification. An extended schema can be defined as an XML document. An example Extended Schema definition is shown as follows.

<?xml version="1.0"?>

<!DOCTYPE rdf:RDF [

<!ENTITY rdf "http://www.w3.org/1999/02/22-rdf-syntax-ns#">

<!ENTITY rdfs "http://www.w3.org/2000/01/rdf-schema#">

<!ENTITY mihbasic "URL_TO_BE_ASSIGNED">

<!ENTITY mihextended "http://www.informationservice.org/2006/08/extended-schema#">

<!ENTITY owl "http://www.w3.org/2002/07/owl#">

<!ENTITY xsd "http://www.w3.org/2001/XMLSchema#">

]>

<rdf:RDF xmlns:rdf="&rdf;" xmlns:rdfs="&rdfs;" xmlns:mihbasic="&mihbasic;"

xmlns:mihextended="&mihextended;" xml:base="&mihextended;" xmlns:owl="&owl;" xmlns:xsd="&xsd;">

<owl:Ontology rdf:about="">

<rdfs:label>Extended Schema</rdfs:label>

</owl:Ontology>

<owl:ObjectProperty rdf:ID="ha_address">

<rdfs:domain rdf:resource="&mihbasic;NETWORK"/>

<rdfs:range rdf:resource="&mihbasic;TRANSPORT_ADDRESS"/>

</owl:ObjectProperty>

</rdf:RDF>

Figure XX Extended Schema

In this example, the new property of “NETWORK” class, “ha_address” is created in the Extended Schema where “ha_address” has the same format as “TRANSPORT_ADDRESS” class.