| Project | IEEE: 802.21 MIHO <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | MIH Acknowledgement |  |
| Date Submitted | January 03, 2007 |  |
| Source(s) | Miriam Tauil, Y. Alice Cheng, Subir Das, Yoshihiro Ohba |  |
| Re: | IEEE 802.21 Session #18 in January 2007 |  |
| Abstract | This document explains the MIH ACK behavior |  |
| Purpose | Complement contributions related to comment 2339 for LB #1a |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.21. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

# Suggested text changes from line 55, page 167,  to reflect comment # 2339 from LB#1a

MIH messages require reliability for remote communication on an end-to-end basis to ensure the receipt of data to the intended destination. It is particularly useful when the underlying transport used for remote communication does not provide reliable services.  Reliability may be provisioned with the optional acknowledgement mechanism as part of the MIH protocol. The MIH acknowledgement mechanism is particularly useful when the underlying transport used for remote communication does not provide reliable services. When the MIH transport layer is reliable, acknowledgement may not be used; however it requires dependency on MIH protocol to know the underlying transport.

The source end point may optionally request for an MIH ACK message to ensure successful receipt of a certain event, command or an information service message. MIH level ACK packet is used to acknowledge the successful receipt of MIH messages at the destination end point. When the MIH ACK is received by the source, it may conclude that the message was reliably delivered to the destination. If the message or the MIH ACK is lost, the source shall time-out and shall retransmit the same MIH message.