| Project | IEEE 802.21 MIHS <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | Available Resource Response |  |
| DCN | 21-07-0xxx-00-0000 |  |
| Date Submitted | January 8, 2007 |  |
| Source(s) | Yan PENG, Jing LIU, Bin XIA |  |
| Re: | IEEE 802.21 Session 18# in London, U.K. |  |
| Abstract | The contribution clarifies the parameter definition about the available resource in MIH_Net_HO_Candidate_Query primitives. |  |
| Purpose |  |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that IEEE 802.2 may make this contribution public. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

# Introduction

During the candidate query phase, the serving network perhaps needs to tell the MN of available resources list on new link(s).

The proposal clarifies the parameter definition about the available resource in MIH_Net_HO_Candidate_Query primitives. We suggest changing the AvailableResourceList parameter into AvailableResourceSet parameter, which contains a set of "LinkIdentifier" and their corresponding "AvailableResourceList": the former is used to identify the link; the latter is a list of resources actually available at the suggested link.

# Problem Statement

In current D3.0, several preferred new links could be returned to MN in the "PreferredLinkList" parameter of MIH_Net_HO_Candidate_Query.confirm primitive.

The returned "AvailableResourceList" should contain available resources. However, they are not associated with any network/link ID, so it makes the MN cannot tell which link the resources belong to, and cannot make appropriate handover decision based on the such unclear returned information.

The same problem exists for MIH_Net_HO_Candidate_Query.response primitive.

# Suggested Remendy

[Modify the text as suggested below: Page 120, line 56]

7.6.19.3 MIH_Net_HO_Candidate_Query.response

7.6.19.3.1 Function

7.6.19.3.2 Semantics of service primitive

MIH_Net_HO_Candidate_Query.response (

SourceIdentifier,

DestinationIdentifier,

CurrentLinkIdentifier,

HandoverAck,

PreferredLinkList,

PreferredPoAList,

ErrorCode,

AvailableResourceSet,

QueryResourceList

)

Local or Remote: Both

MIHF (Mobile Node) <> MIHF (Network)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| SourceIdentifier | Identifier | Any valid individual or group identifier | The identifier of entity where the request is initiated. This field may be optionally left empty if the command is local. |
| DestinationIdentifier | Identifier | Valid MIHF identifier | The destination identifier of request or response. This is the identifier of local or peer MIHF. |
| CurrentLinkIdentifier | NetworkIdentifier. May be one of different 802 and Cellular networks. |  | This identifies the current access network over which the command needs to be sent. This is valid only for remote commands which need to be sent to remote MIHF. The command is then sent either at L2 or at L3. |
| HandoverAck | Boolean | N/A | 1: Initiate Handover 0: Abort Handover If the handover has to be aborted then a reason code is provided |
| PreferredLinkList | NetworkIdentifier. Can be different 802 and Cellular networks | N/A | This is the identifier list of new networks to which handover needs to be initiated. This may be different than the networks that were suggested in the handover request. The front is most suitable, backward is less. |
| PreferredPoAList | MAC_ADDRESS (Optional) | N/A | This is the Suggested Point of Attachment List (AP/BS) on new networks. The front is most suitable, backward is less. |
| ErrorCode | Enumerate | N/A | Lists the reason for aborting/declining the handover request. |
| AvailableResourceSet | Set of LinkIdentifier parameters and their available resource statuses | N/A | Containing a set of LinkIdentifier parameters and their corresponding available resource on candidate network(s): LinkIdentifier AvailableResourceList |
| QueryResourceList | List | N/A | List of resources to be queried at candidate networks. This filed is used in case of the network-initiated handover. |

[Modify the text as suggested below: Page 122, line 23]

7.6.19.4 MIH_Net_HO_Candidate_Query.confirm

7.6.19.4.1 Function

7.6.19.4.2 Semantics of service primitive

MIH_Net_HO_Candidate_Query.confirm (

SourceIdentifier,

DestinationIdentifier,

CurrentLinkIdentifier,

HandoverAck,

PreferredLinkList,

PreferredPoAList,

ErrorCode,

AvailableResourceSet,

Status,

)

Local or Remote: Both

MIHF (Mobile Node) <> MIHF (Network)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Source Identifier | Identifier | Any valid individ ual or group iden tifier | The identifier of entity where the request is initiated. This field may be optionally left empty if the command is local. |
| Destination Identifier | Identifier | MIH_LOCAL, MIH_REMOTE | The destination identifier of request or response. This is the identifier of local or peer MIH Function. |
| CurrentLinkIdentifier | NetworkIdentifier. May be one of different 802 and Cellular networks. | N/A | This identifies the current access network over which the command needs to be sent. This is valid only for remote commands which need to be sent to remote MIHF. The command is then sent either at L2 or at L3. |
| HandoverAck | Boolean | N/A | 1: Initiate Handover 0: Abort Handover If the handover has to be aborted then a reason code is provided |
| PreferredLinkList | NetworkIdentifier. Can be different 802 and Cellular net works | N/A | This is the identifier list of new networks to which handover needs to be initiated. This may be different than the networks that were suggested in the handover request. The front is most suitable, backward is less. |
| PreferredPoAList | MAC_ADDRESS (Optional) | N/A | This is the Suggested Point of Attachment List (AP/BS) on new networks. The front is most suitable, backward is less. |
| ErrorCode | Enumerate | N/A | Lists the reason for aborting/declining the handover request. |
| AvailableResourceSet | Set of LinkIdentifier parameters and their available resource statuses | N/A | Containing a set of LinkIdentifier parameters and their corresponding available resource on candidate network(s): LinkIdentifier AvailableResourceList |
| Status | Enumerated | Success Error | Status of operation |