| Project | IEEE 802.21 MIHS <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | Suggestions about Handover Candidate Query |  |
| DCN | 21-07-0xxx-00-0000 |  |
| Date Submitted | January 5, 2007 |  |
| Source(s) | Yan PENG, Jing LIU, Bin XIA |  |
| Re: | IEEE 802.21 Session 18# in London, U.K. |  |
| Abstract | The contribution amends the inconsistency of parameter definition between the MIH_Net_HO_Candidate_Query primitives and messages. |  |
| Purpose |  |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that IEEE 802.2 may make this contribution public. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

# Introduction

In the current draft, the parameters of primitives and corresponding messages during the MIH handover initialization phase are inconsistent.

The proposal clarifies the parameter definition to avoid the inconsistency between the MIH_Net_HO_Candidate_Query primitives and messages.

# Problem Statement

In the case of Network-initiated handover, the two MIHFs may negotiate the resources by exchanging the MIH_Net_HO_Candidate_Query messages and corresponding primitives. But the parameters in these messages are inconsistent with their corresponding primitives. For example, resource information is contained in MIH_Net_HO_Candidate_Query.request/response primitives, but the corresponding messages do not carry any information about resources, so that the two peers can't communicate resources information.

# Suggested Remendy

[Modify the text as suggested below: Page 119, line 26]

7.6.19.2 MIH_Net_HO_Candidate_Query.indication

7.6.19.2.1 Function

7.6.19.2.2 Semantics of service primitive

MIH_Net_HO_Candidate_Query.indication	(

SourceIdentifier,

DestinationIdentifier,

CurrentLinkIdentifier.

SuggestedNewLinkList,

SuggestedNewPoAList,

HandoverMode,

OldLinkAction,

QueryResourceList,

QueryResourceFlag

)

Local or Remote: Both

MIHF (Mobile Node) <> MIHF (Network)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Source Identifier | Identifier | Any valid individ ual or group iden tifier | The identifier of entity where the request is initiated. This field may be optionally left empty if the command is local. |
| Destination Identifier | Identifier | MIH_LOCAL, MIH_REMOTE | The destination identifier of request or response. This is the identifier of local or peer MIH Function. |
| CurrentLinkIdentifier | NetworkIdenti fier. May be one of different 802 and Cellular networks. | N/A | This identifies the current access net work over which the command needs to be sent. This is valid only for remote commands which need to be sent to remote MIHF. The command is then sent either at L2 or at L3. |
| SuggestedNewLinkList | NetworkIdentifier. Can be different 802 and Cellular net works | N/A | This is the identifier List of new networks to which handover needs to be initiated. The front is most suitable, backward is less. |
| SuggestedNewPoAList | MAC_ADDRESS (Optional) | N/A | This is the Suggested Point of Attachment List (AP/BS) on new networks. The front is most suitable, backward is less. |
| HandoverMode | Enumerated | Make-before-Break Break-before-Make | The handover mode may influence the manner in which links are prepared for handover |
| OldLinkAction | Link Action (13) | Any valid value of Link Action (13) | Specifies the suggested action on link during handover. Combination of the below choices are allowed. Bit #0: LINK_DISCONNECT Bit #1: LINK_LOW_POWER Bit #2: LINK_POWER_DOWN Bit #3: LINK_NO_ACTION Bit #4: LINK_RESOURCE_RETAIN Bit #5: DATA_FORWARDING_REQUEST Bit #6-31: Reserved |
| QueryResourceList | List | N/A | List of resources to be queried at candidate networks. This field is used in case of the mobile-initiated handover. |
| QueryResourceFlag | Boolean | N/A | Flag to query the available list of resources on the new link. This flag is used in case of the network-initiated handover. |

[Modify the text as suggested below: Page 122, line 23]

7.6.19.4 MIH_Net_HO_Candidate_Query.confirm

7.6.19.4.1 Function

7.6.19.4.2 Semantics of service primitive

MIH_Net_HO_Candidate_Query.confirm (

SourceIdentifier,

DestinationIdentifier,

CurrentLinkIdentifier,

HandoverAck,

PreferredLinkList,

PreferredPoAList,

ErrorCode,

AvailableResourceList,

Status,

QueryResourceList

)

Local or Remote: Both

MIHF (Mobile Node) <> MIHF (Network)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Source Identifier | Identifier | Any valid individ ual or group iden tifier | The identifier of entity where the request is initiated. This field may be optionally left empty if the command is local. |
| Destination Identifier | Identifier | MIH_LOCAL, MIH_REMOTE | The destination identifier of request or response. This is the identifier of local or peer MIH Function. |
| CurrentLinkIdentifier | NetworkIdentifier. May be one of different 802 and Cellular networks. | N/A | This identifies the current access network over which the command needs to be sent. This is valid only for remote commands which need to be sent to remote MIHF. The command is then sent either at L2 or at L3. |
| HandoverAck | Boolean | N/A | 1: Initiate Handover 0: Abort Handover If the handover has to be aborted then a reason code is provided |
| PreferredLinkList | NetworkIdentifier. Can be different 802 and Cellular net works | N/A | This is the identifier list of new networks to which handover needs to be initiated. This may be different than the networks that were suggested in the handover request. The front is most suitable, backward is less. |
| PreferredPoAList | MAC_ADDRESS (Optional) | N/A | This is the Suggested Point of Attachment List (AP/BS) on new networks. The front is most suitable, backward is less. |
| ErrorCode | Enumerate | N/A | Lists the reason for aborting/declining the handover request. |
| AvailableResourceList | List | N/A | List of available sources on new link. This filed is used in case of the MN-initiated handover. This parameter can’t coexist with QueryResource. |
| Status | Enumerated | Success Error | Status of operation |
| QueryResourceList | List | N/A | List of resources to be queried at candidate networks. This filed is used in case of the network-initiated handover. |

[Modify the text as suggested below: Page 193, line 24]

8.5.3.15 MIH_Net_HO_Candidate_Query Request

This message is used for communication between MIHF on client with MIHF on network. The function is

used to communicate intent of handover initiation. The handover may be initiated either by the client or the

network.

| Name | Type |
| --- | --- |
| SuggestedNewLinkIdentifier | New Link Identifier (15) |
| SuggestedMACNewPoAIdentifier | PoA MAC Address (8) |
| HandoverMode | Handover Mode (27) |
| OldLinkAction | Link Action (28) |
| QueryResourceList | Link Resource Parameters Type (34) |
| QueryResourceFlag | Query Resource Flag (50) |

[Add new TLV: Page 175, line 29]

| 49 | MIIS TLV Response |
| --- | --- |
| 50 | Query Resource Flag |

[Add new subsection 8.4.1.50 for added new TLV: Page 186, line 17]

8.4.1.50 Query Resource Flag

This parameter specifies whether to query the available list of resources on the new link. This is used in case of the network-initiated handover.

| Type | Length | Value |
| --- | --- | --- |
| 50 | 1 | 0: Not query the available resources list 1: Query the available resources list |