| Project | IEEE 802.21 MIHS <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | Suggestions about Handover Complete |  |
| DCN | 21-07-0xxx-00-0000 |  |
| Date Submitted | January 5, 2007 |  |
| Source(s) | Yan PENG, Jing LIU, Bin XIA |  |
| Re: | IEEE 802.21 Session 18# in London, U.K. |  |
| Abstract | The contribution amends the inconsistency of parameter definition between the MIH_N2N_HO_Complete primitives and messages. |  |
| Purpose |  |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that IEEE 802.2 may make this contribution public. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

# Introduction

In the current draft, the parameters of Handover completion primitives and corresponding messages are inconsistent.

The proposal clarifies the parameter definition to avoid the inconsistency between the MIH_N2N_HO_Complete primitives and messages.

# Problem Statement

After handover completion, the two MIHFs may exchange the status of handover and resources. But the parameters in these messages are inconsistent with their corresponding primitives. For example, these statuses are contained in MIH_N2N_HO_Complete messages, but absent in corresponding primitives. So that the two peers can't communicate relative status information.

# Suggested Remendy

[Modify the text as suggested below: Page 132, line 58]

7.6.25.1 MIH_N2N_HO_Complete.request

7.6.25.1.1 Function

7.6.25.1.2 Semantics of service primitive

MIH_N2N_HO_Complete.request	(

SourceIdentifier,

DestinationIdentifier,

CurrentLinkIdentifier,

HandoverStatus

)

Local or Remote: Both

MIHF (Mobile Node) <> MIHF (Network)

MIHF (Network) <> MIHF (Network)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Source Identifier | Identifier | Any valid individual or group identifier | The identifier of entity where the request is initiated. This field may be optionally left empty if the command is local. |
| Destination Identifier | Identifier | Valid MIHF identifier | The destination identifier of request or response. This is the identifier of local or peer MIHF. |
| CurrentLinkIdentifier | NetworkIdenti fier. May be one of different 802 and Cellular networks. | Identifier | This identifies the current access network over which the command needs to be sent. This parameter is included only for the case where the accompanying MIH_N2N_HO_Complete.request message is transferred between the mobile node and the Serving PoS. The command is then sent either at L2 or at L3. |
| HandoverStatus | Enumerated | 0: Success 1: Failure 2: Rejected | This identifies the status of handover. |

[Modify the text as suggested below: Page 133, line 55]

7.6.25.2 MIH_N2N_HO_Complete.indication

7.6.25.2.1 Function

7.6.25.2.2 Semantics of service primitive

MIH_N2N_HO_Complete.indication (

SourceIdentifier,

DestinationIdentifier,

CurrentLinkIdentifier,

HandoverStatus

)

Local or Remote: Both

MIHF (Mobile Node) <> MIHF (Network)

MIHF (Network) <> MIHF (Network)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Source Identifier | Identifier | Any valid individual or group identifier | The identifier of entity where the request is initiated. |
| Destination Identifier | Identifier | Valid MIHF identifier | The destination identifier of request. |
| CurrentLinkIdentifier | NetworkIdentifier. May be one of different 802 and Cellular networks. | Identifier | This identifies the current access net work over which the command needs to be sent. This parameter is included only for the case where the accompanying MIH_N2N_HO_Complete.request message is transferred between the mobile node and the Serving PoS. The command is then sent either at L2 or at L3. |
| HandoverStatus | Enumerated | 0: Success 1: Failure 2: Rejected | This identifies the status of handover. |

[Modify the text as suggested below: Page 134, line 43]

7.6.25.3 MIH_N2N_HO_Complete.response

7.6.25.3.1 Function

7.6.25.3.2 Semantics of service primitive

MIH_N2N_HO_Complete.response (

SourceIdentifier,

DestinationIdentifier,

CurrentLinkIdentifier,

ResourceStatus

)

Local or Remote: Both

MIHF (Mobile Node) <> MIHF (Network)

MIHF (Network) <> MIHF (Network)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Source Identifier | Identifier | Any valid individual or group identifier | The identifier of entity where the request is initiated. This field may be optionally left empty if the command is local. |
| Destination Identifier | Identifier | Valid MIHF identifier | The destination identifier of request or response. This is the identifier of local or peer MIHF. |
| CurrentLinkIdentifier | NetworkIdentifier. May be one of different 802 and Cellular networks. | Identifier | This identifies the current access network over which the command needs to be sent. This parameter is included only for the case where the accompanying MIH_N2N_HO_Complete.response message is transferred between the mobile node and the Serving PoS. The command is then sent either at L2 or at L3. |
| ResourceStatus | Boolean | N/A | 0: Retain resources 1: Release resources |

[Modify the text as suggested below: Page 135, line 38]

7.6.25.4 MIH_N2N_HO_Complete.confirm

7.6.25.4.1 Function

7.6.25.4.2 Semantics of service primitive

MIH_N2N_HO_Complete.confirm (

SourceIdentifier,

DestinationIdentifier,

CurrentLinkIdentifier,

ResourceStatus,

Status

)

Local or Remote: Both

MIHF (Mobile Node) <> MIHF (Network)

MIHF (Network) <> MIHF (Network)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Source Identifier | Identifier | Any valid individual or group identifier | The identifier of entity where the request is initiated. This field may be optionally left empty if the command is local. |
| Destination Identifier | Identifier | Valid MIHF identifier | The destination identifier of request or response. This is the identifier of local or peer MIHF. |
| CurrentLinkIdentifier | NetworkIdentifier. May be one of different 802 and Cellular networks. | Identifier | This identifies the current access network over which the command needs to be sent. This parameter is included only for the case where the accompanying MIH_N2N_HO_Complete.response message is transferred between the mobile node and the Serving PoS. The command is then sent either at L2 or at L3. |
| ResourceStatus | Boolean | N/A | 0: Retain resources 1: Release resources |
| Status | Enumerated | Success Error | Status of operation |