IEEE 802.21
Media Independent Handover Services

Enhancement to the MIH_Capability_Discovery Message

Date:	February 15, 2007

Authors:	Ulises Olvera-Hernandez

Juan Carlos Zuniga

InterDigital Communications

e-Mail:	 ulises.olvera-hernandez@interdigital.com

juancarlos.zuniga@interdigital.com

Abstract

The current MIH_Capability_Discover (specifically the MIH_Capability_Discover.confirm) message does not allow a full discovery of capabilities on an MIH enabled node. For example, even though the SupportedMihEventList parameter in the MIH_Capability_Discover.confirm message lists the supported commands of the MIHF peer, it does not specify which commands are specific to particular link types.

It is seen important that an MIHF node be able to specify, in details, the different commands supported on different links – simply because different links implement different capabilities.

The same is also true for Event capability discovery i.e. an MIHF should specify the different possible events that individual links can provide to enable full capability discovery.

In addition, it is not specified in the current message format how the SupportedISQueryTypeList parameter is to be used to portray the list of supported MIIS query types.

Moreover, the current MIH_Capability_Discovery message does not allow an MIHF node to learn about the number & type of links that its peer supports, and the handover mode that it can execute i.e. make-before-break (MBB) or break-before-make (BBM) on these links.

Also, currently missing is the ability to show the type of supported transport list.

The submission aims to provide enhancements to the MIH_Capability_Discover.confirm message to do the following:

- Inform about number of supported links

- Inform about the type of supported links and associated MAC addresses

- Inform about the supported even list for every link type

- Inform about the supported command list specific to every link type

- Show how the existing  SupportedISQueryTypeList parameter can be used

- Inform about the supported transport list

- Inform wether MBB and/or BBM handovers are supported on specific links

Proposal

#### Section 7.6.1.2.2 (‘TLV Representation of Information Elements’) – Page 53

7.6.1.2.2 Semantics of service primitive

MIH_Capability_Discover.confirm (

SourceIdentifier,

SupportedMihEventList,

SupportedMihCommandList,

SupportedISQueryTypeList,

Status

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Source Identifier | Identifier | Valid MIHF identifier | This is the MIHF ID of local or peer MIH entity that this primitive is originated. |
| NumberOfSupportedLinks | 8-bit unsigned integer | N/A | An 8-bit unsigned integer that represents the number of supported links in an MIH capable node |
| SupportedLinks | Bitmap | N/A | 64 bits field whose (TABLE 9) individual bit positions map to a specific link type. The following bitmap should be used starting from the most significant bit (Bit #0 and least significant bit would be Bit #15) If a bit is set to 1, then the corresponding link type is supported. If a bit is set to 0 then the corresponding link type is not supported. The number of bits that are set to 1 should be as many as the number indicated by the NumberOfSupportedLinks parameter. Bit #0 – Ethernet Bit #1 – Wireless: IEEE 802.11 Bit #2 – Wireless: IEEE 802.16 Bit #3 – Wireless: CDMA2000 Bit #4 – Wireless: UMTS Bit #5 – Wireless: CDMA2000-HRPD Bit #6 – Wireless: GSM Bit#7 to Bit#15 - Reserved for future use |
| LinksMACs | MAC address | N/A | This is a list of MAC addresses. The number of MAC addresses should be the same as the number of links available in the MIHF peer (determined by the NumberOfSupportedLinks parameter). Furthermore, the order of the individual MAC addresses in the list should correspond to the order of the bits that are set to 1 in the SupportedLinks parameter. |
| Supported MIH Event List | List | Set of supported events | List of supported events on MIHF. The encoding uses a bitmap where the bit position represents the MIH Event ID as defined in Table 25. A bit set to 1 indicates that event is supported. There should be an entry for every link type as specified by the NumberOfSupportedLinks parameter. Moreover, the order of each entry corresponds to the order of the individual bits that are set to 1 in the SupportedLinks parameter. |
| Supported MIH Command List | Bitmap | N/A | List of supported commands on MIHF. The encoding uses the following bitmap where a bit set to 1 indicates that command(s) is supported. Bit 0: MIH_Get_Status Bit 1: MIH_Switch Bit 2: MIH_Configure_Link Bit 3: MIH_Scan Bit 4: MIH_Net_HO_Candidate_Query MIH_Net_HO_Candidate_Commit MIH_N2N_HO_Query_Resources MIH_N2N_HO_Complete Bit 5: MIH_MN_HO_Candidate_Query MIH_MN_HO_Candidate_Commit MIH_MN_HO_Complete Bit 6: MIH_Network_Address_Information Bit 7-31: (Reserved) There should be an entry for every link type as specified by the NumberOfSupportedLinks parameter. Moreover, the order of each entry corresponds to the order of the individual bits that are set to 1 in the SupportedLinks parameter. |
| SupportedTransportList | Bitmap | N/A | 16 bits field whose first (i.e. leftmost or Bit #0) bit if set to 1 indicates that a connectionless transport mode is supported. The next 7 bits (i.e. Bit #1 to Bit #7) map to specific connectionless transport protocols. If Bit #8 is set to 1 then a connection oriented transport mode is supported and the following 7 bits (i.e. Bit #9 to Bit #15) map to specific connection oriented transport protocols Bit #0 – Indicates that connectionless transport mechanism is supported if set to 1 Bit #1 – UDP Bit #2 to Bit #7 – Reserved for other connectionless transport protocol Bit #8 – Indicates that connection oriented transport mechanism is supported if set to 1 Bit #9 – TCP Bit #10 to Bit #15 – Reserved for other connection oriented transport protocols |
| LinksMACs | MAC address | N/A | This is a list of MAC addresses. The number of MAC addresses should be the same as the number of links available in the MIHF peer (determined by the NumberOfSupportedLinks parameter). Furthermore, the order of the individual MAC addresses in the list should correspond to the order of the bits that are set to 1 in the SupportedLinks parameter. |
| Supported IS Query Type List | List | Set of supported MIIS query types | List of supported MIIS query types 64 bits field whose first (i.e. leftmost or Bit #0) bit if set to 1 indicates that a specific Information Service type is supported based on the mapping shown below: Bit #0 – TYPE_IE_NETWORK_TYPE Bit #1 – TYPE_IE_OPERATOR_IDENTIFIER Bit #2 - TYPE_IE_SERVICE_PROVIDER_IDENTIFIER Bit #3 - TYPE_IE_ACCESS_NETWORK_IDENTIFIER Bit #4 – TYPE_IE_ROAMING_PARTNERS Bit #5 - TYPE_IE_COST Bit #6 - TYPE_IE_NETWORK_STANDARDS Bit #7 - TYPE_IE_NETWORK_SECURITY Bit #8 - TYPE_IE_QOS Bit #9 - TYPE_IE_POA_ADDRESS Bit #10 - TYPE_IE_POA_LOCATION Bit #11 - TYPE_IE_POA_DATA_RATE Bit #12 - TYPE_IE_POA_CHANNEL_RANGE Bit #13 - TYPE_IE_POA_SUBNET_INFORMATION Bit #14- TYPE_IE_POA_CAPABILITIES Bit #15 to Bit #63 – Reserved for future and vendor specific use |
| Status | Enumerate | Success Error | Status of operation |
| MBBHandoverSupport | Bitmap | N/A | This is used to indicate if a make before break handover is supported. It is assumed that a break before make handover is supported by default. A bit field that has as many bits as there are in the SupportedLinks parameter. Thus, because the latter has 16 bits, the former parameter should have 16 bits as well. There should be as many entries for this field as there are supported links. The order of appearance of the entries or rows should correspond to the order of the bit positions that are set to 1 in the SupportedLinks parameter. Each of the 16 bits in a row specifies support for make before break (MBB) handover when handing over FROM a technology specified by the order of appearance of the row (or entry) in question, TO another technology specified by the positions of bits that are set to 1 in that row. If a bit position (in a row) maps to the same link type as that of the row (specified by the row’s order) then that bit position should be set to 1. Bit #0 – indicates if a MBB handover to 802.3 is supported Bit #1 – indicates if a MBB handover to 802.11 is supported Bit #2 – indicates if a MBB handover to 802.16 is supported Bit #3 – indicates if a MBB handover to CDMA2000 is supported Bit #4 – indicates if a MBB handover to UMTS is supported Bit #5 – indicates if a MBB handover to CDMA2000-HRPD is supported Bit #6 – indicates if a MBB handover to GSM is supported Bit #7 to Bit #15 – reserved for future use |