| Project | IEEE 802.21 MIHS <http://www.ieee802.org/21/> |
| --- | --- |
| Title | Proposal for the Vendor specific IE |
| DCN | 21-07-xxxx-00-0000 |
| Date Submitted |  |
| Source(s) | Kenichi Taniuchi, Yoshihiro Ohba |
| Re: | IEEE 802.21 Session #19 in Orlando, Florida |
| Abstract | This contribution describes about the proposal of vendor specific IE based on comment number 3099 in LB #1b. |
| Purpose | Letter Ballot 1c Recirculation |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that IEEE 802.2 may make this contribution public. |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |

# Problem

The range of Vendor specific IE is from 0x20000000 – 0x2FFFFFFF, the first half byte is constantly 2 and remaining 3.5 octets are variable. A straightforward approach is that these variable octets should be split into vendor ID and vendor specific variable space for each vendor ID. The well managed existing vendor ID can be reused to avoid duplicated work. Private Enterprise Numbers which is specified by IANA (http://www.iana.org/assignments/enterprise-numbers) would be useful. The number is assigned to iso.org.dod.internet.private.enterprise (1.3.6.1.4.1) in MIB Tree. Since the private enterprise number is OBJECT IDENTIFIER type, the length is 4 octets. But apparently current space of the IEs can not have 4 octets for it.

# Proposal

Define TYPE_IE_VENDOR_SPECIFIC as a new Information Element, and define vender ID, vender specific type and value in the value of TYPE_IE_VENDOR_SPECIFIC.

# Suggested remedy

- Modify 5.1 in Table-7 as below:

| 5.1 | Vendor Specific IEs TYPE_IE_VENDOR_SPECIFIC | Vendor Specific Services | Defined by individual vendors. 6.4.5.3.9 |
| --- | --- | --- | --- |

- Modify a paragraph and Table-8 in Section 6.4.4 in D04.00 as below:

The 802.21 specific Information Elements (Header and other Core 802.21) are assigned identifiers as per IEEE 802.21 standard. Please refer to Table C-1 (in Annex C) for more details. Vendors may specify their own IEs using the name space allocated to them.

Table 8-Information elements name space

| Range | Description | Comments |
| --- | --- | --- |
| 0x00000000 | Reserved |  |
| 0x00000001-0x000000FF | Reserved for IEEE 802.21 | These values are reserved for core IEEE 802.21 specific fields and shall not be used for IEs. Refer to Table 46 for TLV encoding. |
| 0x00000100-0x1FFFFFFF | Reserved for IEEE 802.21 | Core IEEE 802.21 specific IEs except header specific IEs. Refer to Table C-1 ion Annex C. |
| 0x20000000-0x2FFFFFFF | Vendor specific IE | Use for IEs defined by vendors. |
| 0x320000000-0x32FFFFFFF | Reserved for IEEE 802 Working Groups. | Reserved for other IEEE 802 WGs such as 802.11, 802.16 etc., if they want to define anything specific for heterogeneous handovers. |
| 0x321000000-0x321FFFFFF | Reserved for IETF. | Reserved for IETF protocols and other higher layer IEs. |
| 0x322000000-0x322FFFFFF | Reserved for playpen area. | May be used in development and testing. Should not be used in released products. Avoids collision during development. |
| 0x323000000-0xFFFFFFFF | Reserved | For future use |

- Add New Section as below:

6.4.5.4 Other Information Element

6.4.5.4.1 Vendor Specific IE

Vendors can specify their own IEs using Vendor Specific IE.

| Type=TYPE_IE_VENDOR_SPECIFIC | Length= Variable |
| --- | --- |
| Vendor ID [4 octets] (see Table 23) |  |
| Vendor Specific Type [4 octets] (see Table 23) |  |
| Vendor Specific Value [Variable] (see Table 23) |  |

Table 23 – Vendor Specific TLV

| Syntax | Length | Notes |
| --- | --- | --- |
| Vendor ID | 4 | A Private Enterprise Number managed by IANA. |
| Vendor Specific Type | 4 | A type specified by the Vendor identified by Vendor ID |
| Vendor Specific Value | Variable | A value specific to Vendor Specific Type |