| Project | IEEE 802.21 Media Independent Handover Services <http://www.ieee802.org/21/> |  |  |
| --- | --- | --- | --- |
| Title | MIH Capability Discovery |  |  |
| Date Submitted | March, 2007 |  |  |
| Source(s) | Subir Das Telcordia Technologies |  |  |
| Re: | IEEE 802.21 Session #19 in Caribe Royale Resort, Orlando |  |  |
| Abstract | This contribution proposes the MIH Capability Discovery Primitives |  |  |
| Purpose | Adoption |  |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.21. |  |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |  |

- Background

There are no MIH Capability Discovery ‘Response’ and ‘Indication’  primitives defined in Draft specification D04.

7.6.x  MIH_Capability_Discover

7.6.x.x MIH_Capability_Discover.indication

7.6.x.x.x  Function

This primitive is used by MIHF to notify the MIH Users

7.6.x.x.x  Semantics of Service primitive

MIH_Capability_Discover.Indication (

Source Identifier

Destination Identifier

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Source Identifier | Identifier | Any valid individual or group identifier | The identifier of entity where the request is initiated. |
| Destination Identifier | Identifier | Any valid individual or group identifier | The identifier of entity where the request is initiated. |

7.6.x.x.x When genrated

This primitive is used by MIHF to notify the MIH Users when a MIH Capability Discover.request is received

7.6.x.x.x Effect on receipt

MIHF users capable entities may respond with MIH Capability  Discover. Response primitive.

7.6.x.x. MIH_Capability_Discover.Response

This primitive is used by MIH Users to convey the capability of MIH functionality.

7.6.x.x.x Semantics of Service primitive

MIH_Capability_Discover.Response(

Source Identifier,

Destination Identifier,

SupportedMIHEvent List

SupportedMIHCommand List,

SupportedIS QueryType,

Status

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Source Identifier | Identifier | Any valid individual or group identifier | The identifier of entity where the request is initiated. This field may be optionally left empty if the command is local. |
| Destination Identifier | Identifier | Valid MIHF identifier | The destination identifier of request or response. This is the identifier of local or peer MIHF. |
| Supported MIH Event List | List | Set of supported event | List of supported events on MIHF. The list of events and their encodings are defined in Table 25 |
| Supported MIH Command List | List | Set of supported commands | List of supported commnads on MIHF. The list of events and their encodings are defined in Table 26 |
| Supported IS Query Type | Lsit | Set of supported IS Query Type list | List of supported query types on MIHF. The list of events and their encodings are defined in Table xx (As mentioned below) |
| Status | Enumerated | Success Error | Status of operation |

Note: Pl. use the above table for MIH_Capability_Discover.confirm just to be consistent

7.6.x.x.x When generated

This primitive is generated when MIH Capability Discover.indication primitive is received by MIHF

7.6.x.x.x Effect on receipt

Upon receiving this primitive, the MIHF  generates the corresponding response. However, if the Status indicates an error, the recipient shall ignore any other returned values and , instead, shall perform appropriate error handling..

Table XX – IS Query Type Identifiers

| IS Query Type | ID |
| --- | --- |
| TLV | 0 |
| RDF_DATA | 1 |
| RDF_SCHEMA_URL | 2 |
| RDF_SCHEMA | 3 |
| (Reserved) | 4-7 |