| Project | IEEE 802.21 Media Independent Handover Services <http://www.ieee802.org/21/> |  |  |
| --- | --- | --- | --- |
| Title | MIH Function Discovery |  |  |
| Date Submitted | March, 2007 |  |  |
| Source(s) | Subir Das Telcordia Technologies |  |  |
| Re: | IEEE 802.21 Session #19 in Caribe Royale Resort, Orlando |  |  |
| Abstract | This contribution proposes the MIH Function Discovery Primitives |  |  |
| Purpose | Adoption |  |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.21. |  |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |  |

- Background

There are no MIH function discovery primitives defined in Draft specification D04.

7.6.x  MIH_Function_Discover

7.6.x.x  MIH_Function_Discover.request

7.6.x.x.x  Function

This primitive is used by MIH Users to discover the MIH function in the network.

7.6.x.x.x  Semantics of Service primitive

MIH_Function_Discover.request (

Source Identifier

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Source Identifier | Identifier | Any valid individual or group identifier | The identifier of entity where the request is initiated. |

7.6.x.x.x When genrated

This primitive is generated by MIH Users that need to know the MIH function in the network

7.6.x.x.x Effect on receipt

MIHF capable entities may respond with MIH Function  Discover. Confirm primitive.

7.6.x.x. MIH_Function_Discover.Confirm

This primitive is used by MIHF to inform the  MIH Users  about the presence MIH function

7.6.x.x.x Semantics of Service primitive

MIH_Function_Discover.Confirm(

Source Identifier,

Destination Identifier

Network Identifier

Status

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Source Identifier | Identifier | Any valid individual or group identifier | The identifier of entity where the request is initiated. This field may be optionally left empty if the command is local. |
| Destination Identifier | Identifier | Valid MIHF identifier | The destination identifier of request or response. This is the identifier of local or peer MIHF. |
| Network Identifier | Network ID | N/A | Identifier of the network whose MIH function is discovered |
| Status | Enumerated | Success Error | Status of operation |

7.6.x.x.x When generated

This primitive is generated when MIH Function Discover.response primitive is received by MIHF

7.6.x.x.x Effect on receipt

Upon receiving  this primitive, the receiving entity becomes aware of MIH function.  However, if the Status indicates an error, the recipient shall ignore any other returned values and , instead, shall perform appropriate error handling..

7.6.x.x MIH_Function_Discover.indication

7.6.x.x.x  Function

This primitive is used by MIHF to notify the MIH Users

7.6.x.x.x  Semantics of Service primitive

MIH_Function_Discover.Indication (

Source Identifier

Destination Identifier

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Source Identifier | Identifier | Any valid individual or group identifier | The identifier of entity where the request is initiated. |
| Destination Identifier | Identifier | Any valid individual or group identifier | The identifier of entity where the request is initiated. |

7.6.x.x.x When genrated

This primitive is used by MIHF to notify the MIH Users when a MIH Function Discover.request is received

7.6.x.x.x Effect on receipt

MIHF users capable entities may respond with MIH Function  Discover. Response primitive.

7.6.x.x. MIH_Function_Discover.Response

This primitive is used by MIH Users to convey the presence of MIH functionality.

7.6.x.x.x Semantics of Service primitive

MIH_Function_Discover.Response(

Source Identifier,

Destination Identifier

Network Identifier

Status

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Source Identifier | Identifier | Any valid individual or group identifier | The identifier of entity where the request is initiated. This field may be optionally left empty if the command is local. |
| Destination Identifier | Identifier | Valid MIHF identifier | The destination identifier of request or response. This is the identifier of local or peer MIHF. |
| Network Identifier | Network ID | N/A | Identifier of the network whose MIH function is discovered |
| Status | Enumerated | Success Error | Status of operation |

7.6.x.x.x When generated

This primitive is generated when MIH Function Discover.indication primitive is received by MIHF

7.6.x.x.x Effect on receipt

Upon receiving this primitive, the MIHF  generates the corresponding response. However, if the Status indicates an error, the recipient shall ignore any other returned values and , instead, shall perform appropriate error handling..