| Project | IEEE 802.21 MIHO <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | Modification of IE TLV to IE Binary Encoding |  |
| Date Submitted | February 18, 2007 |  |
| Source(s) | Yuu-Heng Alice Cheng, Michael G. Williams, and Qiaobing Xie |  |
| Re: | IEEE 802.21 Session #19 in March 2007 |  |
| Abstract | This document contains the modification required for changing IE TLV representation to IE binary encoding representation. This is a response to Issue #12 assigned during IEEE 802.21 Session #18 in January 2007 |  |
| Purpose | LB#1c comment resolution, LB#1b issue #12 |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.21. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

# Introduction

The terminology for the Information Services encodings needs expanding, since there are a variety of datatypes being created. Although they are all encoded as TLV, simply referring to them as TLV isn’t descriptive and doesn’t help distinguish them. Also, other parts of the MIH protocol are encoded in TLV, but are not related to the IS, so revising the terms helps clarify the separation.

For the information elements, we propose to use “binary encoding” instead of “TLV.” The TLV query method will be renamed Binary query method.

Following are the concepts and datatypes that are clarified in this document:

- MIH protocol messages carry TLVs.

- Information Service Information Elements can be binary encoded.

- A “container” holds a group of IEs

- A “list” holds an ordered group of “containers”.

# Updates Required to D04-00

## Table of Contents

The table of content should be updated according to the title changes in the following subsections.

## Add acronyms in section 4.

IE	Information Element (p.47 Line 12)

## Changes in section 6.4

#### Page 48, Line 43

6.4.3 Information service elements IEs

#### Page 48, Line 46

The Information Service elements IEs are classified into three groups:

#### Page 48, Line 64

The TLV binary encoding representation of these Information Elements is described in 6.4.5.

#### Page 49, Line 1

The IEs may be retrieved using TLV binary encoding or resource description framework (RDF) based query mechanism.

#### Page 52, Line 9

Table 8—Information elements name space binary encoding range

#### Page 52, Line 17

These values are reserved for core IEEE 802.21 specific fields and shall not be used for IEs. Refer to Table 46 for TLV encoding.

#### Page 52, Line 41

6.4.5 TLV Binary encoding representation of information elements

#### Page 53, Line 20

Figure 22—TLV binary encoding representation of information elements

#### Page 60, Line 9

Table 16—Value field format of PoA location TLV binary encoding

#### Page 62, Line 20

Table 19—Value field format of PoA subnet information TLV binary encoding

#### Page 62, Line 38, 40

DHCP Server Address TLV IE [Variable]

FA/AR Address TLV IE [Variable]

#### Page 62, Line 44

An optional DHCP Server Address TLV binary encoding, as defined in Table 21 …

#### Page 63, Line 29

Table 21—DHCP server address TLV binary encoding

#### Page 63, Line 43

Table 22—FA/AR address TLV binary encoding

#### Page 64, Line 15

6.4.5.4 Containers for IEs containers

#### Page 64, Line 18

To effectively represent the heterogeneous neighborhood information in TLV binary encoding format, three Information Element Containers additional binary encodings are defined, namely List of Neighboring Access Networks Containers, Access Network Container, and PoA Container:

• List of Neighboring Access Networks Containers - contains a list of heterogeneous neighboring access networks for a given geophysical location.

A List of Neighboring Access Networks Container shall contain at least one Access Network and may contain one or more Vendor Specific IEs

#### Page 64, Line 29

Type = TYPE_IE_CONTAINER_LIST_OF_ACCESS_NETWORK_CONTAINERS

(i.e. TYPE_LIST_OF_ACCESS_NETWORK_CONTAINERS)

#### Page 64, Line 49

Type = TYPE_ ACCESS_NETWORK_CONTAINER

#### Page 65, Line 25

TYPE = TYPE_ POA_CONTAINER

#### Page 65, Line 57

TLVs for the component IEs containeding in the Access Network Container and PoA Container are defined in 6.4.5.1 to 6.4.5.3.

#### Page 66, Line 1

6.4.6 Information retrieval using TLV binary encoding representation

#### Page 66, Line 11

Information Request (from client to information server) shall carry an MIIS_BINARY_QUERYMIIS_TLV_QUERY TLV which contains:

| Type=MIIS_TLVBINARY_QUERY | Length=Variable |
| --- | --- |
| Querier Location TLV Container |  |
| Network Type Inclusion TLV Container |  |
| Network Inclusion TLV Container |  |
| Reporting Template TLV Container |  |

If the optional TLVs containers are absent then the Value field of the MIIS_TLVBINARY_QUERY TLV should be empty and thus the value of the Length field will be zero.

#### Page 66, Line 30

Should be moved to section 6.4.5.4 and reorganized as the structure in section 6.4.5.4

Querier Location TLV binary encoding is defined as follows:

| Type=MIIS_TLV_TYPE_CONTAINER_QUERIER_LOCATION | Length=Variable |
| --- | --- |
| Location TLV PoA Location IE |  |
| MAC Address TLV PoA MAC Address |  |
| Neighborhood Radius TLV Container |  |

All the following TLV naming should be modified as

| Origin Name | New Name |
| --- | --- |
| Neighborhood Radius TLV | Neighborhood Radius Container |
| Network Type Inclusion TLV | Network Type Inclusion Bitmap |
| Network Inclusion TLV | Network Inclusion Container |
| Reporting Template TLV | Reporting Template Container |

#### Page 67, Line 56 to Page 68, Line36

TYPE_IE_CONTAINER_LIST_OF_NETWORKS

#### Page 68, Line 38

Each of the IEs in the Requested IEs TLV is indicated by its 32 bit identifier as assigned in Table C-1.

#### Page 68, Line 41

Rules for using the Reporting Template TLV binary encoding

Dear Editor,

Please base on the previous examples and modify the rest of the section in 6.4.6.1

#### Page 69, Line 34

Information Response (from information server to client) shall carry a MIIS_TLVBINARY_RESPONSE TLV, which contains a list of returned binary encoding IEs and/or containers:

Dear Editor,

Please base on the previous examples and modify the rest of the section in 6.4.6.2

## Changes in Section 8h

#### Page 194, Line 15 (Table 46)

| 130 | MIIS TLV Binary Query |
| --- | --- |
| 131 | MIIS TLV Queries Location |
| 132 | MIIS TLV Report Template |
| 133 | MIIS TLV Binary Response |
| 134 | MIIS TLV Network Type Inlcusion |
| 135 | MIIS TLV Network Inclusion |
| 136 | MIIS TLV Neighborhood Radius |

Rules for using the Reporting Template TLV