| Project | IEEE 802.21 Media Independent Handover Services <http://www.ieee802.org/21/> |  |  |
| --- | --- | --- | --- |
| Title | Pre-acquisition of System Parameters |  |  |
| Date Submitted | March 6, 2007 |  |  |
| Source(s) | Jin Lee LG Electronics, Inc. 533,Hogye-1dong,Dongan-gu, Anyang-shi, Kyongki-do, Korea | Voice: +82-31-450-1856 Fax: +82-31-450-7912 [mailto: jins978@lge.com] |  |
| Re: | IEEE 802.21 Session #19 in Orlando |  |  |
| Abstract | This contribution proposes to achieve system parameters before media independent handover. |  |  |
| Purpose | Update 802.21 D04.00 draft to support seamless Media Independent Handover |  |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.21. |  |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |  |

- Pre-acquisition of System Parameters

Jin Lee 
LG Electronics, Inc.

1. Introduction

We have discussed effects of system parameters such as DCD and UCD in IEEE 802.16 handover performance. According to [1], reducing synchronization time in IEEE 802.16 system plays a key role to optimize handover performance. Also in [2], UE tries to achieve access parameters including system information block (SIB)s in order to provide a speedy initial access from its target eNB before handover execution.

Having said that, handover performance can be improved gracefully if system parameters for synchronization can be attained in advance. Therefore this contribution proposes system parameters in IEEE 802.16, 3gpp and 3gpp2 shall be obtained either through the information service or through handover commands before handover

2. Proposed Text Changes

# Remedy 1:

System parameter information elements shall be defined for information services.

[In 6.4.3 Information service elements, page 50, line 16, table 7 modified]:

Table 7 – Information elements

| PoA Specific Information Elements |  |  |  |
| --- | --- | --- | --- |
| No | Name of information elements | Description | Reference |
| 3.5 | Type_IE_POA_SystemParameters | System parameters supported by the link layer of a given PoA | 6.4.5.3.x System Parameters |

[In 6.4.5.3.x System Parameters, page 61, line 64 Inserted]:

6.4.5.3.x System Parameters

| Type= Type_IE_SystemParameters | Length = Variable |
| --- | --- |
| SystemParameter Value [Variable] –See Table xx |  |

Table xx

| Syntax | Length(Octet) | Notes |
| --- | --- | --- |
| SystemParameter Value | Variable | This value includes system parameters supported by a given PoA, this is independent on the network type. Network type : IEEE 802.16, 3gpp, 3gpp2 etc IEEE 802.16 – UCD, DCD, UIUC, DIUC 3GPP – Master Information Block, Scheduling blocks, System Information Block Types 3GPP2 – (Extended) System Parameters |

# Remedy 2:

System parameter information elements shall be carried within the current handover commands before handover.

[In 7.6.19.3.2 Semantics of service primitive, page 125, line 64, Inserted]:

7.6.19.3 MIH_Net_HO_Candidate_Query.response

7.6.19.3.2 Semantics of service primitive

The parameters of the primitive are as follows:

MIH_Net_HO_Candidate_Query.response (

DestinationIdentifier,

CurrentLinkIdentifier,

HandoverAck,

PreferredLinkList,

PreferredPoAList,

Errorcode,

AvailableResourceSet,

SystemParameterSet

)

[In 7.6.19.4.2 Semantics of service primitive, page 126, line 41, Inserted]:

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Destination Identifier | Identifier | Valid MIHF iden tifier | The destination identifier of request or response. This is the identifier of local or peer MIHF. |
| Current Link Identifier | NetworkIdentifier. May be one of differ ent 802 and Cellular networks. | N/A | This identifies the current access network over which the command needs to be sent. This is valid only for remote commands which need to be sent to remote MIHF. The command is then sent either at L2 or at L3. |
| Handover Ack | Boolean | N/A | True : Initiate Handover False : Abort Handover If the handover has to be aborted then a reason code is provided |
| Preferred Link List | NetworkIdentifier. Can be different 802 and Cellular net works | N/A | This is the identifier list of new networks to which handover needs to be initiated. This may be different than the networks that were suggested in the handover request. The front is most suitable, backward is less. |
| Preferred PoA List | MAC_ADDRESS (Optional) | N/A | This is the Suggested Point of Attachment List (AP/BS) on new networks. The front is most suitable, backward is less. |
| Error Code | Enumerate | N/A | Lists the reason for aborting/declining the handover request. |
| Available Resource Set | Set of Linkidenfier parameters and their available resource status | N/A | Containing a set of LinkIdentifier parameters and their corresponding available resource on candidate network(s): LinkIdentifier |
| SystemParameterSet | Set of system parameters | N/A | Containing a set of system parameters on preferred PoA(s): Preferred PoA System parameter |

[In 7.6.19.4.2 Semantics of service primitive, page 127, line 29, Inserted]:

7.6.19.4 MIH_Net_HO_Candidate_Query.confirm

7.6.19.4.2 Semantics of service primitive

The parameters of the primitive are as follows:

MIH_Net_HO_Candidate_Query.confirm (

DestinationIdentifier,

CurrentLinkIdentifier,

HandoverAck,

PreferredLinkList,

PreferredPoAList,

Errorcode,

Status,

SystemParameterSet

)

[In 7.6.19.4.2 Semantics of service primitive, page 127, line 66, Inserted]:

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Destination Identifier | Identifier | Valid MIHF iden tifier | The destination identifier of request or response. This is the identifier of local or peer MIHF. |
| Current LinkIdentifier | NetworkIdentifier. May be one of differ ent 802 and Cellular networks. | N/A | This identifies the current access network over which the command needs to be sent. This is valid only for remote commands which need to be sent to remote MIHF. The command is then sent either at L2 or at L3. |
| Handover Ack | Boolean | N/A | True : Initiate Handover False : Abort Handover If the handover has to be aborted then a reason code is provided |
| Preferred Link List | NetworkIdentifier. Can be different 802 and Cellular net works | N/A | This is the identifier list of new networks to which handover needs to be initiated. This may be different than the networks that were suggested in the handover request. The front is most suitable, backward is less. |
| Preferred PoA List | MAC_ADDRESS (Optional) | N/A | This is the Suggested Point of Attachment List (AP/BS) on new networks. The front is most suitable, backward is less. |
| Error Code | Enumerate | N/A | Lists the reason for aborting/declining the handover request. |
| Status | Enumerated | Success Error | Status of operation |
| SystemParameterSet | Set of system parameters | N/A | Containing a set of system parameters on preferred PoA(s): Preferred PoA System parameter |

[Same modification shall be adopted in : ]

MIH_MN_HO_Candidate_Query. response/confirm primitive

MIH_Net_HO_Candidate_Query response message

MIH_MN_HO_Candidate_Query response message

4. References

[1] 21-06-0524-01-0000-802_16_Parameter_Effects_on_Handover_Performance

[2] R2-071124 E-UTRAN Stage 2 TS 36300 v090

[3] 3GPP2 C.S0005-D Version 1.0