| Project | IEEE 802.21 Media Independent Handover Services <http://www.ieee802.org/21/> |  |  |
| --- | --- | --- | --- |
| Title | MIH Capability Discovery |  |  |
| Date Submitted | February 21, 2007 |  |  |
| Source(s) | Junghoon Jee, Eunah Kim ETRI 161 Gajeong-Dong, Yuseong-Gu Daejeon, 305-700, KOREA | Voice: +82-42-860-5126 Fax: +82-42-861-5404 {jhjee, eakim}@etri.re.kr |  |
| Re: | IEEE 802.21 Session #NN in Caribe Royale Resort, Orlando |  |  |
| Abstract | This contribution proposes the final resolution regarding the MIH Capability Discovery. Through the intensive discussion within the 802.21 WG, the SupportedMihEventList and SupportedMihCommandList parameters were included to the MIH_Capability_Discover.request primitive through the contribution, 21-05-0373-02-0000-MIH-Capability-Discovery. However, comments #3215 and #3218 suggested removing those parameters. This contribution provides the reason of including those parameters and suggests the final resolution not to revisit the previously resolved issue. |  |  |
| Purpose | Adoption |  |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.21. |  |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |  |

1. Background

21-05-0373-02-0000-MIH-Capability-Discovery suggested the addition of the SupportedMihEventList and SupportedMihCommandList parameters to the MIH_Capability_Discover.request primitive and the corresponding MIH message and it was incorporated to the IEEE P802.21. By the way, comments #3215 and #3218 from 21-07-0002-05-0000-LB_1b_Comments suggested removing those parameters.

Figure 1. MIH Capability Discovery procedure with current draft

2. MIH Capability Discovery in an efficient manner

MIH functional entity in the terminal and the network can identify each other’s capability only with one transaction by the addition of SupportedMihEventList and SupportedMihCommandList parameters to the MIH_Capability_Discover.request primitive and its corresponding MIH message. If the MIH_Capability_Disover.request parameter and the corresponding message does not contain the MIH message data, two capability discovery hand-shakes are required to recognize each other’s MIH capability.

Figure 2. MIH Capability Discovery procedure with the proposed scheme

3. Suggested Remedy

    - MIH_Capability_Discover

      - MIH_Capability_Discover.request

        - Function

This primitive is used by MIH Users to discover MIH capability of different links.

        - Semantics of service primitive

MIH_Capability_Discover.request			 	(

DestinationIdentifier,

SupportedMihEventList,

SupportedMihCommandList

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Destination Identifier | Identifier | Valid MIHF identifier | The destination identifier of request or response. This is the identifier of local or peer MIHF. |
| SupportedMihEventList | List | Set of supported events | List of supported events on the client. The encoding uses a bitmap where the bit position represents the MIH Event ID as defined in Table . A bit set to 1 indi cates that event is supported. |
| SupportedMihCommandList | List | Set of supported com mands | List of supported commands on the cli ent. The encoding uses a bitmap where the bit position represents the MIH Com mand ID as defined in Table . A bit set to 1 indicates that command is sup ported. |

        - When generated

This primitive is generated by MIH Users that need to determine MIH capability information of the link layers.

        - Effect on receipt

MIHF capable entities may respond with MIH_Capability_Discover.confirm primitive.