| Project | IEEE 802.21 MIHS <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | Multiple_PoAs_Under_Same_PoS |  |
| DCN | 21-07-0xxx-00-0000 |  |
| Date Submitted | February 5, 2007 |  |
| Source(s) | Yan PENG, Jing LIU, Bin XIA |  |
| Re: | IEEE 802.21 Session 19# in Florida,USA. |  |
| Abstract | The contribution brings forward a scenario where MIH PoS connects multiple PoAs, and suggests some remedy on MIH protocol messages to make it applicable in this scenario. |  |
| Purpose |  |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that IEEE 802.2 may make this contribution public. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

# Introduction

In the current draft, the location of an MIH PoS is not fixed by the standard. An MIH PoS may reside next to, or co-located with a PoA node in the access network. Alternatively the PoS may reside deeper inside the access or core networks.

In the latter case, possibly there are multiple PoAs under the control of the same PoS, which includes two cases. One is the PoAs connected with the same PoS have the same link type. The other is the PoAs have different link types. We can see as followed:

Figure 1  MIH PoS connects multiple PoAs which have the same link type

In figure 1, AP1 and AP2 are all WLAN PoA, which are under the control of same MIH PoS (AR). The similar relation exists in BS1, BS2 and ASN-GW.

ePDG

Evolved

Packet Core

GPRS Core

Trusted non 3GPP IP Access

WLAN

3GPP IP

Access

S2b

WLAN

Access NW

S5b

IASA

S5a

SAE

Anchor

3GPP

Anchor

S4

SGi

Evolved RAN

S1

Op.

IP

Serv.

(IMS,

PSS,

etc…)

Rx+

GERAN

UTRAN

Gb

Iu

S3

MME

UPE

HSS

PCRF

S7

S6

SGSN

S2a

Figure 2 3GPP SAE Logic Architecture

The figure above is 3GPP SAE (System Architecture Evolution) logic architecture. If we consider application of MIH in 3GPP SAE, the elliptical entities (Evolved RAN, Trusted non 3GPP IP Access and WLAN Access NW) are all MIH PoAs, and the MIH PoS may reside in 3GPP Anchor or SAE Anchor. This is the scenario that multiple PoAs under same MIH PoS have different link types

Regardless of any cases, currently some of the existing defined MIH protocol messages are not applicable for the scenario where MIH PoS connects multiple PoAs.

# Problem Statement

When the MIH PoS connects multiple PoAs, if it does not specify the link identifier in the exchanged messages between two MIHF entities, the receptor (i.e. the PoS in core network) cannot identify which link(PoA) the message is concerned. Therefore, it results in handover operation errors. This problem occurs during the following procedures:

- MIH_N2N_HO_Query_Resources

- MIH_Configure_Link

- MIH_Get_Status

Due to the absence of link identifier in the corresponding messages, the MIHF message receiver cannot identify which link need to be handled after receipt of these request messages, so it may bring error link handle process.

# Suggested Remendy

[Modify the text as suggested below: Page 132, line 54]

7.6.21.1 MIH_N2N_HO_Query_Resources.request

7.6.21.1.1 Function

7.6.21.1.2 Semantics of service primitive

MIH_N2N_HO_Query_Resources.request	(

DestinationIdentifier,

MNIdentifier,

CandidateLinkList,

QueryResourceList

)

Local or Remote: Both

MIHF (Network) <> MIHF (Network)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Destination Identifier | Identifier | Valid MIHF identifier | The destination identifier of request or response. This is the identifier of local or peer MIHF. |
| MNIdentifier | Link ID | N/A | Identify the MN this primitive is invoked for. |
| CandidateLinkList | List of Link Identifier | N/A | This is the Candidate Link List(AP/BS) on specific candidate network. In this list, each link may be expressed by link type and PoA MAC address. . |
| QueryResourceList | List | N/A | List of resources to be queried at the new candidate network. |

[Modify the text as suggested below: Page 133, line 47]

7.6.21.2 MIH_N2N_HO_Query_Resources.indication

7.6.21.2.1 Function

7.6.21.2.2 Semantics of service primitive

MIH_N2N_HO_Query_Resources.indication (

SourceIdentifier,

MNIdentifier,

CandidateLinkList,

QueryResourceList

)

Local or Remote: Both

MIHF (Network) <> MIHF (Network)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| SourceIdentifier | Identifier | Valid MIHF identifier | This is the MIHF ID of local or peer MIH entity that this primitive is originated. |
| MNIdentifier | Link ID | N/A | Identify the MN this primitive is invoked for. |
| CandidateLinkList | List of Link Identifier | N/A | This is the Candidate Link List(AP/BS) on specific candidate new network. In this list, each link may be expressed by link type and PoA MAC address. |
| QueryResourceList | List | N/A | List of resources to be queried at the new candidate network. |

[Modify the text as suggested below: Page 211, line 39]

8.5.3.1 MIH_Get_Status request

This message is used to discover the status of currently available links.

MIH Header Fixed Fields(SID=3, Opcode=1, AID=14)

Source Identifier = sending MIHF ID

Destination Identifier = receiving MIHF ID

GetStatusLinkIdentifier(14)

GetStatusRequest TLV(26)

[Modify the text as suggested below: Page 212, line 37]

8.5.3.5 MIH_Configure_Link request

This message is used to configure the link and control the link behavior.

MIH Header Fixed Fields(SID=3, Opcode=1, AID=16)

Source Identifier = sending MIHF ID

Destination Identifier = receiving MIHF ID

ConfigureLinkIdentifier(14)

ConfigureRequestSet TLV(29)

ConfigureReqSetParameters TLV(37)

[Modify the text as suggested below: Page 214, line 50]

8.5.3.13 MIH_N2N_HO_Query_Resources request

This message is used by the MIHF on the serving network to communicate with its peer MIHF on the candidate network. This is used to prepare the new link resource for impending handover and to query the available link resource of the candidate network.

MIH Header Fixed Fields(SID=3, Opcode=1, AID=20)

Source Identifier = sending MIHF ID

Destination Identifier = receiving MIHF ID

CandidateLinkList TLV

QueryResourceList TLV(34)

Note:  The parameter CandidateLinkList TLV is composed of a set of Candidate link identifiers, and each Candidate link identifier is also composed of Link Type and PoA MAC Address.