| Project | IEEE 802.21 MIHS <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | Multiple_PoAs_Under_Same_PoS |  |
| DCN | 21-07-0101-01-0000 |  |
| Date Submitted | July 13, 2007 |  |
| Source(s) | Yan PENG, Jing LIU, Bin XIA |  |
| Re: | IEEE 802.21 Session 21# in San Francisco . |  |
| Abstract | The contribution suggests adding LinkIdentifier to some MIH handover messages and primitives related, which is useful where MIH PoS connects multiple PoAs. |  |
| Purpose |  |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that IEEE 802.2 may make this contribution public. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

# Introduction

In the current draft, the location of an MIH PoS is not fixed by the standard. An MIH PoS may reside next to, or co-located with a PoA node in the access network. Alternatively the PoS may reside deeper inside the access or core networks.

In the latter case, possibly there are multiple PoAs under the control of the same PoS. Currently some of the existing defined MIH protocol messages are not applicable for the scenario where MIH PoS connects multiple PoAs.

# Problem Statement

When the MIH PoS connects multiple PoAs, if it does not specify the link identifier in the exchanged messages between two MIHF entities, the receiver (i.e. the PoS in core network) cannot identify which link(PoA) the message is concerned.

MIH_N2N_HO_Query_Resources

Due to the absence of link identifier in the corresponding messages, the MIHF message receiver cannot identify which link need to be handled after receipt of these request messages, so it may bring error link handle process.

# Suggested Remendy

[Modify the text as suggested below: Page 112, line 44]

7.4.20.1 MIH_N2N_HO_Query_Resources.request

7.4.20.1.1 Function

7.4.20.1.2 Semantics of service primitive

MIH_N2N_HO_Query_Resources.request	(

DestinationIdentifier,

CandidateLinkList,

QueryResourceList

)

Local or Remote: Both

MIHF (Network) <> MIHF (Network)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Destination Identifier | Identifier | Valid MIHF identifier | The destination identifier of request or response. This is the identifier of local or peer MIHF. |
| CandidateLinkList | List of Link Identifier | N/A | This is the Candidate Link List(AP/BS) on specific candidate network. In this list, each link may be expressed by link type and PoA MAC address. . |
| QueryResourceList | List | N/A | List of resources to be queried at the new candidate network. |

[Modify the text as suggested below: Page 113, line 43]

7.4.20.2 MIH_N2N_HO_Query_Resources.indication

7.4.20.2.1 Function

7.4.20.2.2 Semantics of service primitive

MIH_N2N_HO_Query_Resources.indication (

SourceIdentifier,

CandidateLinkList,

QueryResourceList

)

Local or Remote: Both

MIHF (Network) <> MIHF (Network)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| SourceIdentifier | Identifier | Valid MIHF identifier | This is the MIHF ID of local or peer MIH entity that this primitive is originated. |
| CandidateLinkList | List of Link Identifier | N/A | This is the Candidate Link List(AP/BS) on specific candidate new network. In this list, each link may be expressed by link type and PoA MAC address. |
| QueryResourceList | List | N/A | List of resources to be queried at the new candidate network. |

[Modify the text as suggested below: Page 214, line 50]

8.5.3.13 MIH_N2N_HO_Query_Resources request

This message is used by the MIHF on the serving network to communicate with its peer MIHF on the candidate network. This is used to prepare the new link resource for impending handover and to query the available link resource of the candidate network.

MIH Header Fixed Fields(SID=3, Opcode=1, AID=20)

Source Identifier = sending MIHF ID

Destination Identifier = receiving MIHF ID

CandidateLinkList TLV (List of Link PoA List TLV)

QueryResourceList TLV (Handover Resource Query List TLV)