Modify section 7.6.23 as follows:

7.6.23 MIH_MN_HO_Candidate_Commit

7.6.23.1 MIH_MN_HO_Candidate_Commit.request

7.6.23.1.1 Function

This primitive may be used by MIH users on a mobile node to inform MIHF to commit the handover to a specific target network. MIHF may forward the message remotely to the peer MIHF in the network in order to inform about the handover decision. The MN may, instead, continue the handover procedure utilizing IP mobility protocols after activating target L2 links natively.

7.6.23.1.2 Semantics of service primitive

The primitive is described as follows:

MIH_MN_HO_Candidate_Commit.request (

SourceIdentifier

DestinationIdentifier

CurrentLinkIdentifier

CurrentLinkAction

TargetLinkIdentifier

TargetLinkAction

TargetPoA

OldLinkActions

)

Local or Remote: Both

MIHF (Network) <>MIHF (Mobile Node)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Source Identifier | Identifier | Valid MIHF identi fier | The source of entity where the request is initiated. |
| Destination Identifier | Identifier | Valid MIHF identi fier | The destination identifier of request. |
| Current Link Identifier | Network Identifier.. | N/A May be one of different 802 or cellular networks | Identifier of currently active link This identifies the current access network over which the command needs to be sent. This is valid only for remote commands which need to be sent to remote MIHF. The command is then sent either at L2 or at L3. |
| Current Link Actions | Link Action | Any valid value of Link Action | Specifies the suggested action on link during handover. Combination of the following choices are allowed. Bit #0: LINK_DISCONNECT Bit #1: LINK_LOW_POWER Bit #2: LINK_POWER_DOWN Bit #3: LINK_NO_ACTION Bit #4: LINK_RESOURCE_RETAIN Bit #5: DATA_FORWARDING_REQUEST Bit #6: BI_CASTING_REQUEST Bit #7: HANDOVER_CANCEL Bit #8: LINK_POWER_UP Bit #9-31: Reserved |
| Target Link Identifier | Network Identifier | N/A Can be different 802 and Cellular networks | This is the identifier target link network to which handover needs to be initiated. This value includes the target MAC PoA. |
| Target Link Actions | Link Action | Any valid value of Link Action | Specifies the suggested action on link during handover. Combination of the following choices are allowed. Bit #0: LINK_DISCONNECT Bit #1: LINK_LOW_POWER Bit #2: LINK_POWER_DOWN Bit #3: LINK_NO_ACTION Bit #4: LINK_RESOURCE_RETAIN Bit #5: DATA_FORWARDING_REQUEST Bit #6: BI_CASTING_REQUEST Bit #7: HANDOVER_CANCEL Bit #8: LINK_POWER_UP Bit #9-31: Reserved |
| Target PoA | MAC_ADDRESS | N/A | This is Target Point of Attachment List (AP/BS |
| Old Link Actions (n) | Link Action | Any valid value of Link Action | Specifies the suggested action on link during handover. Combination of the following choices are allowed. Bit #0: LINK_DISCONNECT Bit #1: LINK_LOW_POWER Bit #2: LINK_POWER_DOWN Bit #3: LINK_NO_ACTION Bit #4: LINK_RESOURCE_RETAIN Bit #5: DATA_FORWARDING_REQUEST Bit #6: BI_CASTING_REQUEST Bit #7: HANDOVER_CANCEL Bit #8: LINK_POWER_UP Bit #9-31: Reserved |

7.6.23.1.3 When generated

This primitive may be generated when a handover decision has been taken (e.g. after a successful MIH_MN_HO_Candidate_Query.response is received) to allow a mobile node to begin handover and the mobile node decides to perform actual handover based on suggested choices for candidate network and PoA.

7.6.23.1.4 Effect on receipt

The recipient may identify that the mobile node may have started handover to the selected target link network. A corresponding indication (MIH_Net_HO_Candidate_Commit.indication) may be sent triggered to send to all subscribed MIH user entities in the local stack. Similarly, an indication may be sent in the remote stack if a corresponding MIH message is forwarded to the peer MIHF. The associated parameters for the generated indication are same as those used in command request.

7.6.23.2 MIH_MN_HO_Candidate_Commit.indication

7.6.23.2.1 Function

This primitive is generated by the MIH Function to indicate that an MIH MN HO Commit request message has been received. This primitive may be used by MIHF to inform MIH users locally that MIH_MN_HO_Candidate_Commit request is received.

7.6.23.2.2 Semantics of service primitive

The primitive is described as follows:

MIH_MN_HO_Candidate_Commit.indication (

SourceIdentifier

DestinationIdentifier

CurrentLinkIdentifier

CurrentLinkAction

TargetLinkIdentifier

TargetLinkAction

TargetPoA

OldLinkActions

)

Local or Remote: Local

MIHF (Network) ->MIH Users

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Source Identifier | Identifier | Valid MIHF identi fier | Source of this indication. |
| Destination Identifier | Identifier | Valid MIHF identi fier | The destination identifier of this indication. |
| Current Link Identifier | Network Identifier.. | N/A May be one of different 802 or cellular networks | Identifier of currently active link This identifies the current access network over which the command needs to be sent. This is valid only for remote commands which need to be sent to remote MIHF. The command is then sent either at L2 or at L3. |
| Current Link Actions | Link Action | Any valid value of Link Action | Specifies the suggested action on link during handover. Combination of the following choices are allowed. Bit #0: LINK_DISCONNECT Bit #1: LINK_LOW_POWER Bit #2: LINK_POWER_DOWN Bit #3: LINK_NO_ACTION Bit #4: LINK_RESOURCE_RETAIN Bit #5: DATA_FORWARDING_REQUEST Bit #6: BI_CASTING_REQUEST Bit #7: HANDOVER_CANCEL Bit #8: LINK_POWER_UP Bit #9-31: Reserved |
| Target Link Identifier | Network Identifier | N/A Can be different 802 and Cellular networks | This is the identifier target link network to which handover needs to be initiated. This value includes the target MAC PoA. |
| Target Link Actions | Link Action | Any valid value of Link Action | Specifies the suggested action on link during handover. Combination of the following choices are allowed. Bit #0: LINK_DISCONNECT Bit #1: LINK_LOW_POWER Bit #2: LINK_POWER_DOWN Bit #3: LINK_NO_ACTION Bit #4: LINK_RESOURCE_RETAIN Bit #5: DATA_FORWARDING_REQUEST Bit #6: BI_CASTING_REQUEST Bit #7: HANDOVER_CANCEL Bit #8: LINK_POWER_UP Bit #9-31: Reserved |
| Target PoA | MAC_address | N/A | This is Target Point of Attachment List (AP/BS |
| Old Link Actions | Link Action | Any valid value of Link Action | Specifies the suggested action on link during handover. Combination of the following choices are allowed. Bit #0: LINK_DISCONNECT Bit #1: LINK_LOW_POWER Bit #2: LINK_POWER_DOWN Bit #3: LINK_NO_ACTION Bit #4: LINK_RESOURCE_RETAIN Bit #5: DATA_FORWARDING_REQUEST Bit #6: BI_CASTING_REQUEST Bit #7: HANDOVER_CANCEL Bit #8: LINK_POWER_UP Bit #9-31: Reserved |

7.6.23.2.3 When generated

This primitive may be generated when MIH_MN_HO_Candidate_Commit request is received.

7.6.23.2.4 Effect on receipt

The recipient may identify identifies that the mobile node may have has started handover to the selected target network link. A corresponding response (MIH_MN_HO_Candidate_Commit.response) may be triggered to acknowledge the receipt of the message.

7.6.23.3 MIH_MN_HO_Candidate_Commit.response

7.6.23.3.1 Function

This primitive may be used by MIH users to send a in response of a to MIH_MN_HO_Candidate_Commit indication request.

7.6.23.3.2 Semantics of service primitive

The primitive is described as follows:

MIH_MN_HO_Candidate_Commit.response (

SourceIdentifier

DestinationIdentifier

CurrentLinkIdentifier

TargetLinkIdentifier

)

Local or Remote: Both

MIHF (Network) ->MIHF (MN)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Source Identifier | Identifier | Valid MIHF identi fier | The source where the indication is initiated. |
| Destination Identifier | Identifier | Valid MIHF identi fier | The destination identifier of this indication. Destination of this response. |
| Current Link Identifier | Network Identifier | N/A May be one of different 802 or cellular networks | Identifier of present link This identifies the current access network over which the command needs to be sent. This is valid only for remote commands which need to be sent to remote MIHF. The command is then sent either at L2 or at L3.. |
| Target Link Identifier | Identifier | N/A | This is the identifier target link to which handover needs to be initiated. This value includes the target MAC PoA. |

7.6.23.1.3 When generated

This primitive may be generated when MIH_MN_HO_Candidate_Commit indication request is received.

7.6.23.1.4 Effect on receipt

MN may now proceed with the next step in handover after taking any local link actions. A corresponding confirm is generated to the MIH users.

7.6.23.4 MIH_MN_HO_Candidate_Commit.confirm

7.6.23.4.1 Function

This primitive is may be used by the MIHF to inform MIH users locally the result of the that MIH MN HO Candidate_Commit request response is received.

7.6.23.4.2 Semantics of service primitive

The primitive is described as follows:

MIH_Net_HO_Candidate_Commit.confirm (

SourceIdentifier,

DestinationIdentifier,

CurrentLinkIdentifier

CurrentLinkResultCode

TargetLinkIdentifier,

TagetLinkResultCode

)

Local or Remote: Local

MIHF (Mobile Node) ->MIH Users

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Source Identifier | Identifier | Valid MIHF identi fier | The source of entity where the request is initiated. |
| Destination Identifier | Identifier | Valid MIHF identi fier | The destination identifier of request or response. This is the identifier of local or peer MIHF. |
| Current Link Identifier | Network Identifier. | N/A May be one of different 802 or cellular networks | Identifier of current link This identifies the current access network over which the command needs to be sent. This is valid only for remote commands which need to be sent to remote MIHF. The command is then sent either at L2 or at L3. |
| CurrentLinkResultCode | Enumarate | Success Failure Rejection | Result of the actions specified in the Link_Action.request primitive Result Codes are given as follows: 0= ResourceUnavailable 1= Resouce Busy 2 = Resource Available 3 = Abort Handover 4 = Perform Handover 5 = Success 6 = Failure 8 = Rejection 7-31 Reserved |
| Target Link Identifier | Identifier | N/A | This is the identifier target link to which handover needs has been initiated. This value includes the target MAC PoA. |
| TargetLinkResultCode | Enumarate | Success Failure Rejection | Result of the actions specified in the Link_Action.request primitive Result Codes are given as follows: 0= ResourceUnavailable 1= Resouce Busy 2 = Resource Available 3 = Abort Handover 4 = Perform Handover 5 = Success 6 = Failure 8 = Rejection 7-31 Reserved |

7.6.23.4.3 When generated

This primitive is may be generated after performing the HO actions received in the when MIH_MN_HO_Candidate_Commit request response is received.

7.6.23.4.4 Effect on receipt

MN may now proceed with the next step in handover.

### Modification to existing messages

Modify section 8.5.3.17 and 18 as follows:

8.5.3.17 MIH_MN_HO_Candidate_Commit request

MIH Header Fixed Fields (SID=3, Opcode=1, AID=22)

Source Identifier = sending MIHF ID

Destination Identifier = receiving MIHF ID

CurrentLinkIdentifier TLV (14)

CurrentLinkAction TLV (28)

TargetLinkIdentifier TLV (14)

TargetLinkAction TLV (28)

TargetPoA TLV (??)

OldLinkActions TLV (28)

8.5.3.18 MIH_MN_HO_Candidate_Commit response

MIH Header Fixed Fields (SID=3, Opcode=2, AID=22)

Source Identifier = sending MIHF ID

Destination Identifier = receiving MIHF ID

CurrentLinkIdentifier TLV (14)

CurrentLinkResultCode TLV (47)

TargetLinkIdentifier TLV (14)

TargetLinkResultCode TLV (47)