IEEE P802.21/D04.00
Draft IEEE Standard for Local and Metropolitan Area Networks:
Media Independent Handover Services

| Extension to Normative Text Proposal handling of Link-Action command towards Link Layers |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-March 30 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Juan Carlos Zuniga | InterDigital Communications | 1000, rue Sherebrook, Ouest, 10e étage, Montreal, QC, H3A 3G4 | +1514-904-6251 | JuanCarlos.Zuniga@InterDigital.com |
| Ulises Olvera | InterDigital Communications | 1000, rue Sherebrook, Ouest, 10e étage, Montreal, QC, H3A 3G4 | +1514-904-6258 | Ulises.Olvera-Hernandez@InterDigital.com |

- Abstract

This document contains a normative text proposal in support of link layer action commands that can be mapped to existing link layer technologies’ operations.

Notice: This document has been prepared to assist IEEE 802.21. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.21.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

# Background

The current 802.21 specification provides a uniform, well defined set of primitives that can be used by an MIH User to avail of Command, Event and Information services. The MIHF receives MIH primitives from the user and determines whether or not one or more link layer primitives should be triggered in order to satisfy the request from the user.

Likewise the MIHF may generate MIH primitives toward the user upon receipt of one or more link layer primitives coming from link layers.

Furthermore, 802.21 link layer primitives may be mapped directly or indirectly into existing primitives supported by other link layer technologies.

In this regard the link layer primitives described by 802.21 serves as a “blueprint” of what is expected from the link layer technology and therefore a mapping can be established between these “blue print” primitives and one or more existing primitives within a relevant link layer technology such as 802.11, 802.16 or even 3GPP/3GPP2.

Several examples exist depicting such behavior. Thus, MIH_Link_Detected is characterized as a Link Detected, MIH_Ling_Going_Down is characterized as a Link Going Down an so on.

Problem Description

Document 21-07/124r0 has been accepted by the group and it incorporates the required changes to support the link layer actions on the Net_HO_Commit messages and primitives. A similar functionality should be added on the MN_HO_Commit messages and primitives for consistency.

Proposed Solution

This document presents the required changes for the D4 draft to support the link layer actions on the MN_HO_Commit messages and primitives.

Modify section 7.6.23 as follows:

7.6.23 MIH_MN_HO_Candidate_Commit

7.6.23.1 MIH_MN_HO_Candidate_Commit.request

7.6.23.1.1 Function

This primitive may be used by MIH users on a mobile node to inform MIHF to commit the handover to a specific target network. MIHF may forward the message remotely to the peer MIHF in the network in order to inform about the handover decision. The MN may, instead, continue the handover procedure utilizing IP mobility protocols after activating target L2 links natively.

7.6.23.1.2 Semantics of service primitive

The primitive is described as follows:

MIH_MN_HO_Candidate_Commit.request (

SourceIdentifier

DestinationIdentifier

CurrentOldLinkIdentifier

CurrentOldLinkAction

TargetNewLinkIdentifier

TargetNewLinkAction

TargetPoA

OldLinkActions

)

Local or Remote: Both

MIHF (Network) <>MIHF (Mobile Node)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Source Identifier | Identifier | Valid MIHF identi fier | The source of entity where the request is initiated. |
| Destination Identifier | Identifier | Valid MIHF identi fier | The destination identifier of request. |
| CurrentOld Link Identifier | Network Identifier.. | N/A May be one of different 802 or cellular networks | Identifier of currently active link This identifies the current access network over which the command needs to be sent. This is valid only for remote commands which need to be sent to remote MIHF. The command is then sent either at L2 or at L3. |
| CurrentOld Link Actions | Link Action | Any valid value of Link Action | Specifies the suggested action on link during handover. Combination of the following choices are allowed. Bit #0: LINK_DISCONNECT Bit #1: LINK_LOW_POWER Bit #2: LINK_POWER_DOWN Bit #3: LINK_NO_ACTION Bit #4: LINK_RESOURCE_RETAIN Bit #5: DATA_FORWARDING_REQUEST Bit #6: BI_CASTING_REQUEST Bit #7: HANDOVER_CANCEL Bit #8: LINK_POWER_UP Bit #9-31: Reserved |
| TargetNew Link Identifier | Network Identifier | N/A Can be different 802 and Cellular networks | This is the identifier new link network to which handover needs to be initiated. This value includes the new MAC PoA. |
| TargetNew Link Actions | Link Action | Any valid value of Link Action | Specifies the suggested action on link during handover. Combination of the following choices are allowed. Bit #0: LINK_DISCONNECT Bit #1: LINK_LOW_POWER Bit #2: LINK_POWER_DOWN Bit #3: LINK_NO_ACTION Bit #4: LINK_RESOURCE_RETAIN Bit #5: DATA_FORWARDING_REQUEST Bit #6: BI_CASTING_REQUEST Bit #7: HANDOVER_CANCEL Bit #8: LINK_POWER_UP Bit #9-31: Reserved |
| Target PoA | MAC_ADDRESS | N/A | This is Target Point of Attachment List (AP/BS |
| Old Link Actions (n) | Link Action | Any valid value of Link Action | Specifies the suggested action on link during handover. Combination of the following choices are allowed. Bit #0: LINK_DISCONNECT Bit #1: LINK_LOW_POWER Bit #2: LINK_POWER_DOWN Bit #3: LINK_NO_ACTION Bit #4: LINK_RESOURCE_RETAIN Bit #5: DATA_FORWARDING_REQUEST Bit #6: BI_CASTING_REQUEST Bit #7: HANDOVER_CANCEL Bit #8: LINK_POWER_UP Bit #9-31: Reserved |

7.6.23.1.3 When generated

This primitive may be generated when a handover decision has been taken (e.g. after a successful MIH_MN_HO_Candidate_Query.response is received) to allow a mobile node to begin handover and the mobile node decides to perform actual handover based on suggested choices for candidate network and PoA.

7.6.23.1.4 Effect on receipt

The recipient may identify that the mobile node may have started handover to the selected target link network. A corresponding indication (MIH_Net_HO_Candidate_Commit.indication) may be sent triggered to send to all subscribed MIH user entities in the local stack. Similarly, an indication may be sent in the remote stack if a corresponding MIH message is forwarded to the peer MIHF. The associated parameters for the generated indication are same as those used in command request.

7.6.23.2 MIH_MN_HO_Candidate_Commit.indication

7.6.23.2.1 Function

This primitive is generated by the MIH Function to indicate that an MIH MN HO Commit request message has been received. This primitive may be used by MIHF to inform MIH users locally that MIH_MN_HO_Candidate_Commit request is received.

7.6.23.2.2 Semantics of service primitive

The primitive is described as follows:

MIH_MN_HO_Candidate_Commit.indication (

SourceIdentifier

DestinationIdentifier

CurrentOldLinkIdentifier

CurrentOldLinkAction

TargetNewLinkIdentifier

TargetNewLinkAction

TargetPoA

OldLinkActions

)

Local or Remote: Local

MIHF (Network) ->MIH Users

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Source Identifier | Identifier | Valid MIHF identi fier | Source of this indication. |
| Destination Identifier | Identifier | Valid MIHF identi fier | The destination identifier of this indication. |
| CurrentOld Link Identifier | Network Identifier.. | N/A May be one of different 802 or cellular networks | Identifier of currently active link This identifies the current access network over which the command needs to be sent. This is valid only for remote commands which need to be sent to remote MIHF. The command is then sent either at L2 or at L3. |
| CurrentOld Link Actions | Link Action | Any valid value of Link Action | Specifies the suggested action on link during handover. Combination of the following choices are allowed. Bit #0: LINK_DISCONNECT Bit #1: LINK_LOW_POWER Bit #2: LINK_POWER_DOWN Bit #3: LINK_NO_ACTION Bit #4: LINK_RESOURCE_RETAIN Bit #5: DATA_FORWARDING_REQUEST Bit #6: BI_CASTING_REQUEST Bit #7: HANDOVER_CANCEL Bit #8: LINK_POWER_UP Bit #9-31: Reserved |
| TargetNew Link Identifier | Network Identifier | N/A Can be different 802 and Cellular networks | This is the identifier new link network to which handover needs to be initiated. This value includes the new MAC PoA. |
| TargetNew Link Actions | Link Action | Any valid value of Link Action | Specifies the suggested action on link during handover. Combination of the following choices are allowed. Bit #0: LINK_DISCONNECT Bit #1: LINK_LOW_POWER Bit #2: LINK_POWER_DOWN Bit #3: LINK_NO_ACTION Bit #4: LINK_RESOURCE_RETAIN Bit #5: DATA_FORWARDING_REQUEST Bit #6: BI_CASTING_REQUEST Bit #7: HANDOVER_CANCEL Bit #8: LINK_POWER_UP Bit #9-31: Reserved |
| Target PoA | MAC_address | N/A | This is Target Point of Attachment List (AP/BS |
| Old Link Actions | Link Action | Any valid value of Link Action | Specifies the suggested action on link during handover. Combination of the following choices are allowed. Bit #0: LINK_DISCONNECT Bit #1: LINK_LOW_POWER Bit #2: LINK_POWER_DOWN Bit #3: LINK_NO_ACTION Bit #4: LINK_RESOURCE_RETAIN Bit #5: DATA_FORWARDING_REQUEST Bit #6: BI_CASTING_REQUEST Bit #7: HANDOVER_CANCEL Bit #8: LINK_POWER_UP Bit #9-31: Reserved |

7.6.23.2.3 When generated

This primitive may be generated when MIH_MN_HO_Candidate_Commit request is received.

7.6.23.2.4 Effect on receipt

The recipient may identify identifies that the mobile node may have has started handover to the selected target network link. A corresponding response (MIH_MN_HO_Candidate_Commit.response) may be triggered to acknowledge the receipt of the message.

7.6.23.3 MIH_MN_HO_Candidate_Commit.response

7.6.23.3.1 Function

This primitive may be used by MIH users to send a in response of a to MIH_MN_HO_Candidate_Commit indication request.

7.6.23.3.2 Semantics of service primitive

The primitive is described as follows:

MIH_MN_HO_Candidate_Commit.response (

SourceIdentifier

DestinationIdentifier

OldLinkIdentifier

NewLinkIdentifier

)

Local or Remote: Both

MIHF (Network) ->MIHF (MN)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Source Identifier | Identifier | Valid MIHF identi fier | The source where the indication is initiated. |
| Destination Identifier | Identifier | Valid MIHF identi fier | The destination identifier of this indication. Destination of this response. |
| CurrentOld Link Identifier | Network Identifier | N/A May be one of different 802 or cellular networks | Identifier of present link This identifies the current access network over which the command needs to be sent. This is valid only for remote commands which need to be sent to remote MIHF. The command is then sent either at L2 or at L3.. |
| TargetNew Link Identifier | Identifier | N/A | This is the identifier new link to which handover needs to be initiated. This value includes the new MAC PoA. |

7.6.23.1.3 When generated

This primitive may be generated when MIH_MN_HO_Candidate_Commit indication request is received.

7.6.23.1.4 Effect on receipt

MN may now proceed with the next step in handover after taking any local link actions. A corresponding confirm is generated to the MIH users.

7.6.23.4 MIH_MN_HO_Candidate_Commit.confirm

7.6.23.4.1 Function

This primitive is may be used by the MIHF to inform MIH users locally the result of the that MIH MN HO Candidate_Commit request response is received.

7.6.23.4.2 Semantics of service primitive

The primitive is described as follows:

MIH_Net_HO_Candidate_Commit.confirm (

SourceIdentifier,

DestinationIdentifier,

CurrentOldLinkIdentifier

CurrentOldLinkResultCode

TargetNewLinkIdentifier,

TargetNewLinkResultCode

)

Local or Remote: Local

MIHF (Mobile Node) ->MIH Users

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Source Identifier | Identifier | Valid MIHF identi fier | The source of entity where the request is initiated. |
| Destination Identifier | Identifier | Valid MIHF identi fier | The destination identifier of request or response. This is the identifier of local or peer MIHF. |
| CurrentOld Link Identifier | Network Identifier. | N/A May be one of different 802 or cellular networks | Identifier of current link This identifies the current access network over which the command needs to be sent. This is valid only for remote commands which need to be sent to remote MIHF. The command is then sent either at L2 or at L3. |
| CurrentOldLinkResultCode | Enumarate | Success Failure Rejection | Result of the actions specified in the Link_Action.request primitive Result Codes are given as follows: 0= ResourceUnavailable 1= Resouce Busy 2 = Resource Available 3 = Abort Handover 4 = Perform Handover 5 = Success 6 = Failure 8 = Rejection 7-31 Reserved |
| TargetNew Link Identifier | Identifier | N/A | This is the identifier new link to which handover needs has been initiated. This value includes the new MAC PoA. |
| TargetNewLinkResultCode | Enumarate | Success Failure Rejection | Result of the actions specified in the Link_Action.request primitive Result Codes are given as follows: 0= ResourceUnavailable 1= Resouce Busy 2 = Resource Available 3 = Abort Handover 4 = Perform Handover 5 = Success 6 = Failure 8 = Rejection 7-31 Reserved |

7.6.23.4.3 When generated

This primitive is may be generated after performing the HO actions received in the when MIH_MN_HO_Candidate_Commit request response is received.

7.6.23.4.4 Effect on receipt

MN may now proceed with the next step in handover.

### Modification to existing messages

Modify section 8.5.3.17 and 18 as follows:

8.5.3.17 MIH_MN_HO_Candidate_Commit request

MIH Header Fixed Fields (SID=3, Opcode=1, AID=22)

Source Identifier = sending MIHF ID

Destination Identifier = receiving MIHF ID

CurrentOldLinkIdentifier TLV (14)

CurrentOldLinkAction TLV (28)

TargetNewLinkIdentifier TLV (14)

TargetNewLinkAction TLV (28)

TargetPoA TLV (??)

OldLinkActions TLV (28)

8.5.3.18 MIH_MN_HO_Candidate_Commit response

MIH Header Fixed Fields (SID=3, Opcode=2, AID=22)

Source Identifier = sending MIHF ID

Destination Identifier = receiving MIHF ID

CurrentOldLinkIdentifier TLV (14)

CurrentOldLinkResultCode TLV (47)

TargetNewLinkIdentifier TLV (14)

TargetNewLinkResultCode TLV (47)