| Project | IEEE 802.21 MIHO <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | Data Types |  |
| Date Submitted | May 12, 2007 |  |
| Source(s) | Yoshihiro Ohba, Subir Das, Yuu-Heng Alice Cheng, Miriam Tauil, Kenichi Taniuchi |  |
| Re: | IEEE 802.21 Session #20 in May 2007 |  |
| Abstract | This document is trying to propose a complete set of data types. |  |
| Purpose | LB#1d comment resolution |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.21. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

# Introduction

This document proposes line-by-line remedy on LB comments 5125, 5247 and 5490. The proposed resolution is outlined in 21-07-0141-01-0000-Data_Type_Encoding.ppt.

The document also addresses the issue on LINK ID/CONNECTION ID(LINK TUPLE ID) issue that are related to LB comments 5158, 5159, 5181, 5193, 5209, 5252, 5253, 5263,

## Motivation

- Avoid duplicate data type definition for IEs, primitives and TLVs. This will eliminate inconsistency in the specification.

- Clearly separate abstract data type and encoding rule. By using  TLV-in-TLV, it is difficult to achieve the clear separation for primitive parameters)

- Solve ordering issues on repetitive TLVs in messages (e.g., PreferredCandidateLink 1..k )

# Proposed Remedy

## Change on Section 6.4.3

[1] Change the paragraph starting from Page 49, line 60 as follows:

“

Table 7 represents the list of Information Elements supported by the IEEE 802.21 standard. The value of each Information Element has an abstract data type for which semantics and binary encoding are defined in Annex M. The TLV representation of these Information Elements is described in 6.4.6. Another way of representing these IEs using RDF is described in 6.4.7.1. The IEs may be retrieved using TLV or resource description framework (RDF) based query mechanism. The standard does not recommend or mandate the choice of a particular mechanism. Vendors may need to specify additional IEs beyond the IEs specified in Table 7. Vendors can choose to do that in the Vendor Specific IE format. These IEs will then be available only in vendor specific deployments.

“

[2] Change Table 7 as follows.

| No | Name of Information Element | Description | Data Type Name |
| --- | --- | --- | --- |
| General Information Elements |  |  |  |
| 1.1 | TYPE_IE_NETWORK_TYPE | [Same as D05] | NETWORK_TYPE |
| 1.2 | TYPE_IE_OPERATOR_IDENTIFIER | [Same as D05] | OPERATOR_ID |
| 1.3 | TYPE_IE_SERVICE_PROVIDER_IDENTIFIER | [Same as D05] | SERVICE_PROVIDER_ID |
| Access Network Specific Information Elements |  |  |  |
| 2.1 | TYPE_IE_ACCESS_NETWORK_I DENTIFIER | [Same as D05] | ACCESS_NETWORK_ID |
| 2.2 | TYPE_IE_NETWORK_SYSTEM_ID | [Same as D05] | NETWORK_SYSTEM_ID |
| 2.3 | TYPE_IE_ROAMING_PARTNERS | [Same as D05] | ROAMING_PARTNERS |
| 2.4 | TYPE_IE_COST | [Same as D05] | COST |
| 2.5 | TYPE_IE_NETWORK_SECURITY | [Same as D05] | NETWORK_SECURITY |
| 2.6 | TYPE_IE_NETWORK_QOS | [Same as D05] | NETWORK_QOS |
| 2.7 | TYPE_IE_NETWORK_DATA_RATE | [Same as D05] | DATA_RATE |
| 2.8 | TYPE_IE_NETWORK_IP_CONFIG_METHODS | [Same as D05] | IP_CONFIG_METHODS |
| 2.9 | TYPE_IE_NETWORK_CAPABILI TIES | [Same as D05] | NETWORK_CAPABILITIES |
| PoA Specific Information Elements |  |  |  |
| 3.1 | TYPE_IE_POA_MAC_ADDRESS | [Same as D05] | MAC_ ADDRESS |
| 3.2 | TYPE_IE_POA_LOCATION | [Same as D05] | LOCATION |
| 3.3 | TYPE_IE_POA_DATA_RATE | [Same as D05] | DATA_RATE |
| 3.4 | TYPE_IE_POA_CHANNEL_RANGE | [Same as D05] | CHANNEL_RANGE |
| PoA Specific Higher Layer Service Information Elements |  |  |  |
| 4.1 | TYPE_IE_POA_SUBNET_INFORMATION | [Same as D05] | IP_SUBNET_INFO |
| 4.2 | TYPE_IE_POA_IP_CONFIG_MET HODS | [Same as D05] | IP_CONFIG_METHODS |
| 4.3 | TYPE_IE_POA_ CAPABILITIES | [Same as D05] | NETWORK_CAPABILITIES |
| 4.4 | TYPE_IE_POA_IP_ADDRESS | [Same as D05] | IP_ADDRESS |
| Other Information Elements |  |  |  |
| 5.1 | Vendor Specific IEs | [Same as D05] | [TBD] |

## Change on Section 6.4.6

[1] Remove Section 6.4.6.1 and its subsections since this subsection is covered in Annex M.

[2] Remove Section 6.4.6.2 and its subsections since this subsection is covered in Annex M.

[3] Remove Section 6.4.6.3 and its subsections since this subsection is covered in Annex M.

## Change on Section 7

[1] Change Section 7.2.1 as follows:

“

7.2.1 SAP primitives and parameter types

The SAPs are defined as a set of primitives. Taken together, the primitives define the services. Within the definition of each primitive there is a table of allowable parameters. Each parameter is defined using abstract data types. These types indicate the semantic value of that parameter. The parameters defined within the clause for a particular primitive are produced or consumed by that primitive. Several of the abstract data

types are used in multiple primitive definitions. In each abstract data type definition, the various names applied to this type are listed.

Most of the primitives have corresponding MIH protocol messages. In those messages, there are TLV encoded parameters that implement the primitive parameter abstract data types within the protocol. The definition of the full binary encoding for each of these instantiations is in Clause 8.5 Annex M.

“

[2] Remove Table 22.

[3] Change “Semantics of service primitive” section of each primitive definition sections as follows:

7.3.1.1.2 Semantics of service primitive

The primitive parameters are as follows:

Link_Event_Subscribe.request (

RequestedLinkEventList

)

Parameters:

| Name | Type | Description |
| --- | --- | --- |
| Requested Link Event List | LINK_EVENT_LIST | List of link layer events that the endpoint would like to receive indications for, from the Event Source. |

[Delete Table 31.]

7.3.1.2.2 Semantics of service primitive

The primitive parameters are as follows:

Link_Event_Subscribe.confirm (

ResponseLinkEventList,

Status

)

Parameters:

| Name | Type | Description |
| --- | --- | --- |
| Response Link Event List | LINK_EVENT_LIST | List of link layer events along with their subscription status. |
| Status | STATUS | Status of operation. |

7.3.2.1.2 Semantics of service primitive

The primitive parameters are as follows:

Link_Event_Unsubscribe.request (

RequestedLinkEventList

)

Parameters:

| Name | Type | Description |
| --- | --- | --- |
| Requested Link Event List | LINK_EVENT_LIST | List of link layer events for which indications need to be unsubscribed from the Event Source. |

7.3.1.2.2 Semantics of service primitive

The primitive parameters are as follows:

Link_Event_Unsubscribe.confirm (

ResponseLinkEventList,

Status

)

Parameters:

| Name | Type | Description |
| --- | --- | --- |
| Response Link Event List | LINK_EVENT_LIST | List of link layer events along with their subscription status. |
| Status | STATUS | Status of operation. |

7.3.3.1.2 Semantics of service primitive

The primitive parameter is as follows:

Link_Configure_Thresholds.request (

LinkConfigureParameterList

)

Parameters:

| Name | Type | Description |
| --- | --- | --- |
| Link Configure Parameter List | LIST(LINK_PARAM) | A list of Link Configure Parameter. |
| Status | STATUS | Status of operation. |

7.3.3.2.2 Semantics of service primitive

The primitive parameters are as follows:

Link_Configure_Thresholds.confirm (

LinkConfigureStatusList,

Status

)

Parameters:

| Name | Type | Description |
| --- | --- | --- |
| Link Configure Status List | LIST(LINK_CONFIG_STATUS) | A list of Link Configure Status. |
| Status | STATUS | Status of operation. |

7.3.4.2 Semantics of service primitive

The primitive parameters are as follows:

Link_Up.indication (

LinkIdentifier,

MacOldAccessRouter,

MacNewAccessRouter,

IPRenewalFlag,

Mobility Management Support

)

Parameters:

| Name | Type | Description |
| --- | --- | --- |
| Link Identifier | LINK_TUPLE_ID | Identifier of the link associated with the event. |
| MAC Old Access Router | MAC_ADDRESS | (Optional) MAC Address of old Access Router. |
| MAC New Access Router | MAC_ADDRESS | (Optional) MAC Address of new Access Router. |
| IP Renewal Flag | IP_RENEWAL_FLAG | (Optional) Indicates whether the MN shall change IP Address in the new PoA. |
| Mobility Management Support | MOBILITY_MGMT | (Optional) Indicates the type of Mobility Management Protocol supported by the new PoA. |

7.3.5.2 Semantics of service primitive

Link_Down.indication (

LinkIdentifier,

MacOldAccessRouter,

ReasonCode

)

Parameters:

| Name | Type | Description |
| --- | --- | --- |
| Link Identifier | LINK_TUPLE_ID | Identifier of the link associated with the event. |
| MAC Old Access Router | MAC_ADDRESS | (Optional) MAC Address of old Access Router. |
| Reason Code | LINK_DOWN_REASON_CODE | Reason for why the link went down. |

7.3.6.2 Semantics of service primitive

Link_Going_Down.indication (

LinkIdentifier,

TimeInterval,

ConfidenceLevel,

LinkGoingDownReason,

UniqueEventIdentifier

)

Parameters:

| Name | Type | Description |
| --- | --- | --- |
| Link Identifier | LINK_TUPLE_ID | Identifier of the link associated with the event. |
| Time Interval | INTEGER(2) | Time Interval (in milliseconds) in which the link is expected to go down. The link connectivity is expected to be available at least for time specified by TimeInterval. |
| Confidence Level | PERCENTAGE | The confidence level (%) for link to go down within the specified time interval. |
| Link Going Down Reason | LINK_DOWN_REASON | The reason for why the link is going to be down. |
| Reason Code | LINK_GOING_DOWN_REASON | Reason for why the link went down. |
| Unique Event Identifier | INTEGER(2) | Used to uniquely identify the event. To be used in case of event rollback. |

7.3.7.2 Semantics of service primitive

The primitive parameters are as follows:

Link_Event_Rollback.indication (

LinkIdentifier,

UniqueEventIdentifier

)

Parameters:

| Name | Type | Description |
| --- | --- | --- |
| Link Identifier | LINK_TUPLE_ID | Identifier of the link associated with the event. |
| Unique Event Identifier | INTEGER(2) | Used to identify the event which needs to be rolled back. |

7.3.8.2 Semantics of service primitive

The primitive parameters are as follows:

Link_Detected.indication (

LinkIdentifier,

MIHCapabilityFlag

)

Parameters:

| Name | Type | Description |
| --- | --- | --- |
| Link Identifier | LINK_TUPLE_ID | Identifier of the link associated with the event. |
| MIH Capability Flag | LINK_MIH_CAPABILITY_FLAG | Used to indicate whether the link supports MIH capability. |

7.3.9.2 Semantics of service primitive

Link_Parameters_Report.indication(

LinkIdentifier,

LinkParametersReportList

)

Parameters:

| Name | Type | Description |
| --- | --- | --- |
| Link Identifier | LINK_TUPLE_ID | Identifier of the link associated with the event. |
| Link Parameters Report List | LIST(LINK_PARAM) | A list of Link Parameter Report. |

7.3.10.2 Semantics of service primitive

The primitive parameters are as follows:

Link_PDU_Transmit_Status.indication (

LinkIdentifier,

PacketIdentifier,

TransmissionStatus

)

Parameters:

| Name | Type | Description |
| --- | --- | --- |
| Link Identifier | LINK_TUPLE_ID | Identifier of the link associated with the event. |
| Packet Identifier | INTEGER(2) | Identifier for higher layer data packet that was delivered successfully. |
| Transmission Status | BOOLEAN | Status of transmission of the packet. TRUE: Success FALSE: Failure |

7.3.11.2 Semantics of service primitive

The primitive parameters are as follows:

Link_Handover_Imminent.indication (

LinkIdentifier,

MacOldAccessRouter,

MacNewAccessRouter

)

Parameters:

| Name | Type | Description |
| --- | --- | --- |
| Link Identifier | LINK_TUPLE_ID | Identifier of the link associated with the event. |
| Mac Old Access Router | MAC_ADDRESS | (Optional) MAC Address of old and Access Router. |
| Mac New Access Router | MAC_ADDRESS | (Optional) MAC Address of new Access Routers. |

7.3.13.2.2 Semantics of service primitive

The primitive parameters are as follows:

Link_Capability_Discover.confirm (

SupportedLinkEventList,

SupportedLinkCommandList,

Status

)

Parameters:

| Name | Type | Description |
| --- | --- | --- |
| Supported Link Event List | LINK_EVENT_LIST | List of link layer events supported by the link layer. |
| Supported Link Command List | LINK_COMMAND_LIST | List of link layer commands supported by the link layer. |
| Status | STATUS | Status of operation. |

7.3.14.1.2 Semantics of service primitive

The primitive parameter is as follows:

Link_Get_Parameters.request (

LinkParametersRequest

)

Parameters:

| Name | Type | Description |
| --- | --- | --- |
| Link Parameters Request | LINK_PARAM_TYPE | A bitmap representing the list of link parameters for which status is requested. |

7.3.14.2.2 Semantics of service primitive

The primitive parameters are as follows:

Link_Get_Parameters.confirm (

LinkParametersStatusList,

Status

)

Parameters:

| Name | Type | Description |
| --- | --- | --- |
| Link Parameters Status List | LIST(LINK_PARAM) | A list of QoS Parameters. |
| Status | STATUS | Status of operation. |

7.3.15.1.2 Semantics of Service Primitive

The parameters of the service primitive are as follows:

Link_Action.request (

LinkIdentifier

LinkAction

ExecutionTime

)

| Name | Type | Description |
| --- | --- | --- |
| Link Identifier | LINK_TUPLE_ID | Identifier of present link |
| Link Actions | LINK_ACTIONS | Specifies the suggested action on link during handover. |
| Execution Time | INTEGER(2) | Time (in ms) elapsed before action needs to be taken, if Execution Time = 0 then the action is expected to be taken on receipt of this command. |

7.3.15.2.2 Semantics of Service Primitive

The parameters of the service primitive are as follows:

Link_Action.confirm (

ScanResponseSet,

Status

)

| Name | Type | Description |
| --- | --- | --- |
| Scan Response Sets | LINK_SCAN_RESP | (Optional) A list of discovered links and related information. |
| Status | STATUS | Status of operation. |

7.4.1.1.2 Semantics of service primitive

MIH_Capability_Discover.request (

DestinationIdentifier,

SupportedMihEventList,

SupportedMihCommandList,

SupportedIsQueryTypeList,

SupportedTransportList

)

| Name | Type | Description |
| --- | --- | --- |
| Destination Identifier | MIHF_ID | [Same as D5] |
| Supported MIH Event List | MIH_EVENT_LIST | (Optional) List of supported events on MIHF. |
| Supported IS Query Type List | MIH_IS_QUERY_TYPE_LIST | (Optional) List of supported MIIS query types. |
| Supported TransportList | MIH_TRANSPORT_LIST | (Optional) List of supported transport types. |

7.4.1.2.2 Semantics of Service primitive

MIH_Capability_Discover.indication (

SourceIdentifier,

SupportedMihEventList,

SupportedMihCommandList,

SupportedIsQueryTypeList,

SupportedTransportList

)

| Name | Type | Description |
| --- | --- | --- |
| Source Identifier | MIHF_ID | [Same as D5] |
| Supported MIH Event List | MIH_EVENT_LIST | (Optional) List of supported events on MIHF. |
| Supported IS Query Type List | MIH_IS_QUERY_TYPE_LIST | (Optional) List of supported MIIS query types. |
| Supported TransportList | MIH_TRANSPORT_LIST | (Optional) List of supported transport types. |

7.4.1.3.1 Semantics of Service primitive

MIH_Capability_Discover.response(

DestinationIdentifier,

SupportedMihEventList,

SupportedMihCommandList,

SupportedIsQueryTypeList,

SupportedTransportList,

Status

)

| Name | Type | Description |
| --- | --- | --- |
| Destination Identifier | MIHF_ID | [Same as D5] |
| Supported MIH Event List | MIH_EVENT_LIST | (Optional) List of supported events on MIHF. |
| Supported IS Query Type List | MIH_IS_QUERY_TYPE_LIST | (Optional) List of supported MIIS query types. |
| Supported TransportList | MIH_TRANSPORT_LIST | (Optional) List of supported transport types. |
| Status | STATUS | Status of operation. |

7.4.1.4.2 Semantics of service primitive

MIH_Capability_Discover.confirm (

SourceIdentifier,

SupportedMihEventList,

SupportedMihCommandList,

SupportedIsQueryTypeList,

SupportedTransportList,

Status

)

| Name | Type | Description |
| --- | --- | --- |
| Source Identifier | MIHF_ID | [Same as D5] |
| Supported MIH Event List | MIH_EVENT_LIST | (Optional) List of supported events on MIHF. |
| Supported IS Query Type List | MIH_IS_QUERY_TYPE_LIST | (Optional) List of supported MIIS query types. |
| Supported TransportList | MIH_TRANSPORT_LIST | (Optional) List of supported transport types. |
| Status | STATUS | Status of operation. |

7.4.2.1.2 Semantics of service primitive

MIH_Register.request (

DestinationIdentifier,

RequestCode

)

| Name | Type | Description |
| --- | --- | --- |
| Destination Identifier | MIHF_ID | [Same as D5] |
| Request Code | REG_REQUEST_CODE | Registration request code. |

7.4.2.2.2 Semantics of service primitive

MIH_Register.indication (

SourceIdentifier,

RequestCode

)

| Name | Type | Description |
| --- | --- | --- |
| Source Identifier | MIHF_ID | [Same as D5] |
| Request Code | REG_REQUEST_CODE | Registration request code. |

7.4.2.3.2 Semantics of service primitive

MIH_Register.response (

Destination Identifier,

ValidityTime,

Status

)

| Name | Type | Description |
| --- | --- | --- |
| Destination Identifier | MIHF_ID | [Same as D5] |
| Validity Time | INTEGER(4) | [Same as D5] |
| Status | STATUS | Status of operation. |

7.4.2.4.2 Semantics of service primitive

MIH_Register.confirm (

SourceIdentifier,

ValidityTime,

Status

)

| Name | Type | Description |
| --- | --- | --- |
| Source Identifier | MIHF_ID | [Same as D5] |
| Validity Time | INTEGER(2) | [Same as D5] |
| Status | STATUS | Status of operation. |

7.4.3.2.2 Semantics of service primitive

MIH_DeRegister.indication(

SourceIdentifier

)

| Name | Type | Description |
| --- | --- | --- |
| Source Identifier | MIHF_ID | [Same as D5] |

MIH_DeRegister.response (

DestinationIdentifier,

Status

)

| Name | Type | Description |
| --- | --- | --- |
| Destination Identifier | MIHF_ID | [Same as D5] |
| Status | STATUS | Status of operation. |

7.4.3.4.2 Semantics of service primitive

MIH_DeRegister.confirm (

SourceIdentifier,

Status

)

| Name | Type | Description |
| --- | --- | --- |
| Source Identifier | MIHF_ID | [Same as D5] |
| Status | STATUS | Status of operation. |

7.4.4.1.2 Semantics of service primitive

The primitive parameters are as follows:

MIH_Event_Subscribe.request (

DestinationIdentifier,

LinkIdentifier,

RequestedMihEventList

)

| Name | Type | Description |
| --- | --- | --- |
| Destination Identifier | MIHF_ID | [Same as D5] |
| Link Identifier | LINK_TUPLE_ID | Identifier of the link for event subscription. |
| Requested MIH Event List | MIH_EVENT_LIST | List of MIH events that the endpoint would like to receive indications for, from the Event Source. |

7.4.4.2.2 Semantics of service primitive

The primitive parameters are as follows:

MIH_Event_Subscribe.confirm (

SourceIdentifier

LinkIdentifier,

ResponseMihEventList,

Status

)

| Name | Type | Description |
| --- | --- | --- |
| Source Identifier | MIHF_ID | [Same as D5] |
| Link Identifier | LINK_TUPLE_ID | [Same as D5] |
| Response MIH Event List | MIH_EVENT_LIST | List of MIH events along with their subscription status |
| Status | STATUS | Status of operation. |

7.4.5.1.2 Semantics of service primitive

The primitive parameters are as follows:

MIH_Event_Unsubscribe.request (

DestinationIdentifier,

LinkIdentifier,

RequestedMihEventList

)

| Name | Type | Description |
| --- | --- | --- |
| Destination Identifier | MIHF_ID | [Same as D5] |
| Link Identifier | LINK_TUPLE_ID | Identifier of the link for event unsubscription. |
| Requested MIH Event List | MIH_EVENT_LIST | List of MIH events for which indications need to be unsubscribed from the Event Source. |

7.4.5.2.2 Semantics of service primitive

The primitive parameters are as follows:

MIH_Event_Unsubscribe.confirm (

SourceIdentifier,

LinkIdentifier,

ResponseMihEventList,

Status

)

| Name | Type | Description |
| --- | --- | --- |
| Source Identifier | MIHF_ID | [Same as D5] |
| Link Identifier | LINK_TUPLE_ID | [Same as D5] |
| Response MIH Event List | MIH_EVENT_LIST | List of MIH events along with their unsubscription status. |
| Status | STATUS | Status of operation. |

7.4.10.1.2 Semantics of service primitive

The primitive parameters are as follows:

MIH_Link_Parameters_Report.indication (

SourceIdentifier,

LinkIdentifier,

LinkParametersStatusList

)

| Name | Type | Description |
| --- | --- | --- |
| Source Identifier | MIHF_ID | [Same as D5] |
| Link Identifier | LINK_TUPLE_ID | [Same as D5] |
| Link Parameters Status List | LIST(LINK_PARAM) | A list of Link Parameter Status. |

7.4.13.1.2 Semantics of service primitive

The primitive parameters are as follows:

MIH_Link_Handover_Imminent.indication (

SourceIdentifier,

Old Link Identifier,

New Link Identifier,

HO_Type,

MacOldAccessRouter,

MacNewAccessRouter

)

| Name | Type | Description |
| --- | --- | --- |
| Source Identifier | MIHF_ID | [Same as D5] |
| Old Link Identifier | LINK_TUPLE_ID | [Same as D5] |
| New Link Identifier | LINK_TUPLE_ID | [Same as D5] |
| HO_Type | HANDOVER_TYPE | Indicates whether intra-technology or inter-technology handover: |
| Mac Old Access Router | MAC_ADDRESS | [Same as D5] |
| Mac New Access Router | MAC_ADDRESS | [Same as D5] |

The primitive parameters are as follows:

MIH_Link_Handover_Complete.indication (

SourceIdentifier,

OldLinkIdentifier,

NewLinkIdentifier,

MacOldAccessRouter,

MacNewAccessRouter

)

| Name | Type | Description |
| --- | --- | --- |
| Source Identifier | MIHF_ID | [Same as D5] |
| Old Link Identifier | LINK_TUPLE_ID | [Same as D5] |
| New Link Identifier | LINK_TUPLE_ID | [Same as D5] |
| Mac Old Access Router | MAC_ADDRESS | [Same as D5] |
| Mac New Access Router | MAC_ADDRESS | [Same as D5] |

7.4.15.1.2 Semantics of the service primitive

The parameters of the primitive are as follows:

MIH_Get_Link_Parameters.request (

DestinationIdentifier,

LinkIdentifierList,

GetStatusRequestSet

)

| Name | Type | Description |
| --- | --- | --- |
| Destination Identifier | MIHF_ID | [Same as D5] |
| Link Identifier List | LIST(LINK_TUPLE_ID) | List of link identifiers for which status is requested. If the list is empty, return the status of all available links. |
| Get Status Request Set | LINK_STATUS_REQ | A type to represent a link status being requested. |

7.4.15.2.2 Semantics of the service primitive

The parameters of the primitive are as follows:

MIH_Get_Link_Parameters.confirm (

SourceIdentifier,

GetStatusResponseList,

Status

)

| Name | Type | Description |
| --- | --- | --- |
| Source Identifier | MIHF_ID | [Same as D5] |
| Get Status Response List | LIST(LINK_STATUS_RESP) | List of Status Responses. |
| Status | STATUS | Status of operation. |

[Delete Tables 39, 40 and 41.]

7.4.16.1.2 Semantics of the service primitive

The parameters of the primitive are as follows:

MIH_Configure_Link.request (

DestinationIdentifier,

LinkIdentifier,

ConfigurationRequestsList

)

| Name | Type | Description |
| --- | --- | --- |
| Destination Identifier | MIHF_ID | [Same as D5] |
| Link Identifier | LINK_TUPLE_ID | [Same as D5] |
| Configuration Request List | LIST(LINK_CONFIG_REQ) | List of Configuration Requests. |

[Remove Table 32]

7.4.16.2.2 Semantics of the service primitive

The parameters of the primitive are as follows:

MIH_Configure_Link.confirm (

SourceIdentifier,

LinkIdentifier,

ConfigurationResponseList,

Status

)

| Name | Type | Description |
| --- | --- | --- |
| Source Identifier | MIHF_ID | [Same as D5] |
| Link Identifier | LINK_TUPLE_ID | [Same as D5] |
| Configuration Response List | LIST(LINK_CONFIG_STATUS) | A type to represent a link status being requested. |
| Status | STATUS | Status of operation. |

[Remove Table 44.]

7.4.17.1.2 Semantics of the service primitive

The parameters of the primitive are as follows:

MIH_Scan.request (

DestinationIdentifier,

ScanLinkIdentifier

)

| Name | Type | Description |
| --- | --- | --- |
| Destination Identifier | MIHF_ID | [Same as D5] |
| Scan Link Identifier | LINK_ID | [Same as D5] |

7.4.17.2.2 Semantics of the service primitive

The parameters of the primitive are as follows:

MIH_Scan.confirm (

SourceIdentifier,

ScanLinkIdentifier,

ScanResponseSets,

Status

)

| Name | Type | Description |
| --- | --- | --- |
| Source Identifier | MIHF_ID | [Same as D5] |
| Scan Link Identifier | LINK_ID | [Same as D5] |
| Scan Response Sets | LIST(LINK_SCAN_RESP) | A list of discovered PoAs and related information. |
| Status | STATUS | Status of operation. |

[Remove Table 45.]

7.4.18.1.2 Semantics of service primitive

The parameters of the primitive are as follows:

MIH_Net_HO_Candidate_Query.request (

DestinationIdentifier,

SuggestedNewLinkList,

HandoverMode,

OldLinkAction,

QueryResourceList

)

| Name | Type | Description |
| --- | --- | --- |
| Destination Identifier | MIHF_ID | [Same as D5] |
| Suggested New Link List | LIST(LINK_POA_LIST) | A list of PoAs for each link, suggesting the new access networks to which handover initiation should be considered. The access networks towards the top of the list are more preferable than those towards the bottom of the list. |
| Handover Mode | HANDOVER_MODE | The handover mode may influence the manner in which links are prepared for handover. |
| Old Link Actions | LINK_ACTIONS | Specifies the suggested action on link during handover. |
| Query Resource List | HANDOVER_QUERY_RESOURCE | TBD. |

[Remove Table 46]

[Remove lines 9 – 26 of page 127.]

7.4.18.2.2 Semantics of service primitive

The parameters of the primitive are as follows:

MIH_Net_HO_Candidate_Query.indication (

SourceIdentifier,

SuggestedNewLinkList,

HandoverMode,

OldLinkAction,

QueryResourceList

)

| Name | Type | Description |
| --- | --- | --- |
| Source Identifier | MIHF_ID | [Same as D5] |
| Suggested New Link List | LIST(LINK_POA_LIST) | A list of PoAs for each link, suggesting the new access networks to which handover initiation should be considered. The access networks towards the top of the list are more preferable than those towards the bottom of the list. |
| Handover Mode | HANDOVER_MODE | The handover mode may influence the manner in which links are prepared for handover. |
| Old Link Actions | LINK_ACTIONS | Specifies the suggested action on link during handover. |
| Query Resource List | HANDOVER_QUERY_RESOURCE | TBD. |

7.4.18.3.2 Semantics of service primitive

The parameters of the primitive are as follows:

MIH_Net_HO_Candidate_Query.response (

DestinationIdentifier,

CurrentLinkIdentifier,

HandoverAck,

PreferredLinkList,

RequestedResourceSet,

ErrorCode,

Status

)

| Name | Type | Description |
| --- | --- | --- |
| Destination Identifier | MIHF_ID | [Same as D5] |
| Current Link Identifier | LINK_TUPLE_ID | [Same as D5] |
| Handover Ack | HANDOVER_ACK | If the handover has to be aborted then a reason code is provided |
| Preferred Link List | LIST(LINK_POA_LIST) | A list of PoAs for each link, suggesting the new access networks to which handover initiation should be considered. This may be different than the networks that were suggested in the handover request. The list is sorted from most preferred first to least preferred last. |
| Requested Resource Set | LINK_RESOURCE_SET | [Same as D5] |
| Error Code | ERROR_CODE | [Same as D5] |
| Status | STATUS | Status of operation. |

7.4.18.4.2 Semantics of service primitive

The parameters of the primitive are as follows:

MIH_Net_HO_Candidate_Query.confirm (

SourceIdentifier,

CurrentLinkIdentifier,

HandoverAck,

PreferredLinkList,

RequestedResourceSet,

ErrorCode,

Status

)

| Name | Type | Description |
| --- | --- | --- |
| Source Identifier | MIHF_ID | [Same as D5] |
| Current Link Identifier | LINK_TUPLE_ID | [Same as D5] |
| Handover Ack | HANDOVER_ACK | If the handover has to be aborted then a reason code is provided |
| Preferred Link List | LIST(LINK_POA_LIST) | A list of PoAs for each link, suggesting the new access networks to which handover initiation should be considered. This may be different than the networks that were suggested in the handover request. The list is sorted from most preferred first to least preferred last. |
| Requested Resource Set | LINK_RESOURCE_SET | [Same as D5] |
| Error Code | ERROR_CODE | [Same as D5] |
| Status | STATUS | Status of operation. |

7.4.19.1.2 Semantics of service primitive

The parameters of the primitive are as follows:

MIH_MN_HO_Candidate_Query.request (

DestinationIdentifier,

CurrentLinkIdentifier,

CandidateLinkList,

QueryResourceList,

IPConfigurationMethod,

DHCPServerAddress,

FAAddress,

AccessRouterAddress

)

| Name | Type | Description |
| --- | --- | --- |
| Destination Identifier | MIHF_ID | [Same as D5] |
| Current Link Identifier | LINK_TUPLE_ID | [Same as D5] |
| Preferred Link List | LIST(LINK_POA_LIST) | A list of PoAs for each link, identifying candidate networks to which handover needs to be initiated. The list is sorted from most preferred first to least preferred last. |
| Query Resource List | QUERY_RESOURCE_LIST | [Same as D5] |
| IP Configuration Method | IP_CONFIG_METHODS | Current IP configuration method is included. |
| DHCP Server Address | IP_ADDRESS | [Same as D5] |
| FA Address | IP_ADDRESS | [Same as D5] |
| Access Router Address | IP_ADDRESS | [Same as D5] |

7.4.19.2.2 Semantics of service primitive

The parameters of the primitive are as follows:

MIH_MN_HO_Candidate_Query.indication (

SourceIdentifier,

CurrentLinkIdentifier,

CandidateLinkList,

QueryResourceList,

IPConfigurationMethod,

DHCPServerAddress,

FAAddress,

AccessRouterAddress

)

| Name | Type | Description |
| --- | --- | --- |
| Source Identifier | MIHF_ID | [Same as D5] |
| Current Link Identifier | LINK_TUPLE_ID | [Same as D5] |
| Candidate Link List | LIST(LINK_POA_LIST) | A list of PoAs for each link, identifying candidate networks to which handover needs to be initiated. The list is sorted from most preferred first to least preferred last. |
| Query Resource List | QUERY_RESOURCE_LIST | [Same as D5] |
| IP Configuration Method | IP_CONFIG_METHODS | Current IP configuration method is included. |
| DHCP Server Address | IP_ADDRESS | [Same as D5] |
| FA Address | IP_ADDRESS | [Same as D5] |
| Access Router Address | IP_ADDRESS | [Same as D5] |

7.4.19.3.2 Semantics of service primitive

The parameters of the primitive are as follows:

MIH_MN_HO_Candidate_Query.response (

DestinationIdentifier,

CurrentLinkIdentifier,

PreferredCandidateLinkList,

AvailableResourceSet,

IPConfigurationMethod,

DHCPServerAddress,

FAAddress,

AccessRouterAddress,

IPAddressInformationStatus,

Status

)

| Name | Type | Description |
| --- | --- | --- |
| Destination Identifier | MIHF_ID | [Same as D5] |
| Current Link Identifier | LINK_TUPLE_ID | [Same as D5] |
| Preferred Candidate Link List | LIST(LINK_POA_LIST) | A list of PoAs for each link, identifying candidate networks to which handover needs to be initiated. The list is sorted from most preferred first to least preferred last. |
| Available Resource Set | LIST(LINK_RESOURCE_SET) | A list of available resources for each link. |
| Query Resource List | QUERY_RESOURCE_LIST | [Same as D5] |
| IP Configuration Method | IP_CONFIG_METHODS | Current IP configuration method is included. |
| DHCP Server Address | IP_ADDRESS | [Same as D5] |
| FA Address | IP_ADDRESS | [Same as D5] |
| Access Router Address | IP_ADDRESS | [Same as D5] |
| IP Address Information Status | IP_CONFIG_STATUS |  |
| Status | STATUS | Status of operation. |

7.4.19.4.2 Semantics of service primitive

The parameters of the primitive are as follows:

MIH_MN_HO_Candidate_Query.confirm (

SourceIdentifier,

CurrentLinkIdentifier,

PreferredCandidateLinkList,

AvailableResourceSet,

IPConfigurationMethod,

DHCPServerAddress,

FAAddress,

AccessRouterAddress,

IP Address Information Status,

Status

)

| Name | Type | Description |
| --- | --- | --- |
| Source Identifier | MIHF_ID | [Same as D5] |
| Current Link Identifier | LINK_TUPLE_ID | [Same as D5] |
| Preferred Candidate Link List | LIST(LINK_POA_LIST) | A list of PoAs for each link, identifying candidate networks to which handover needs to be initiated. The list is sorted from most preferred first to least preferred last. |
| Available Resource Set | LIST(LINK_RESOURCE_SET) | A list of available resources for each link. |
| Query Resource List | QUERY_RESOURCE_LIST | [Same as D5] |
| IP Configuration Method | IP_CONFIG_METHODS | Current IP configuration method is included. |
| DHCP Server Address | IP_ADDRESS | [Same as D5] |
| FA Address | IP_ADDRESS | [Same as D5] |
| Access Router Address | IP_ADDRESS | [Same as D5] |
| IP Address Information Status | IP_CONFIG_STATUS |  |
| Status | STATUS | Status of operation. |

7.4.20.1.2 Semantics of service primitive

The parameters of the primitive are as follows:

MIH_N2N_HO_Query_Resources.request (

DestinationIdentifier,

QueryResourceList,

IPConfigurationMethod,

DHCPServerAddress,

FAAddress,

AccessRouterAddress

)

| Name | Type | Description |
| --- | --- | --- |
| Destination Identifier | MIHF_ID | [Same as D5] |
| Query Resource List | QUERY_RESOURCE_LIST | [Same as D5] |
| IP Configuration Method | IP_CONFIG_METHODS | Current IP configuration method is included. |
| DHCP Server Address | IP_ADDRESS | [Same as D5] |
| FA Address | IP_ADDRESS | [Same as D5] |
| Access Router Address | IP_ADDRESS | [Same as D5] |

7.4.20.2.2 Semantics of service primitive

The parameters of the primitive are as follows:

MIH_N2N_HO_Query_Resources.indication (

SourceIdentifier,

QueryResourceList,

IPConfigurationMethod,

DHCPServerAddress,

FAAddress,

AccessRouterAddress

)

| Name | Type | Description |
| --- | --- | --- |
| Source Identifier | MIHF_ID | [Same as D5] |
| Query Resource List | QUERY_RESOURCE_LIST | [Same as D5] |
| IP Configuration Method | IP_CONFIG_METHODS | Current IP configuration method is included. |
| DHCP Server Address | IP_ADDRESS | [Same as D5] |
| FA Address | IP_ADDRESS | [Same as D5] |
| Access Router Address | IP_ADDRESS | [Same as D5] |

7.4.20.3.2 Semantics of service primitive

The parameters of the primitive are as follows:

MIH_N2N_HO_Query_Resources.response (

DestinationIdentifier,

ResourceStatus,

AvailableResourceSet,

IPConfigurationMethod,

DHCPServerAddress,

FAAddress,

AccessRouterAddress,

IPAddressInformationStatus,

Status

)

| Name | Type | Description |
| --- | --- | --- |
| Destination Identifier | MIHF_ID | [Same as D5] |
| Resource Status | LINK_RESOURCE_STATUS | Specifies whether requested resources are available or not at the new PoA. |
| Available Resource Set | LIST(LINK_RESOURCE_SET) | A list of available resources for each link. |
| IP Configuration Method | IP_CONFIG_METHODS | Current IP configuration method is included. |
| DHCP Server Address | IP_ADDRESS | [Same as D5] |
| FA Address | IP_ADDRESS | [Same as D5] |
| Access Router Address | IP_ADDRESS | [Same as D5] |
| IP Address Information Status | IP_CONFIG_STATUS |  |
| Status | STATUS | Status of operation. |

The parameters of the primitive are as follows:

MIH_N2N_HO_Query_Resources.confirm (

SourceIdentifier,

ResourceStatus,

AvailableResourceSet,

IPConfigurationMethod,

DHCPServerAddress,

FAAddress,

AccessRouterAddress,

IPAddressInformationStatus,

Status

)

| Name | Type | Description |
| --- | --- | --- |
| Source Identifier | MIHF_ID | [Same as D5] |
| Resource Status | LINK_RESOURCE_STATUS | Specifies whether requested resources are available or not at the new PoA. |
| Available Resource Set | LIST(LINK_RESOURCE_SET) | A list of available resources for each link. |
| IP Configuration Method | IP_CONFIG_METHODS | Current IP configuration method is included. |
| DHCP Server Address | IP_ADDRESS | [Same as D5] |
| FA Address | IP_ADDRESS | [Same as D5] |
| Access Router Address | IP_ADDRESS | [Same as D5] |
| IP Address Information Status | IP_CONFIG_STATUS |  |
| Status | STATUS | Status of operation. |

7.4.21.1.2 Semantics of service primitive

The parameters of the primitive are as follows:

MIH_Net_HO_Commit.request (

DestinationIdentifier,

LinkActionSetList

)

[Note: A list of {LinkIdentifer(n), LinkActions(n), ExecutionTime(n)} is replaced with LinkActionSetList.]

| Name | Type | Description |
| --- | --- | --- |
| Destination Identifier | MIHF_ID | [Same as D5] |
| Handover Commit Parameter List | LIST(LINK_ACTION_REQ) | A list of handover action requests. |

7.4.21.2.2 Semantics of service primitive

The parameters of the primitive are as follows:

MIH_Net_HO_Commit.indication (

LinkActionSetList

)

[Note: A list of {LinkIdentifer(n), LinkActions(n), ExecutionTime(n)} is replaced with LinkActionSetList.]

| Name | Type | Description |
| --- | --- | --- |
| Source Identifier | MIHF_ID | [Same as D5] |
| Link Action Set List | LIST(LINK_ACTION_REQ) | A list of handover action requests. |

7.4.21.3.2 Semantics of service primitive

The parameters of the primitive are as follows:

MIH_Net_HO_Commit.response (

DestinationIdentifier,

LinkActionRespList,

Status

)

[Note: A list of {LinkIdentifer(n), LinkActionsResutCode(n)} is replaced with LinkActionRespList.]

| Name | Type | Description |
| --- | --- | --- |
| Destination Identifier | MIHF_ID | [Same as D5] |
| Link Action Resp List | LIST(LINK_ACTION_RESP) | A list of handover action results. |
| Status | STATUS | Status of operation. |

7.4.21.4.2 Semantics of service primitive

The parameters of the primitive are as follows:

MIH_Net_HO_Commit.confirm (

SourceIdentifier,

LinkActionRespList,

Status

)

[Note: A list of {LinkIdentifer(n), LinkActionsResutCode(n)} is replaced with LinkActionRespList.]

| Name | Type | Description |
| --- | --- | --- |
| Source Identifier | MIHF_ID | [Same as D5] |
| Handover Result Parameter List | LIST(LINK_ACTION_RESP) | A list of handover action results. |
| Status | STATUS | Status of operation. |

7.4.22.1.2 Semantics of service primitive

The parameters of the primitive are as follows:

MIH_MN_HO_Commit.request (

DestinationIdentifier,

CurrentLinkIdentifier,

TargetLinkIdentifier,

OldLinkActions

)

[Note: TargetPoA is removed because it is contained in TargetLinkIdentifier.]

| Name | Type | Description |
| --- | --- | --- |
| Destination Identifier | MIHF_ID | [Same as D5] |
| Current Link Identifier | LINK_TUPLE_ID | [Same as D5] |
| Target Link Identifier | LINK_TUPLE_ID | [Same as D5] |
| Old Link Actions | LINK_ACTIONS | Specifies the suggested action on link during handover. |

7.4.22.2.2 Semantics of service primitive

The parameters of the primitive are as follows:

MIH_MN_HO_Commit.indication (

SourceIdentifier,

CurrentLinkIdentifier,

TargetLinkIdentifier,

OldLinkActions

)

[Note: TargetPoA is removed because it is contained in TargetLinkIdentifier.]

| Name | Type | Description |
| --- | --- | --- |
| Source Identifier | MIHF_ID | [Same as D5] |
| Current Link Identifier | LINK_TUPLE_ID | [Same as D5] |
| Target Link Identifier | LINK_TUPLE_ID | [Same as D5] |
| Old Link Actions | LINK_ACTIONS | Specifies the suggested action on link during handover. |

7.4.22.3.2 Semantics of service primitive

The parameters of the primitive are as follows:

MIH_MN_HO_Commit.response (

DestinationIdentifier,

CurrentLinkIdentifier,

Status

)

| Name | Type | Description |
| --- | --- | --- |
| Destination Identifier | MIHF_ID | [Same as D5] |
| Current Link Identifier | LINK_TUPLE_ID | [Same as D5] |
| Status | STATUS | Status of operation. |

7.4.22.4.2 Semantics of service primitive

The parameters of the primitive are as follows:

MIH_MN_HO_Commit.confirm (

SourceIdentifier,

CurrentLinkIdentifier,

Status

)

| Name | Type | Description |
| --- | --- | --- |
| Source Identifier | MIHF_ID | [Same as D5] |
| Current Link Identifier | LINK_TUPLE_ID | [Same as D5] |
| Status | STATUS | Status of operation. |

The parameters of the primitive are as follows:

MIH_MN_HO_Complete.request (

DestinationIdentifier,

LinkIdentifier,

HandoverResult

)

| Name | Type | Description |
| --- | --- | --- |
| Destination Identifier | MIHF_ID | [Same as D5] |
| Link Identifier | LINK_TUPLE_ID | [Same as D5] |
| Handover Result | HANDOVER_RESULT | Handover result. |

7.4.23.2.2 Semantics of service primitive

The parameters of the primitive are as follows:

MIH_MN_HO_Complete.indication (

SourceIdentifier,

LinkIdentifier,

HandoverResult

)

| Name | Type | Description |
| --- | --- | --- |
| Source Identifier | MIHF_ID | [Same as D5] |
| Link Identifier | LINK_TUPLE_ID | [Same as D5] |
| Handover Result | HANDOVER_RESULT | Handover result. |

7.4.23.3.2 Semantics of service primitive

The parameters of the primitive are as follows:

MIH_MN_HO_Complete.response (

DestinationIdentifier,

CurrentLinkIdentifier,

Status

)

| Name | Type | Description |
| --- | --- | --- |
| Destination Identifier | MIHF_ID | [Same as D5] |
| Current Link Identifier | LINK_TUPLE_ID | [Same as D5] |
| Status | STATUS | Status of operation. |

7.4.23.4.2 Semantics of service primitive

The parameters of the primitive are as follows:

MIH_MN_HO_Complete.confirm (

SourceIdentifier,

CurrentLinkIdentifier,

Status

)

| Name | Type | Description |
| --- | --- | --- |
| Source Identifier | MIHF_ID | [Same as D5] |
| Current Link Identifier | LINK_TUPLE_ID | [Same as D5] |
| Status | STATUS | Status of operation. |

7.4.24.1.2 Semantics of service primitive

The parameters of the primitive are as follows:

MIH_N2N_HO_Complete.request (

DestinationIdentifier,

CurrentLinkIdentifier,

HandoverResult

)

| Name | Type | Description |
| --- | --- | --- |
| Destination Identifier | MIHF_ID | [Same as D5] |
| Link Identifier | LINK_TUPLE_ID | [Same as D5] |
| Handover Result | HANDOVER_RESULT | Handover result. |

7.4.24.2.2 Semantics of service primitive

The parameters of the primitive are as follows:

MIH_N2N_HO_Complete.indication(

SourceIdentifier,

CurrentLinkIdentifier,

HandoverResult

)

| Name | Type | Description |
| --- | --- | --- |
| Source Identifier | MIHF_ID | [Same as D5] |
| Link Identifier | LINK_TUPLE_ID | [Same as D5] |
| Handover Result | HANDOVER_RESULT | Handover result. |

7.4.24.3.2 Semantics of service primitive

The parameters of the primitive are as follows:

MIH_N2N_HO_Complete.response (

DestinationIdentifier,

CurrentLinkIdentifier,

ResourceStatus,

Status

)

| Name | Type | Description |
| --- | --- | --- |
| Destination Identifier | MIHF_ID | [Same as D5] |
| Current Link Identifier | LINK_TUPLE_ID | [Same as D5] |
| Resource Status | LINK_RESOURCE_RETENTION_STATUS | Status of resource. |
| Status | STATUS | Status of operation. |

7.4.24.4.2 Semantics of service primitive

The parameters of the primitive are as follows:

MIH_N2N_HO_Complete.confirm (

SourceIdentifier,

CurrentLinkIdentifier,

ResourceStatus,

Status

)

| Name | Type | Description |
| --- | --- | --- |
| Source Identifier | MIHF_ID | [Same as D5] |
| Current Link Identifier | LINK_TUPLE_ID | [Same as D5] |
| Resource Status | LINK_RESOURCE_RETENTION_STATUS | Status of resource. |
| Status | STATUS | Status of operation. |

7.4.25.1.2 Semantics of service primitive

The parameters of the primitive are as follows:

MIH_Get_Information.request (

DestinationIdentifier,

InfoQueryBinaryDataList,

InfoQueryRDFDataList,

InfoQueryRDFSchemaURL,

InfoQueryRDFSchemaList,

MaxResponseSize

)

| Name | Type | Description |
| --- | --- | --- |
| Destination Identifier | MIHF_ID | [Same as D5] |
| Info Query Binary Data List | LIST(INFO_QUERY_BINARY_DATA) | A list of binary queries. |
| Info Query RDFDataList | LIST(INFO_QUERY_RDF_DATA) | A list of RDF queries. |
| Info Query RDF Schema URL | NULL | An RDF Schema URL query. |
| Info Query RDF Schema List | LIST(INFO_QUERY_RDF_SCHEMA) | A list of RDF Schema queries. |
| Max Response Size | INTEGER(2) | [Same as D5] |

[Remove the entire text after the table.]

7.4.25.2.2 Semantics of service primitive

The parameters of the primitive are as follows:

MIH_Get_Information.indication (

SourceIdentifier,

InfoQueryBinaryDataList,

InfoQueryRDFDataList,

InfoQueryRDFSchemaURL,

InfoQueryRDFSchemaList,

MaxResponseSize

)

| Name | Type | Description |
| --- | --- | --- |
| Source Identifier | MIHF_ID | [Same as D5] |
| Info Query Binary Data List | LIST(QUERY_BINARY_DATA) | A list of binary queries. |
| Info Query RDF Data List | LIST(INFO_QUERY_RDF_DATA) | A list of RDF queries. |
| Info Query RDF Schema URL | NULL | An RDF Schema URL query. |
| Info Query RDF Schema List | LIST(INFO_QUERY_RDF_SCHEMA) | A list of RDF Schema queries. |
| Max Response Size | INTEGER(2) | [Same as D5] |

[Remove the entire text after the table.]

7.4.25.3.2 Semantics of service primitive

The parameters of the primitive are as follows:

MIH_Get_Information.response (

DestinationIdentifier,

InfoRespBinaryDataList,

InfoRespRDFDataList,

InfoRespRDFSchemaURL,

InfoRespRDFSchemaList,

Status

)

| Name | Type | Description |
| --- | --- | --- |
| Destination Identifier | MIHF_ID | [Same as D5] |
| Info Resp Binary Data List | LIST(INFO_RESP_BINARY_DATA) | A list of binary query responses. |
| Info Resp RDFDataList | LIST(INFO_RESP_RDF_DATA) | A list of RDF query responses. |
| Info Resp RDF Schema URL List | LIST(INFO_RESP_SCHEMA_URL) | A list of RDF Schema URL query responses. |
| Info Resp RDF Schema List | LIST(INFO_RESP_RDF_SCHEMA) | A list of RDF Schema query responses. |
| Status | STATUS | Status of operation. |

[Remove the entire text after the table.]

7.4.25.3.2 Semantics of service primitive

The parameters of the primitive are as follows:

MIH_Get_Information.confirm (

SourceIdentifier,

InfoRespBinaryDataList,

InfoRespRDFDataList,

InfoRespRDFSchemaURL,

InfoRespRDFSchemaList,

Status

)

| Name | Type | Description |
| --- | --- | --- |
| Source Identifier | MIHF_ID | [Same as D5] |
| Info Resp Binary Data List | LIST(INFO_RESP_BINARY_DATA) | A list of binary query responses. |
| Info Resp RDFDataList | LIST(INFO_RESP_RDF_DATA) | A list of RDF query responses. |
| Info Resp RDF Schema URL List | LIST(INFO_RESP_SCHEMA_URL) | A list of RDF Schema URL query responses. |
| Info Resp RDF Schema List | LIST(INFO_RESP_RDF_SCHEMA) | A list of RDF Schema query responses. |
| Status | STATUS | Status of operation. |

[Remove the entire text after the table.]

7.6.1.1.2 Semantics

The primitive shall provide parameters as follows:

MIH_TP_Connect.request (

TransportType,

SourceAddress,

DestinationAddress

)

| Name | Type | Description |
| --- | --- | --- |
| Transport Type | TRANSPORT_TYPE | Identifies the protocol layer specific transport option. |
| Source Address | TRANSPORT_ADDRESS | [Same as D5] |
| Destination Address | TRANSPORT_ADDRESS | [Same as D5] |

7.6.1.2.2 Semantics

The primitive shall provide parameters as follows:

MIH_TP_Connect.indication (

TransportType,

SourceAddress,

DestinationAddress

)

| Name | Type | Description |
| --- | --- | --- |
| Transport Type | TRANSPORT_TYPE | Identifies the protocol layer specific transport option. |
| Source Address | TRANSPORT_ADDRESS | [Same as D5] |
| Destination Address | TRANSPORT_ADDRESS | [Same as D5] |

7.6.1.3.2 Semantics

The primitive shall provide parameters as follows:

MIH_TP_Connect.response (

TransportType,

SourceAddress,

DestinationAddress,

Status

)

| Name | Type | Description |
| --- | --- | --- |
| Transport Type | TRANSPORT_TYPE | Identifies the protocol layer specific transport option. |
| Source Address | TRANSPORT_ADDRESS | [Same as D5] |
| Destination Address | TRANSPORT_ADDRESS | [Same as D5] |
| Status | STATUS | Status of operation. |

7.6.1.4.2 Semantics

The primitive shall provide parameters as follows:

MIH_TP_Connect.confirm (

TransportType,

SourceAddress,

DestinationAddress,

Status

)

| Name | Type | Description |
| --- | --- | --- |
| Transport Type | TRANSPORT_TYPE | Identifies the protocol layer specific transport option. |
| Source Address | TRANSPORT_ADDRESS | [Same as D5] |
| Destination Address | TRANSPORT_ADDRESS | [Same as D5] |
| Status | STATUS | Status of operation. |

7.6.2.1.2 Semantics

The primitive shall provide parameters as follows:

MIH_TP_Disconnect.request (

TransportType,

SourceAddress,

DestinationAddress

)

| Name | Type | Description |
| --- | --- | --- |
| Transport Type | TRANSPORT_TYPE | Identifies the protocol layer specific transport option. |
| Source Address | TRANSPORT_ADDRESS | [Same as D5] |
| Destination Address | TRANSPORT_ADDRESS | [Same as D5] |

7.6.2.2.2 Semantics

The primitive shall provide parameters as follows:

MIH_TP_Disconnect.indication (

TransportType,

SourceAddress,

DestinationAddress

)

| Name | Type | Description |
| --- | --- | --- |
| Transport Type | TRANSPORT_TYPE | Identifies the protocol layer specific transport option. |
| Source Address | TRANSPORT_ADDRESS | [Same as D5] |
| Destination Address | TRANSPORT_ADDRESS | [Same as D5] |

7.6.3.1.2 Semantics

The primitive shall provide parameters as follows:

MIH_TP_Reset.request (

TransportType,

SourceAddress,

DestinationAddress

)

| Name | Type | Description |
| --- | --- | --- |
| Transport Type | TRANSPORT_TYPE | Identifies the protocol layer specific transport option. |
| Source Address | TRANSPORT_ADDRESS | [Same as D5] |
| Destination Address | TRANSPORT_ADDRESS | [Same as D5] |

7.6.3.2.2 Semantics

The primitive shall provide parameters as follows:

MIH_TP_Reset.indication (

TransportType,

SourceAddress,

DestinationAddress

)

| Name | Type | Description |
| --- | --- | --- |
| Transport Type | TRANSPORT_TYPE | Identifies the protocol layer specific transport option. |
| Source Address | TRANSPORT_ADDRESS | [Same as D5] |
| Destination Address | TRANSPORT_ADDRESS | [Same as D5] |

7.6.3.3.2 Semantics

The primitive shall provide parameters as follows:

MIH_TP_Reset.response (

TransportType,

SourceAddress,

DestinationAddress,

Status

)

| Name | Type | Description |
| --- | --- | --- |
| Transport Type | TRANSPORT_TYPE | Identifies the protocol layer specific transport option. |
| Source Address | TRANSPORT_ADDRESS | [Same as D5] |
| Destination Address | TRANSPORT_ADDRESS | [Same as D5] |
| Status | STATUS | Status of operation. |

7.6.3.4.2 Semantics

The primitive shall provide parameters as follows:

MIH_TP_Reset.confirm (

TransportType,

SourceAddress,

DestinationAddress,

Status

)

| Name | Type | Description |
| --- | --- | --- |
| Transport Type | TRANSPORT_TYPE | Identifies the protocol layer specific transport option. |
| Source Address | TRANSPORT_ADDRESS | [Same as D5] |
| Destination Address | TRANSPORT_ADDRESS | [Same as D5] |
| Status | STATUS | Status of operation. |

7.6.4.1.2 Semantics

The primitive shall provide parameters as follows:

The parameters of the primitive are as follows:

MIH_TP_Data.request (

TransportType,

SourceAddress,

DestinationAddress,

ReliableDeliveryFlag,

MIHProtocolPDU

)

| Name | Type | Description |
| --- | --- | --- |
| Transport Type | TRANSPORT_TYPE | Identifies the protocol layer specific transport option. |
| Source Address | TRANSPORT_ADDRESS | [Same as D5] |
| Destination Address | TRANSPORT_ADDRESS | [Same as D5] |
| Reliable Delivery Flag | BOOLEAN | Indicate whether an acknowledgment is required. TRUE: Reliable delivery is required. FALUSE: Reliable delivery is not required. |
| MIH Protocol PDU | OCTET_STRING | [Same as D5] |

7.6.4.2.2 Semantics

The primitive shall provide parameters as follows:

The parameters of the primitive are as follows:

MIH_TP_Data.indication (

TransportType,

SourceAddress,

DestinationAddress,

ReliableDeliveryFlag,

MIHProtocolPDU

)

| Name | Type | Description |
| --- | --- | --- |
| Transport Type | TRANSPORT_TYPE | Identifies the protocol layer specific transport option. |
| Source Address | TRANSPORT_ADDRESS | [Same as D5] |
| Destination Address | TRANSPORT_ADDRESS | [Same as D5] |
| Reliable Delivery Flag | BOOLEAN | Indicate whether an acknowledgment is required. TRUE: Reliable delivery is required. FALUSE: Reliable delivery is not required. |
| MIH Protocol PDU | OCTET_STRING | [Same as D5] |

7.6.4.3.2 Semantics

The primitive shall provide parameters as follows:

MIH_TP_Data.response (

TransportType,

SourceAddress,

DestinationAddress,

Status

)

| Name | Type | Description |
| --- | --- | --- |
| Transport Type | TRANSPORT_TYPE | Identifies the protocol layer specific transport option. |
| Source Address | TRANSPORT_ADDRESS | [Same as D5] |
| Destination Address | TRANSPORT_ADDRESS | [Same as D5] |
| Status | STATUS | Status of operation. |

7.6.4.4.2 Semantics

The primitive shall provide parameters as follows:

MIH_TP_Data.confirm (

TransportType,

SourceAddress,

DestinationAddress,

Status

)

| Name | Type | Description |
| --- | --- | --- |
| Transport Type | TRANSPORT_TYPE | Identifies the protocol layer specific transport option. |
| Source Address | TRANSPORT_ADDRESS | [Same as D5] |
| Destination Address | TRANSPORT_ADDRESS | [Same as D5] |
| Status | STATUS | Status of operation. |

7.7.1.2.2 Semantics

The parameter of the primitive is as follows:

MIH_NMS_Initialize.confirm (

Status

)

| Name | Type | Description |
| --- | --- | --- |
| Status | STATUS | Status of operation. |

7.7.2.1.2 Semantics

The parameter of the primitive is as follows:

The parameter of the primitive is as follows:

MIH_NMS_Get_State.request (

StateInformationRequestList

)

| Name | Type | Description |
| --- | --- | --- |
| State Information Request List | LIST(MNS_STATE_REQ) | List of the state information to be queried. |

7.7.2.2.2 Semantics

The parameter of the primitive is as follows:

MIH_NMS_Get_State.confirm (

StateInformationResponseList,

Status

)

| Name | Type | Description |
| --- | --- | --- |
| State Information Response List | LIST(MNS_STATE_RESP) | A list of a pair of {state information; state information value}. |
| Status | STATUS | Status of operation. |

7.7.3.2.2 Semantics

The parameter of the primitive is as follows:

MIH_NMS_Reset.confirm (

Status

)

| Name | Type | Description |
| --- | --- | --- |
| Status | STATUS | Status of operation. |

## A new Annex for Data Type Definition

Insert the following Annex M as Normative.

### Annex M. Data Type Definition in 802.21

### M.1 Basic Data Types

The data types defined in this section are used as the basis for defining any other data types. All basic data types are for general purpose.

| Type Name | Definition | Range | Binary Encoding Rule |
| --- | --- | --- | --- |
| LIST(DATATYPE) | A list of values of type DATATYPE. | N/A | See Annex M.1.1. |
| SEQUENCE(DATATYPE1,DATATYPE2[,…]) | A data type that consists of two or mote data types. | N/A | DATATYPE1,DATATYPE2,[,…] are encoded in the order of appearance. Each data type is encoded using the encoding rule for the data type. |
| CHOICE( DATATYPE1,DATATYPE2[,…]) | A data type that consists only one of data types DATATYPE1,DATATYPE2[,…]). | N/A | A one-octet Selector field, followed by a variable length Value field. The Selector value determines the data type. If Selector==i, (i+1)-th data type in the list of data types DATATYPE1,DATATYPE2,[,…] is selected. The Value field is encoded using the encoding the rule for the selected data type. |
| NULL | A data type with an empty data. | N/A | No octet is encoded for this data type. This data type is used for to define an optional data type. |
| BITMAP(size) | A bitmap of the specified size in number of bits. | Each bit has a value of ‘0’ or ‘1’. | Each bit of a BITMAP(N) value [N=8*i, i=1, 2, …] is encoded as an N/8-octet value in order of significance. |
| INTEGER(size) | An integer of the specified size in number of octets. | Each octet has a value of 0x00 to 0xff. | Each octet of an INTEGER(N) value [N=1,2,..] is encoded in network-byte order into an N-octet field. |
| CHAR(size) | An array of 8-bit characters with the specified number of octets. | N/A | Each character is encoded in order of appearance where each bit of each character is encoded in order of significance. |
| INFORMATION_ELEMENT | A binary encoded structure that is defined in Section 6.4.6. | N/A | See Section 6.4.6. |

M.1.1 Encoding Rule of LIST(DATATYPE) Data Type

The following encoding shall be used for LIST(DATATYPE) data type.

A variable length Length field, followed by a variable length Value field. The Length field value is in terms of number of list elements in the list.

The format of the Length field is defined as follows. Specifically, if the actual number of list elements in the Value field is less than or equal to 127, then:

— The Length field shall occupy one octet

— The MSB of the Length field shall be set to 0, and

— The other 7 bits of the Length field shall be used to indicate the actual length of the Value field in the number of list elements.

If the number of list elements of the Value field is exactly 128, then:

— The Length field shall occupy one octet

— The MSB of the Length field shall be set to the value '1', and

— The other seven bits of the Length field shall all be set to the value '0'.

If the number of list elements of the Value field is more than 127, then:

— The Length field shall occupy more than one octet

— The MSB of the first octet of the Length field shall be set to 1.

— The other 7 bits of the first octet of the Length field shall be used to indicate the number of additional octets of the Length field (i.e., excluding the first octet), and

- The value of the remaining octets (i.e., excluding the first octet) of the Length field, when added to 128, shall be used to indicate the actual length of the Value field in number of list elements.

- Each of list element is encoded in the Value field in the order of appearance.

- If there is no list element, the Value field is not encoded.

For example, an attribute of type LIST(LINK-ID) with two list elements is encoded in the following way (LINK_ID is defined in M.2.3):

| LIST(LINK_ID) (29-octet) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Length (1-octet) | Value (28-octet) |  |  |  |  |  |
| 2 | LINK_ID (14-octet) |  |  | LINK_ID (14-octet) |  |  |
|  | LINK_TYPE (type:INTEGER(4)) (4-octet) | MAC_ADDRESS (type:OCTET_STRING =LIST(CHAR(1)) (10-octet) |  | LINK_TYPE (type:INTEGER(4)) (4-octet) | MAC_ADDRESS (type: OCTET_STRING =LIST(CHAR(1)) (10-octet) |  |
|  | 19 (802.11) | Length (1-octet) | Value (9-octet) | 27 (802.16) | Length (1-octet) | Value (9-octet) |
|  |  | 9 | 2-octet address family followed by 6-octet IEEE MAC address |  | 9 | 2-octet address family followed by 6-octet IEEE MAC address. |

### M.2 Derived Data Types

Derived data types are those which are derived from other data types or parent data types. A derived data type uses the same encoding as the parent data type.

### M.2.1 Data Types for General Purpose

The derived data types defined in this section are for general purpose only.

| Type Name | Derived From | Definition | Valid Range |
| --- | --- | --- | --- |
| ENUMERATED | INTEGER(1) | A type to represent an enumerated attribute. | 0 - 255 |
| BOOLEAN | ENUMERATED | A type to represent a Boolean attribute. | 0 : False, 1 : True |
| OCTET_STRING | LIST(CHAR(1)) | A type to represent an array of octets. | N/A |
| PERCENTAGE | INTEGER(1) | A type to represent a percentage. | 0 - 100 |
| STATUS | ENUMERATED | A type to represent a status of a primitive execution. | 0 Success 1 Unspecified Failure 2 Rejected 3 Authorization Failure 4 Network Error |

### M.2.2 Data Types for Addresses

The data types defined in this section are related to addresses of network elements.

| Type Name | Derived From | Definition | Valid Range |
| --- | --- | --- | --- |
| TRANSPORT_ADDRESS | OCTET_STRING | A type to represent a transport address using Address base type defined in RFC3588. The First 2-octet contains AddressType. | AddressType values are defined in http://www.iana.org/assignments/address-family-numbers. |
| MAC_ADDRESS | TRANSPORT_ADDRESS | A type to represent a MAC address. | The AddressType contains the one used for a specific link layer. |
| IP_ADDRESS | TRANSPORT_ADDRESS | A type to represent an IP address. | The AddressType contains either 1 (IPv4) or 2 (IPv6). |

### M.2.3 Data Types for Link Identification and Manipulation

The data types defined in this section are used for representing attributes for identification and manipulation of links.

| Type Name | Derived From | Definition | Valid Range |
| --- | --- | --- | --- |
| LINK TYPE | INTEGER(4) | A type to represent a link type. | 0: Reserved 1: Wireless - GSM 2: Wireless – GPRS 3: Wireless – EDGE 15: Ethernet 18: Wireless – Other 19: Wireless - IEEE 802.11 22: Wireless - CDMA2000 23: Wireless – UMTS 24: Wireless - cdma2000-HRPD 27: Wireless - IEEE 802.16 28: Wireless - IEEE 802.20 29: Wireless - IEEE 802.22 |

| Type Name | Derived From | Definition | Valid Range |
| --- | --- | --- | --- |
| LINK_ID | SEQUENCE( LINK_TYPE MAC_ADDRESS) | A type to represent the identifier of a link that is not associated with any PoA. The MAC_ADDRESS contains a MAC address of MN. This may be used for the current link being used by MN or the link that was used by MN before handover. | N/A |
| LINK_TUPLE_ ID | SEQUENCE( LINK_ID, CHOICE( MAC_ADDRESS, NULL)) | A type to represent the identifier of a link that may be associated with a PoA. The optional MAC_ADDRESS contains a MAC address of PoA. | N/A |
| LINK_POA_LIST | SEQUENCE( LINK_ID, LIST(MAC_ADDRESS)) | A type to represent a list of PoAs for a particular link. The LIST(MAC_ADDRESS) is sorted from most preferred first to least preferred last. | N/A |

| Type Name | Derived From | Definition | Valid Range |
| --- | --- | --- | --- |
| LINK_ACTIONS | BITMAP(32) | A type to represent a set of actions for a link. The meaning of each link action is defined in Table M3. | Bit 0: LINK_DISCONNECT Bit 1: LINK_LOW_POWER Bit 2: LINK_POWER_DOWN Bit 3: LINK_NO_ACTION Bit 4: LINK_RESOURCE_RETAIN Bit 5: DATA_FORWARDING_REQUEST Bit 6: LINK_POWER_UP Bit 7-31: (Reserved) |

### Table M1: Link Actions

| Action Name | Description |
| --- | --- |
| LINK_DISCONNECT | Disconnect the link connection directly. |
| LINK_LOW_POWER | Cause the link to adjust its RF power level to be low power consumption. |
| LINK_POWER_DOWN | Cause the link to power down so as to stop transmitting and enter idle mode or turn off the radio. |
| LINK_NO_ACTION | The link need not take any action. |
| LINK_RESOURCE_RETAIN | The link will be disconnected but the resource for the link connection still remains so reestablishing the link connection later can be more efficient. |
| DATA_FORWARDING_REQUEST | This indication requires the buffered data at the old serving PoS entity to be forwarded to the new target PoS entity in order to avoid data loss. This action can be taken immediately after the old serving PoS receives the MIH_Handover_Commit.response message from the new target PoS. |
| LINK_POWER_UP | TBD. |

| Type Name | Derived From | Definition | Valid Range |
| --- | --- | --- | --- |
| LINK_ACTION_REQ | SEQUENCE( LINK_ID, LINK_ACTIONS, LINK_AC_EXEC_TIME) | A type to represent a set of handover action request parameters. | N/A |
| LINK_ACTION_RESP | SEQUENCE( LINK_ID, LINK_AC_RESULT_CODE) | A type to represent a set of handover result parameters. | N/A |
| LINK_AC_EXEC_TIME | INTEGER(2) | A type to represent a time (in msec) elapsed before an action needs to be taken, if Execution Time = 0 then the action is expected to be taken immediately. | 0-1000. |
| LINK_AC_RESULT_CODE | ENUMERATED | A type to represent an handover action result. | 0 Resource Unavailable 1 Resource Busy 2 Resource Available 3 Abort Handover 4 Perform Handover |

| Type Name | Derived From | Definition | Valid Range |
| --- | --- | --- | --- |
| LINK_EVENT_LIST | BITMAP(32) | A type to represent a bitmap of link event types. | Bit 0: Link_Up Bit 1: Link_Down Bit 2: Link_Going_Down Bit 3: Link_Event_Rollback Bit 4: Link_Detected Bit 5: Link_Parameters_Report Bit 6: Link_PDU_Transmit_Status Bit 7: Link_Handover_Imminent Bit 8: Link_Handover_Complete Bit 9-31: (Reserved) |

| Type Name | Derived From | Definition | Valid Range |
| --- | --- | --- | --- |
| LINK_COMMAND_LIST | BITMAP(32) | A type to represent a set of supported link events. | Bit 0: Link_Capability_Discover Bit 1: Link_Event_Subscribe Bit 2: Link_Event_Unsubscribe Bit 3: Link_Configure_Thresholds Bit 4: Link_Get_Parameters Bit 5-31: (Reserved) |

| Type Name | Derived From | Definition | Valid Range |
| --- | --- | --- | --- |
| LINK_PARAM | CHOICE( QOS_PARAM, NON_QOS_PARAM) | A type to represent a link parameter. [Note: The NON_QOS_PARAM needs to be defined.] | N/A |

| Type Name | Derived From | Definition | Valid Range |
| --- | --- | --- | --- |
| LINK_CONFIG_STATUS | SEQUENCE( LINK_PARAM_TYPE, CONFIG_STATUS) | A type to represent a status of link parameter configuration. | N/A |
| LINK_PARAM_TYPE | CHOICE( QOS_TYPE, NON_QOS_TYPE) | A type to represent a configuration response. [Note: The NON_QOS_TYPE needs to be defined.] | N/A |
| CONFIG_STATUS | ENUMERATED | A type to represent a status of link parameter configuration. | 0 Success 1 Error |

| Type Name | Derived From | Definition | Valid Range |
| --- | --- | --- | --- |
| LINK_DOWN_REASON | ENUMERATED | A type to represent the reason of a link down event. | See Table M2. |

Table M2: Valid Range of LINK_DOWN_REASON_CODE

[Move Table 34 of D05 here].

| Type Name | Derived From | Definition | Valid Range |
| --- | --- | --- | --- |
| LINK_GOING_DOWN_REASON | ENUMERATED | A type to represent the reason of a link going down event. | See Table M3. |

Table M3: Valid Range of LINK_GOING_DOWN_REASON_CODE

[Move Table 35 of D05 here].

| Type Name | Derived From | Definition | Valid Range |
| --- | --- | --- | --- |
| LINK_MIH_CAPABILITY_FLAG | BOOLEAN | A type to represent whether the MIH capability is supported or not. | TRUE: MIH Capability Not Supported FALSE: MIH Capability Supported |

| Type Name | Derived From | Definition | Valid Range |
| --- | --- | --- | --- |
| LINK_STATUS_REQ | BITMAP(32) | A type to represent a link status being requested. | Bit #0: DEVICE_INFO Bit# 1: OPERATION_MODE Bit# 2: CHANNEL_ID Bit# 3: BATTERY_LEVEL Bit# 4: LINK_QOS_TYPE_LIST Bit# 5-255: reserved |
| LINK_STATUS_RESP | SEQUENCE( LINK_ID, LIST(LINK_STATUS)) | A type to represent a set of parameters of the status of the link specified by LINK_ID. | N/A |
| LINK_STATUS | CHOICE( DEVICE_INFO, OPERATION_MODE, CHANNEL_ID, BATTERY_LEVEL, QOS_TYPE) | A type to represent a parameter of the status of a link. | N/A |
| DEVICE_INFO | OCTET_STRING | A type to represent information on manufacturer, model number, revision number of the software/firmware and serial number in displayable text are returned. | N/A |
| OPERATION_MODE | INTEGER(1) | A type to represent the power mode of a link. | 0x00 Normal Mode 0x01 Power Saving Mode 0x02 Power Down |
| CHANNEL_ID | INTEGER(1) | The ID of the channel currently in use. | N/A |
| BATTERY_LEVEL | PERCENTAGE | Battery level in percentage. | 1 - 100 |

| Type Name | Derived From | Definition | Valid Range |
| --- | --- | --- | --- |
| LINK_CONFIG_REQ | CHOICE( OPERATION_MODE, LIST(LINK_PARAM)) | A type to represent a configuration request. | N/A |

| Type Name | Derived From | Definition | Valid Range |
| --- | --- | --- | --- |
| SIGNAL_STRENGTH | PERCENTAGE | A type to represent signal strength. | 1 - 100 |
| LINK_SCAN_RESP | SEQUENCE( MAC_ADDRESS, SIGNAL_STRENGTH) | A type to represent a scan response. The MAC_ADDRESS contains a MAC address of PoA. | N/A |

| Type Name | Derived From | Definition | Valid Range |
| --- | --- | --- | --- |
| LINK_RESOURCE_SET | SEQUENCE( LINK_ID, AVAILABLE_RESOURCE_LIST) | A type to represent a available resources for a link. | N/A |
| LINK_RESOURCE_STATUS | BOOLEAN | A type to represent whether resources are available or not. | TRUE: Not Available FALSE: Available |

| Type Name | Derived From | Definition | Valid Range |
| --- | --- | --- | --- |
| LINK_RESOURCE_STATUS | BOOLEAN | A type to represent whether resources are available or not. | TRUE: Not Available FALSE: Available |

| Type Name | Derived From | Definition | Valid Range |
| --- | --- | --- | --- |
| LINK_RESOURCE_RETENTION_STATUS | BOOLEAN | A type to represent a status of resource. | TRUE: Retain resources FALSE: Release resources |

### M.2.4 Data Types for QoS

The data types defined in this section are related to QoS.

| Type Name | Derived From | Definition | Valid Range |
| --- | --- | --- | --- |
| QOS_TYPE | CHOICE( QOS_TYPE_GENERIC,…) | A type to represent a QoS type. [Note: Currently QoS type for Generic Link is defined.] | See below. |
| QOS_TYPE_GENERIC | ENUMERATED | A type to represent a QoS type for Generic Link type. | 0: Number of supported CoS 1: Throughput (kbs) 2: Packet Error Rate 3: CoS Minimum Packet Transfer Delay (ms) 4: CoS Average Packet Transfer Delay (ms) 5: CoS Maximum Packet Transfer Delay (ms) 6: CoS Packet Transfer Delay Jitter (ms) 7: CoS Packet Loss Rate 8~255: Reserved [Editor’s note: The QoS parameters for other link specific types are TBD.] |
| … | … | … | … |

| Type Name | Derived From | Definition | Valid Range |
| --- | --- | --- | --- |
| QOS_PARAM | CHOICE( QOS_PARAM_GENERIC, …) | A type to represent QoS parameters for a link type. [Note: Currently QoS parameters for Generic Link is defined.] | N/A |
| QOS_PARAM_GENERIC | CHOICE( NUM_QOS_TYPES, THROUGHPUT, PACKET_ERROR_RATE, MIN_PACKET_DELAY, AVG_PACKET_DELAY, MAX_PACKET_DELAY, PACKET_DELAY_JITTER, PACKET_LOSS_RATE) | A type to represent a QoS parameter. | N/A |
| … | … | … | … |

| Type Name | Derived From | Definition | Valid Range |
| --- | --- | --- | --- |
| NUM_QOS_TYPES | INTEGER(1) | A type to represent the number of differentiable classes of service supported. | 0 – 255 |
| THROUGHPUT | INTEGER(4) | A type to represent the maximum information transfer rate achievable. It is measured in kbs. | 0 - 2^32-1 |
| PACKET_ERROR_RATE | INTEGER(2) | A type to represent a value equal to the integer part of the result of multiplying -100 times the log10 of the ratio between the number of packets received in error and the total number of packets transmitted in a link population of interest. | N/A |
| MIN_PACKET_DELAY | SEQUENCE( COS_ID, INTEGER(2)) | A type to represent the minimum packet transfer delay in msec for the specific CoS specified by the COS_ID. | N/A |
| AVG_PACKET_DELAY | SEQUENCE( COS_ID, INTEGER(2)) | A type to represent the average packet transfer delay in msec for the specific CoS specified by the COS_ID. | N/A |
| MAX_PACKET_DELAY | SEQUENCE( COS_ID, INTEGER(2)) | A type to represent the maximum packet transfer delay in msec for the specific CoS specified by the COS_ID. | N/A |
| PACKET_DELAY_JITTER | SEQUENCE( COS_ID, INTEGER(2)) | A type to represent the packet transfer delay jitter in msec for the specific CoS specified by the COS_ID. | N/A |
| PACKET_LOSS_RATE | SEQUENCE（ COS_ID, INTEGER(2)) | A type to represent the packet loss rate for the specific CoS specified by the COS_ID. The loss rate is equal to the integer part of the result of multiplying -100 times the log10 of the ratio between the number of packets lost and the total number of packets transmitted in the class population of interest. | N/A |
| COS_ID | INTEGER(2) | A type to represent a class of service identifier. | 0 - 255 |

### M.2.5 Data Types for Location

The data types defined in this section are related to location.

| Type Name | Derived From | Definition | Valid Range |
| --- | --- | --- | --- |
| LOCATION | CHOICE( SEQUENCE( CIVIC_LO_METHOD, CIVIC_LOCATION)), SEQUENCE( GEO_LO_METHOD, GEO_LOCATION)) | A type to represent the way location information was derived or discovered. | See below. |
| CIVIC_LO_METHOD | ENUMERATED | A type to represent the way civic address location information was derived or discovered. | 0: Manual 1-255: Reserved |
| CIVIC_LOCATION | SEQUENCE( CIVIC_COUNTRY_CODE, CIVIC_ADDR_ELEM) | A type to represent a civic address. | See below. |
| CIVIC_COUNTRY_CODE | CHAR(2) | A type to represent a country code. | Two-letter ISO 3166 country code in capital ASCII letters. |
| CIVIC_ADDR_ELEM | OCTET_STRING | A type to represent civic address elements. | Civic address elements, as defined in IETF RFC 4676. |
| GEO_LO_METHOD | ENUMERATED | A type to represent the way geospatial location information was derived or discovered. | 0: GPS 1: Assisted GPS 2: Manual 3: Provided by dynamic host configuration protocol (DHCP) 4: Triangulation 5-255: Reserved |
| GEO_LOCATION | CHAR(16) | A type to represent a geospatial location. | See Table M4. |

Table M4: Valid Range of GEOSPATIAL_LOCATION

[Move Table 17 of D05 here].

### M.2.6 Data Types for IP Configuration

The data types defined in this section are related to IP configuration.

| Type Name | Derived From | Definition | Valid Range |
| --- | --- | --- | --- |
| IP_CONFIG_METHODS | BITMAP(32) | A type to represents a set of IP configuration methods. | Bit 0: IPv4 static configuration Bit 1: IPv4 dynamic configuration (DHCPv4) Bit 2: Mobile IPv4 with foreign agent (FA) care of address (CoA) (FA-CoA) Bit 3: Mobile IPv4 without FA (Co-located CoA) Bits 4-10: reserved for IPv4 address configurations Bit 11: IPv6 stateless address configuration Bit 12: IPv6 stateful address configuration (DHCPv6) Bit 13: IPv6 manual configuration Bits 14-31: Reserved. |
| IP_MOBILITY_MGMT | BITMAP(8) | A type to indicate the supported mobility management protocols. | 0: Mobile IPv4 with FA (FA-CoA) 1: Mobile IPv4 without FA (Co-located CoA) 2: Mobile IPv6 3: Mobile IPv6 with DHCPv6 4-7: (Reserved) |
| IP_CONFIG_STATUS | BITMAP(8) | A type to represent a status of IP address configuration methods. | Bit 0: IP configuration Method is not available Bit 1: DHCP Server address is not available Bit 2: FA address is not available Bit 3: Access Router Address is not available Bit 4: No information is provided due to accessibility of same entity (FA, Access Router, DHCP Server, etc.) Bit 5-7: Reserved |
| IP_PREFIX_LEN | INTEGER(1) | A type to represent the length of an IP subnet prefix. | 0 - 32 for IPv4 subnet. 0 – 128 for IPv6 subnet. |
| IP_SUBNET_INFO | SEQUENE( IP_PREFIX_LEN, IP_ADDRESS)) | A type to represent an IP subnet. The IP_PREFIX_LEN contains the bit length of the prefix of the subnet to which the IP_ADDRESS belongs. | N/A |
| IP_RENEWAL_FLAG | BOOLEAN | A type to represent whether MN’s IP address needs to be changed or not. | TRUE: Change Required FALSE: Change Not required |

### M.2.7 Data Types used for Information Service

### M.2.7.1 Data Types used by Information Elements

The data types defined in this section are used only by IEs.

| Type Name | Derived From | Definition | Valid Range |
| --- | --- | --- | --- |
| NETWORK_TYPE | SEQUENCE( LINK_TYPE, CHOICE(BITMAP(64), NULL)) | A type to represent a network type. The BITMAP(64) assignment depends on the LINK_TYPE. | See Table M5. |

Table M5: Valid Range of NETWORK_TYPE

[Move Table 9 of D05 here].

| Type Name | Derived From | Definition | Valid Range |
| --- | --- | --- | --- |
| OPERATOR_ID | SEQUENCE( OPERATOR_NS, OPERATOR_NAMESPACE) | A type to represent an operator identifier. | See below. |
| OPERATOR_NAMESPACE | INTEGER(1) | A type to represent a type of operator name. | 0: GSM/UMTS 1: CDMA 2: REALM (as defined in [30]). 3: ITU-T/TSB 4-255: (Reserved) |
| OPERATOR_NAME | OCTET_STRING | A type to represent a operator name. The value uniquely identifies the operator name within the scope of the OPERATOR_NS. | The value is a non NULL terminated string whose length shall not exceed 253 octets. |

| Type Name | Derived From | Definition | Valid Range |
| --- | --- | --- | --- |
| SERVICE_PROVIDER_ID | OCTET_STRING | A type to represent a service provider identifier. | A non-NULL terminated string whose length shall not exceed 253 octets. |

| Type Name | Derived From | Definition | Valid Range |
| --- | --- | --- | --- |
| ACCESS_NETWORK_ID | OCTET_STRING | A type to represent a network identifier. | A non-NULL terminated string whose length shall not exceed 253 octets. |

| Type Name | Derived From | Definition | Valid Range |
| --- | --- | --- | --- |
| NETWORK_SYSTEM_ID | OCTET_STRING | A type to represent a network system identifier. | A string value identifying the SSID established to use for association. |

| Type Name | Derived From | Definition | Valid Range |
| --- | --- | --- | --- |
| ROAMING_PARTNERS | LIST(OPERATOR_ID) | A type to represent a list of roaming partners. | N/A |

| Type Name | Derived From | Definition | Valid Range |
| --- | --- | --- | --- |
| COST | SEQUENCE( COST_UNIT, COST_VALUE, COST_CURRENCY) | A type to represent a cost. | N/A |
| COST_UNIT | ENUMERATED | A type to represent the unit of a cost. | 0: second 1: minute 2: hours 3: day 4: week 5: month 6: year 7: free 8: flat rate 9-255: (Reserved) |
| COST_VALUE | SEQUENCE( INTEGER(4), INTEGER(2)) | A type to represent the value of a cost. | The first 4-octet contains the integer part of the cost. The last 2-octet contains the fraction part where it represents a 3-digit fraction. Therefore, the value range of the fraction part is [0,999]. For example, for a value of “0.5”, the integer part is zero and the fraction part is 500. |
| COST_CURRENCY | CHAR(3) | A type to represent the currency of a cost. | A three-letter currency code (e.g., “USD”) specified by ISO 4217 [ISO 4217]. |

| Type Name | Derived From | Definition | Valid Range |
| --- | --- | --- | --- |
| NETWORK_QOS | SEQUENCE( LINK_TYPE, LIST(QOS_PARAM)) | A type to represent a list of QoS parameters of a specific link type. | N/A |

| Type Name | Derived From | Definition | Valid Range |
| --- | --- | --- | --- |
| DATA_RATE | INTEGER(4) | A type to represent the maximum data rate in Kbps. | 0 - 2^32-1 |

| Type Name | Derived From | Definition | Valid Range |
| --- | --- | --- | --- |
| NETWORK_CAPABILITIES | BITMAP(32) | A type to represents a set of network capabilities. | Bit 0: Security Bit 1: QoS Bit 2: Internet Access Bit 3: Emergency Services Bit 4: MIH Capability Bit 5-31: Reserved |

### M.2.7.2 Data Types for Information Service Query

| Type Name | Derived From | Definition | Valid Range |
| --- | --- | --- | --- |
| INFO_QUERY_BINARY_DATA | SEQUENCE( CHOICE( QUERIER_LOCATION, NULL), CHOICE( NETWORK_TYPE_INCLUSION, NULL), CHOICE( NETWORK_INCLUSION, NULL), CHOICE( REPORTING_TEMPLATE, NULL), CHOICE( REPORT_LIMITATION, NULL), CHOICE( COST_CURRENCY, NULL)) | A type to represent a binary query data. If the CURRENCY is included, the currency indicated in the CURRENCY shall be used for Cost IEs carried in an INFO_RESP_BINARY. See below for the processing rule of QUERIER_LOCATION, NETWORK_INCLUSION, NETWORK_INCLUSION, REPORTING_TEMPLATE and REPORT_LIMITATION. | N/A |

| Type Name | Derived From | Definition | Valid Range |
| --- | --- | --- | --- |
| QUERIER_LOCATION | SEQUENCE( CHOICE( LOCATION, NULL), CHOICE( MAC_ADDRESS, NULL), CHOICE( NEIGHBORHOOD_RADIUS, NULL)) | A type to represent a querier’s location. It is not valid to use both LOCATION and MAC_ADDRESS at the same time. | N/A |

| Type Name | Derived From | Definition | Valid Range |
| --- | --- | --- | --- |
| NETWORK_TYPE_INCLUSION | BITMAP(64) | A type to represent a set of link types. | The value is a 4 octet bitmap: Bit 0: Wireless - GSM Bit 1: Wireless - GPRS Bit 2: Wireless - EDGE Bit 3: IEEE 802.3 (Ethernet) Bit 4: Wireless - Other Bit 5: Wireless - IEEE 802.11 Bit 6: Wireless - CDMA2000 Bit 7: Wireless - UMTS Bit 8: Wireless - cdma2000-HRPD Bit 9: Wireless - IEEE 802.16 Bit 10: Wireless - IEEE 802.20 Bit 11: Wireless - IEEE 802.22 Bit 12-31: (Reserved AND shall not be used and shall be always set to “0”) |

NETWORK_TYPE_INCLUSION, if provided in the query, is used to indicate the neighboring network types the querier wants to include in the response. The querier indicates the network types it wants to include in the query response by setting the corresponding bits to “1”. If not provided, the Information Server shall include all available network types in the query response.

| Type Name | Derived From | Definition | Valid Range |
| --- | --- | --- | --- |
| NETWORK_INCLUSION | LIST(ACCESS_NETWORK_ID) | A type to represent a list of network identifiers. | N/A |

NETWORK_INCLUSION, if provided in the query, is used to indicate the specific access networks the querier wants to include in the query response. If not provided, the Information Server shall include all available access networks in the query response.

| Type Name | Derived From | Definition | Valid Range |
| --- | --- | --- | --- |
| REPORTING_TEMPLATE | LIST(IE_TYPE) | A type to represent a list of IE types. Inclusion of any IE type is optional. | N/A. |
| IE_TYPE | INTEGER(4) | A type to represent an IE type. | See Table C-1. |

REPORTING_TEMPLATE, if present, it indicates to the information server a template of the list of IEs which shall be included in the information response.

Rules for using REPORTING_TEMPLATE.

- If the REPORTING_TEMPLATE is absent, the entire list of neighboring networks container shall be returned in the response.

- If a container is listed without any of its component IEs, the entire container shall be returned in the response. For example, inclusin of  TYPE_IE_CONTAINER_POA solely returns a list of PoA Containers with all their component IEs.

- if a container is listed with one or more of its component IEs, the container with only the listed component IEs shall be returned. For example, inclusion of  TYPE_IE_CONTAINER_NETWORK, TYPE_IE_NETWORK_TYPE and  TYPE_IE_OPERATOR_IDENTIFIER solely returns a list of Network Containers with each containing only Network Type and Operator ID.

- If a component IE is listed without its parent container, the listed component IE shall be returned as an individual IE. For example, inclusion of TYPE_IE_NETWORK_TYPE and TYPE_IE_COST solely returns a list of Network Types and a list of Costs. Note, a list of individual IEs out of their context may have very limited usefulness. This is only an example to show the flexible use of Reporting Template.

| Type Name | Derived From | Definition | Valid Range |
| --- | --- | --- | --- |
| REPORT_LIMITATION | SEQUENCE( INTEGER(2), INTEGER(2)) | A type to represent a report limitation. The first INTEGER(2) contains the maximum number of IEs in the INFO_RESPONSE_BINARY_DATA. The second INTEGER(2) contains the starting entry number (offset = 1 points to the first entry) from which a chunk of entries are to be included in the INFO_QUERY_BINARY_DATA. It is assumed that the IS server generates the same ordered list of entries for queries from the same IS client with the same INFO_RESPONSE_BINARY_DATA content (except for REPORT_LIMITATION) before the limitation on the REPORT_LIMITATION is applied. | N/A |

| Type Name | Derived From | Definition | Valid Range |
| --- | --- | --- | --- |
| INFO_RESPONSE_BINARY_DATA | LIST( INFORMATION_ELEMENT ) | A type to represent a binary query response data. | N/A |

Rules for generating returned IEs:

Upon receipt of a binary query, the information server will:

1) create the list of neighboring access network information for the given location;

— If a NETWORK_TYPE_INCLUSION is provided in the query, include only the information of the neighboring access networks of the network type(s) indicated in the NETWORK_TYPE_INCLUSION. Otherwise, include information of all available neighboring access networks for the given location.

— If a NETWORK_INCLUSION is provided in the query, include only the information of the neighboring access network(s) indicated in the NETWORK_INCLUSION. Otherwise, include information of all available neighboring access networks for the given location.

2) if no REPORTING_TEMPLATE is given in the query, send the list of neighboring access network information in a List of Neighboring Access Networks Container in an Information Response.

3) if a REPORTING_RESPONSE is given in the query, extract the requested IE(s)/Containers from the list of neighboring access network information using the REPORTING_TEMPLATE processing rules describe above and send them in an Information Response.

| Type Name | Derived From | Definition | Valid Range |
| --- | --- | --- | --- |
| MIME_TYPE | OCTET_STRING | A type to represent a MIME type. When the MIME type is “application/ sparql-results+xml”, this field contains XML text. | N/A |

| Type Name | Derived From | Definition | Valid Range |
| --- | --- | --- | --- |
| INFO_QUERY_RDF_DATA | SEQUENCE( CHOICE(MIME_TYPE, NULL), OCTET_STRING) | A type to present a RDF data query. If MIME_TYPE is not contained, MIME type “application/ sparql-results+xml” is used. Each OCTET_STRING is formatted with the MIME type. | N/A |
| INFO_QUERY RDF_SCHEMA | OCTET_STRING | A type to represent the URL of an RDF schema to obtain. | N/A |

| Type Name | Derived From | Definition | Valid Range |
| --- | --- | --- | --- |
| INFO_RESPONSE_RDF_DATA | SEQUENCE( CHOICE(MIME_TYPE, NULL), OCTET_STRING) | A type to present a RDF data query. If MIME_TYPE is not contained, MIME type “application/ sparql-results+xml” is used, the OCTET_STRING is formatted with the MIME type. | N/A |
| INFO_RESP_SCHEMA_URL | OCTET_STRING | A type to represent an URL of an RDF schema. | N/A |
| INFO_RESP_RDF_SCHEMA | SEQUENCE{ CHOICE(MIME_TYPE,NULL), OCTET_STRING) | A type to represent the content of an RDF schema. If MIME_TYPE is not contained, MIME type “application/ sparql-results+xml” is used, the OCTET_STRING is formatted with the MIME type. | N/A |

### M.2.8 Data Type for MIHF Identification

| Type Name | Derived From | Definition | Valid Range |
| --- | --- | --- | --- |
| MIHF_ID | OCTET_STRING | A type for presenting an MIHF Identifier (see definition in 8.3.1). | N/A |

### M.2.9 Data Type for Event Subscription Identification

| Type Name | Derived From | Definition | Valid Range |
| --- | --- | --- | --- |
| SUBSCRIPTION_ID | INTEGER(2) | A type for representing the identifier for a remote event subscription. | N/A |

### M.2.10 Data Types for MIH Capabilities

| Type Name | Derived From | Definition | Valid Range |
| --- | --- | --- | --- |
| MIH_EVENT_LIST | BITMAP(32) | A type to represent a set of MIH events. | Bit 0: MIH_Link_Up Bit 1: MIH_Link_Down Bit 2: MIH_Link_Going_Down Bit 3: MIH_Link_Detected Bit 4: MIH_Link_Parameters_Report Bit 5: MIH_Link_Event_Rollback Bit 6: MIH_Link_PDU_Transmit_Status Bit 7: MIH_Link_Handover_Imminent Bit 8: MIH_Link_Handover_Complete Bit 9-31: (Reserved) |
| MIH_COMMAND_LIST | BITMAP(32) | A type to represent a set of MIH commands. | Bit 0: MIH_Get_Link_Parameters Bit 1: MIH_Configure_Link Bit 2: MIH_Scan Bit 3: MIH_Net_HO_Candidate_Query MIH_Net_HO_Commit MIH_N2N_HO_Query_Resources MIH_N2N_HO_Complete Bit 4: MIH_MN_HO_Candidate_Query MIH_MN_HO_Commit MIH_MN_HO_Complete Bit 5-31: (Reserved) |
| MIH_IS_QUERY_TYPE_LIST | BITMAP(8) | A type to represent a set of MIH IS query types. | Bit 0: TLV Bit 1: RDF_DATA Bit 2: RDF_SCHEMA_URL Bit 3: RDF_SCHEMA Bit 4-7: (Reserved) |
| MIH_TRANSPORT_LIST | BITMAP(32) | A type to represent a set of MIH transport list. | Bit 0: L2 for service management Bit 1: L3 or higher layer protocol for service management Bit 4: L2 for event service Bit 5: L3 or higher layer protocol for event service Bit 8: L2 for command service Bit 9: L3 or higher layer protocol for command service Bit 12: L2 for information service Bit 13: L3 or higher layer protocol for information service Other Bits: (Reserved) |

### M.2.11 Data Type for MIH Registration

| Type Name | Derived From | Definition | Valid Range |
| --- | --- | --- | --- |
| REG_REQUEST_CODE | ENUMERATED | A type to represent a registration request code. | 0 - Registration 1 - Re-Registration [Editor’s note: Re-registration scope to be defined.] |

### M.2.12 Data Types for Handover Operation

| Type Name | Derived From | Definition | Valid Range |
| --- | --- | --- | --- |
| HANDOVER_TYPE | BOOLEAN | A type to represent a handover type. | FALSE: Intra-technology TRUE: Inter-technology |
| HANDOVER_MODE | ENUMERATED | A type to represent a handover mode. | 0 Make-before-Break 1 Break-before-Make |
| HANDOVER_ACK | BOOLEAN | A type to represent a handover acknowledgment. | TRUE: Initiate Handover FALSE: Abort Handover |
| HANDOVE_QUERY_RESOURCE | BITMAP(8) | A type to represent a set of resource types to query. | Bit 0: Maximum Bandwidth Bit 1-7: (Reserved) |
| HANDOVER_RESULT | ENUMERATED | A type to represent a handover result. | 0: Success 1: Failure 2: Rejected |

### M.2.13 Data Types for MIH_NET SAP Primitives

| Type Name | Derived From | Definition | Valid Range |
| --- | --- | --- | --- |
| TRANSPORT_TYPE | ENUMERATED | A type to represent a transport type. | 0: L2 1: L3 or higher layer protocol |

### M.2.14 Data Types for MIH_NMS SAP Primitives

TBD.

### [Note: Undefined Data Types (Needs to be defined)]

Type Name

NETWORK_SECURITY

ERROR_CODE

NON_QOS_TYPE

NON_QOS_PARAM

RESOURVE_SET

AVAILABLE_RESOURCE_LIST

NMS_STATE_REQ

NMS_STATE_RESP

## Changes on Section 8

[1] Replace reference to TLV type values with reference to TLV type names.

For example, MIH_Capability_Discover request message format should look like:

MIH Header Fixed Fields (SID=1, Opcode=1, AID=1)

Source Identifier = sending MIHF ID (SOURCE MIHF ID TLV)

Destination Identifier = receiving MIHF ID (DESTINATION MIHF ID TLV)

SupportedMihEventList (MIH EVENT LIST TLV)

SupportedMihCommandList (MIH COMMAND LIST TLV)

SupportedISQueryTypeList (MIH IS QUERY TYPE LIST TLV)

SupportedTransportList (TRANSPORT OPTION LIST TLV)

[Note: The mapping between attribute names in MIH message format and actual TLV Type name is provided below. (the mapping is not part of proposed text) ]

| Attribute Name in Message Format | TLV Type Name |
| --- | --- |
| Source Identifier | SOURCE MIHF ID |
| Destination Identifier | DESTINATION MIHF ID |
| Subscription Identifier | SUBSCRIPTION ID |
| SupportedMihEventList | MIH EVENT LIST |
| SupportedMihCommandList | MIH COMMAND LIST |
| SupportedISQueryTypeList | MIH IS QUERY TYPE LIST |
| SupportedTransportList | TRANSPORT OPTION LIST |
| RequestCode | REQUEST CODE |
| ValidityPeriod | VALIDITY PERIOD |
| Status | STATUS |
| LinkIdentifier | LINK ID |
| MacOldAccessRouter | MAC OLD ACCESS ROUTER |
| MacNewAccessRouter | MAC NEW ACCESS ROUTER |
| IPRenewalFlag | IP RENEWAL FLAG |
| MobilityManagementSupport | MOBILITY MGMT SUPPORT |
| IPConfigurationMethods | IP ADDRESS CONFIG METHOD |
| LinkDownReasonCode | LINK DOWN REASON CODE |
| TimeInterval | TIME INTERVAL |
| ConfidenceLevel | CONFIDENCE LEVEL |
| LinkGoingDownReason | LINK GOING DOWN REASON |
| UniqueEventIdentifier | UNIQUE EVENT IDENTIFIER |
| MIHCapabilityFlag | MIH CAPABILITY FLAG |
| LinkParameterReport | LINK PARAMETER REPORT |
| HandoverType | HANDOVER TYPE |
| OldLinkIdentifier | LINK IDENTIFIER |
| NewLinkIdentifier | NEW LINK IDENTIFIER |
| TargetLinkIdentifier | LINK IDENTIFIER |
| GetStatusRequestSet | GET STATUS REQUEST SET |
| GetStatusResponseSet | GET_STATUS_RESPONSE_SET |
| ConfigureRequestSet | CONFIGURE REQUEST SET |
| ConfigurationResponseList | CONFIGURE_RESPONSE SET |
| ScanLinkIdentifier | SCAN LINK IDENTIFIER |
| ScanResponseSets | SCAN RESPONSE SET |
| SuggestedNewLink 1..k | LIST OF LINK POA LIST (Note: bundled into a single TLV) |
| HandoverMode | HANDOVER MODE |
| OldLinkActions | LINK ACTIONS |
| QueryResourceList | QUERY RESOURCE LIST |
| RequestedResourceSet | REQUESTED RESOURCE SET |
| CurrentLinkIdentifier | LINK IDENTIFIER |
| PreferredLink 1..k | LIST OF LINK POA LIST (Note: bundled into a single TLV) |
| PreferredCandidateLink 1..k | LIST OF LINK POA LIST (Note: bundled into a single TLV) |
| HandoverAck | HANDOVER ACK |
| ServingFAAdress | FA ADDRESS |
| ServingAccessRouterAddress | AR ADDRESS |
| DHCPServerAddress | DHCP SERVER ADDRESS |
| IPAddressInformationStatus | IP ADDRESS INFO STATUS |
| ResourceStatus | RESOURCE STATUS |
| AvailableResourceSet | AVAILABLE RESOURCE SET |
| LinkActionSet 1 .. k | LINK ACTION SET LIST (Note: bundled into a single TLV) |
| LinkActionResult 1…k | LINK ACTION RESULT LIST (Note: bundled into a single TLV) |
| HandoverResult | HANDOVER RESULT |
| ResourceRetainStatus | RESOURCE RETENTION STATUS |
| InfoQueryBinary (0 to k) | INFO QUERY BINARY LIST (Note: bundled into a single TLV) |
| InfoQueryRDFData (0 to k) | INFO QUERY RDF DATA LIST (Note: bundled into a single TLV) |
| InfoQueryRDFSchemaURL (0 to k) | INFO QUERY RDF SCHEMA URL (Note: bundled into a single TLV) |
| InfoQueryRDFSchema (0 to k) | INFO QUERY RDF SCHEMA LIST (Note: bundled into a single TLV) |
| MaxResponseSize | MAX RESPONSE SIZE |
| InfoResponseBinary (0 to k) | INFO RESP BINARY LIST (Note: bundled into a single TLV) |
| InfoResponseRDFData (0 to k) | INFO RESP RDF DATA LIST (Note: bundled into a single TLV) |
| InfoResponseRDFSchemaURL (0 to k) | INFO RESP RDF SCHEMA URL LIST (Note: bundled into a single TLV) |
| InfoResponseRDFSchema (0 to k) | INFO RESP RDF SCHEMA LIST (Note: bundled into a single TLV) |

[Note: TargetPoA is removed from the parameter since TargetLinkIdentifier can contain PoA MAC address.]

[2] Remove the following sections:

- 8.5.1 through 8.5.44

- 8.6.4.1.1

- 8.6.4.1.2

- 8.6.4.1.3

- 8.6.4.1.4

- 8.6.4.2.1

- 8.6.4.2.2

- 8.6.4.2.3

- 8.6.4.2.4

[3] Add STATUS TLV to MIH_Get_Information response message.

## A new Annex for TLV Table

Add a new Annex N as follows.

### Annex N. List of TLVs

| TLV Type Name | TLV Type Value | TLV Value Data Type | Defined Annex |
| --- | --- | --- | --- |
| SOURCE MIHF ID | 0 | MIHF_ID | M.2.8 |
| DESINTATION MIHF ID | 1 | MIHF_ID | M.2.8 |
| SUBSCRIPTION ID | 2 | SUBSCRIPTION_ID | M.2.9 |
| STATUS | 3 | ENUMERATED | M.2.1 |
| MIH EVENT LIST | 4 | MIH_EVENT_LIST | M.2.10 |
| MIH COMMAND LIST | 5 | MIH_COMMAND_LIST | M.2.10 |
| MIH IS QUERY TYPE LIST | 6 | MIH_IS_QUERY_TYPE_LIST | M.2.10 |
| TRANSPORT OPTION LIST | 7 | MIH_TRANSPORT_LIST | M.2.10 |
| REQUEST CODE | 8 | REG_REQUEST_CODE | M.2.11 |
| VALIDITY PERIOD | 9 | INTEGER(4) | M.2.1 |
| LINK IDENTIFIER | 10 | LINK_TUPLE_ID | M.2.3 |
| NEW LINK IDENTIFIER | 11 | LINK_TUPLE_ID | M.2.3 |
| MAC OLD ACCESS ROUTER | 12 | MAC_ADDRESS | M.2.5 |
| MAC NEW ACCESS ROUTER | 13 | MAC_ADDRESS | M.2.5 |
| IP RENEWAL FLAG | 14 | IP_RENEWAL_FLAG | M.2.6 |
| MOBILITY MGMT SUPPORT | 15 | IP_MOBILITY_MGMT | M.2.6 |
| IP ADDRESS CONFIG METHOD | 16 | IP_CONFIG_METHODS | M.2.6 |
| LINK DOWN REASON CODE | 17 | LINK_DOWN_REASON | M.2.3 |
| TIME INTERVAL | 18 | INTEGER(2) | M.2.1 |
| CONFIDENCE LEVEL | 19 | PERCENTAGE | M.2.1 |
| LINK GOING DOWN REASON | 20 | LINK_GOING_DOWN_REASON | M.2.3 |
| UNIQUE EVENT IDENTIFIER | 21 | INTEGER(2) | M.2.1 |
| MIH CAPABILITY FLAG | 22 | LINK_MIH_CAPABILITY_FLAG | M.2.3 |
| LINK PARAMETER REPORT | 23 | LIST(LINK_PARAM) | M.2.3 |
| HANDOVER TYPE | 24 | HANDOVER_TYPE | M.2.12 |
| GET STATUS REQUEST SET | 25 | LINK_STATUS_REQ | M.2.3 |
| GET_STATUS_RESPONSE_SET | 26 | LIST(LINK_STATUS_RESP) | M.2.3 |
| CONFIGURE REQUEST SET | 27 | LIST(LINK_CONFIG_REQ) | M.2.3 |
| CONFIGURE_RESPONSE SET | 28 | LIST(LINK_CONFIG_STATUS) | M.2.3 |
| SCAN LINK IDENTIFIER | 29 | LINK_ID | M.2.3 |
| SCAN RESPONSE SET | 30 | LIST(LINK_SCAN_RESP) | M.2.3 |
| LIST OF LINK POA LIST | 31 | LIST(LINK_POA_LIST) | M.2.3 |
| HANDOVER MODE | 32 | HANDOVER_MODE | M.2.12 |
| LINK ACTIONS | 33 | LINK_ACTIONS | M.2.3 |
| REQUESTED RESOURCE SET | 34 | LINK_RESOURCE_SET | M.2.3 |
| QUERY RESOURCE LIST | 35 | QUERY_RESOURCE_LIST | M.2.3 |
| HANDOVER ACK | 36 | HANDOVER_ACK | M.2.12 |
| AR ADDRESS | 37 | IP_ADDRESS | M.2.6 |
| DHCP SERVER ADDRESS | 38 | IP_ADDRESS | M.2.6 |
| FA ADDRESS | 39 | IP_ADDRESS | M.2.6 |
| IP ADDRESS INFO STATUS | 40 | IP_CONFIG_STATUS | M.2.6 |
| LINK ACTION SET | 41 | LIST(LINK_ACTION_REQ) | M.2.3 |
| LINK ACTION RESULT | 42 | LIST(LINK_ACTION_RESP) | M.2.3 |
| HANDOVER RESULT | 43 | HANDOVER_RESULT | M.2.12 |
| RESOURCE STATUS | 44 | LINK_RESOURCE_STATUS | M.2.3 |
| AVAILABLE RESOURCE SET | 45 | LIST(LINK_RESOURCE_SET) | M.2.3 |
| RESOURCE RETENTION STATUS | 46 | LINK_RESOURCE_RETENTION_STATUS | M.2.3 |
| INFO QUERY BINARY LIST | 47 | LIST(INFO_RESP_BINARY_DATA) | M.2.7.2 |
| INFO QUERY RDF DATA LIST | 48 | LIST(INFO_QUERY_RDF_DATA) | M.2.7.2 |
| INFO QUERY RDF SCHEMA URL | 49 | NULL | M.2.7.2 |
| INFO_QUERY_RDF SCHEMA LIST | 50 | LIST(INFO_QUERY_RDF_SCHEMA) | M.2.7.2 |
| MAX RESPONSE SIZE | 51 | INTEGER(2) | M.2.1 |
| INFO RESP BINARY LIST | 52 | LIST(INFO_RESP_BINARY_DATA) | M.2.7.1 |
| INFO RESP RDF DATA LIST | 53 | LIST(INFO_RESP_RDF_DATA) | M.2.7.2 |
| INFO RESP RDF SCHEMA URL LIST | 54 | LIST(INFO_RESP_SCHEMA_URL) | M.2.7.2 |
| INFO RESP RDF SCHEMA LIST | 55 | LIST(INFO_RESP_RDF_SCHEMA) | M.2.7.2 |