| Project | IEEE 802.21 MIHO <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | Transport Address |  |
| Date Submitted | April 27, 2007 |  |
| Source(s) | Yoshihiro Ohba |  |
| Re: | IEEE 802.21 Session #20 in May 2007 |  |
| Abstract | This document proposes to use a single transport address format for MAC address and IP address throughout the specification. |  |
| Purpose | LB#1d comment resolution |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.21. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

# Problem

MAC or IP address encodings are not consistent in the draft:

Type 1: Encoded as Address type of RFC 3588.

Type 2: Encoded as 4-octet (IPv4) or 16-octet (IPv6) value.

| Page | Section | IE or TLV | Encoding Type |
| --- | --- | --- | --- |
| 60 | 6.4.6.3.1 | PoA MAC Address | Type 1 |
| 62 | 6.4.6.3.5 | PoA subnet information (Address part) | Type 1 |
| 63 | 6.4.6.3.6 | DHCP server address, FA/AR address | Type 2 |
| 64 | 6.4.6.3.8 | PoA IP address | Type 2 |
| 195 | 8.5.8 | MAC Address | Type 1 |
| 201 | 8.5.31 | IP address | Type 2 |

On the other hand, Table 22 defines TRANSPORT ADDRESS which is defined as a protocol layer specific transport address including MAC address and IP address.

It is really better to define MAC address and IP address as a special type of transport address.

This problem is part of general data type problem discussed in

# Proposed Text

[1] Change Address field of Table 18 as follows:

| Address | Variable | An IP address of the PoA encoded as Address base type defined in RFC3588. The first 2-octet contains AddressType, which may be is either 1 (IPv4) or 2 (IPv6). If AddressType is 1, the subsequent octets contain a 4-octet IPv4 address. If AddressType is 2, the subsequent octets contain a 16-octet IPv6 address. |
| --- | --- | --- |

[2] Change Table 20 as follows:

| Type=TYPE_IE_POA_DHCP_SERVER_ADDRESS | Length = 4 ot 16 octets Variable |
| --- | --- |
| Provides an optional DHCP server address when “IPv4 dynamic configuration (DHCPv4)”, “Mobile IPv4 without FA (Co-located CoA)” or “IPv6 stateful address configuration (DHCPv6)” is supported. (4-octet for IPv4 or 16-octet for IPv6) The address is encoded as Address base type defined in RFC3588. The first 2-octet contains AddressType, which is either 1 (IPv4) or 2 (IPv6). If AddressType is 1, the subsequent octets contain a 4-octet IPv4 address. If AddressType is 2, the subsequent octets contain a 16-octet IPv6 address. |  |

[3] Change Table 21 as follows:

| Type=TYPE_IE_POA_FA_AR_ADDRESS | Length = 4 ot 16 octets Variable |
| --- | --- |
| Provides an optional foreign agent or access router address, when “Mobile IPv4 with FA (FA-CoA)” or “IPv6 stateless address configuration” is supported. (4-octet for IPv4 or 16-octet for IPv6) The address is encoded as Address base type defined in RFC3588. The first 2-octet contains AddressType, which is either 1 (IPv4) or 2 (IPv6). If AddressType is 1, the subsequent octets contain a 4-octet IPv4 address. If AddressType is 2, the subsequent octets contain a 16-octet IPv6 address. |  |

[4] Change Section 6.4.6.3.8 as follows:

| Type=TYPE_IE_POA_IP_ADDRESS | Length = 4 ot 16 octets Variable |
| --- | --- |
| IP Address of PoA. (4-octet for IPv4 or 16-octet for IPv6) The address is encoded as Address base type defined in RFC3588. The first 2-octet contains AddressType, which is either 1 (IPv4) or 2 (IPv6). If AddressType is 1, the subsequent octets contain a 4-octet IPv4 address. If AddressType is 2, the subsequent octets contain a 16-octet IPv6 address. |  |

[5] Change 8.5.31 as follows:

| Defined in messages | 4 or 16 Variable | An IP address encoded as Address base type defined in RFC3588. The first 2-octet contains AddressType, which is either 1 (IPv4) or 2 (IPv6). If AddressType is 1, the subsequent octets contain a 4-octet IPv4 address. If AddressType is 2, the subsequent octets contain a 16-octet IPv6 address. |
| --- | --- | --- |