| Project | IEEE 802.21 MIHS <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | MIH_Configure_Link |  |
| DCN | 21-07-0145-01-0000 |  |
| Date Submitted | May 16th, 2007 |  |
| Source(s) | Kenichi Taniuchi |  |
| Re: | IEEE 802.21 Session #20 in Montreal |  |
| Abstract | Suggested Proposal related to Configuration of Link Threshold |  |
| Purpose | LB#1d comment resolution |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that IEEE 802.2 may make this contribution public. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

# in Section 7.3.3.1.2 Link_Configure_Thresholds.request (#5172)

## Proposal

No change is required. This comment is merged with the other comments.

# in Section 7.3.3.2.2 Link_Configure_Thresholds.confirm (#5177)

## Proposal

No change is required. This comment is merged with the other comments.

# In Section 7.3.9.2 Link_Parameters_Report.indicaiton (#5173)

## Proposal

No change is required. This comments is merged with the other comments.

# In Section 7.4.16.1 MIH_Configure_Link.request  (#5274)

## Proposal

[This change effects Section 8, Section 6, consistency is needed.]

Split MIH_Configure_Link.request to MIH_Link_Configure_Thresholds.request and MIH_Link_Action.request

Page 121 Line 45

7.4.16.1 MIH_Link_Configure_Thresholds.request

7.4.16.1.1 Function

This primitive is issued by an MIH User to configure the threshold of a lower layer link.

7.4.16.1.2 Semantics of the service primitive

The parameters of the primitive are as follows:

MIH_Link_Configure_Thresholds.request(

DestinationIdentifier,

LinkIdentifier,

Link Configure Parameters List

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Destination Identifier | MIHF ID | N/A | This identifies the local MHIF or a remote MIHF which will be the destination of this request. |
| Link Identifier | LINK ID | N/A | Identifier of the link to be configured. |
| Link Configure Parameters List | LIST | N/A | A list of link parameters and their corresponding thresholds as defined in 7.3.3.1.2. |

7.4.16.1.3 When generated

This primitive is invoked by an MIH User when it attempts to configure the threshold of a local or remote lower layer link.

7.4.16.1.4 Effect of receipt

If the destination of the request is the local MIHF itself, the local MIHF shall issue a Link_Configure_Thresholds request to the lower layer link to set the thresholds for the link according to the specified configuration parameters.

If the destination of the request is a remote MIHF, the local MIHF shall generate and send an MIH_Link_Configure_Thresholds request message to the remote MIHF. Upon the receipt of the message, the remote MIHF shall then issue a Link_Configure_Thresholds request to the lower layer link to set the thresholds for the link according to the specified configuration parameters.

7.4.X MIH_Link_Action

7.4.X.1 MIH_Link_Action.request

7.4.X.1.1 Function

This primitive is used by an MIH User to control the behavior of a lower layer link.

7.4.X.1.2 Semantics of Service Primitive

The parameters of the service primitive are as follows:

MIH_Link_Action.request (

Destination Identifier,

LinkIdentifier,

LinkAction,

ExecutionTime

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Destination Identifier | MIHF ID | N/A | This identifies the local MIHF or a remote MIHF which will be destination of this request. |
| Link Identifier | LINK ID | N/A | Identifier of present link |
| Link Actions | BITMAP(32) | N/A | Specifies the suggested action on link during handover. Combination of the following choices is allowed. Bit 0: LINK_DISCONNECT Bit 1: LINK_LOW_POWER Bit 2: LINK_POWER_DOWN Bit 3: LINK_NO_ACTION Bit 4: LINK_RESOURCE_RETAIN Bit 5: DATA_FORWARDING_REQUEST Bit 6: LINK_POWER_UP Bit 7-31: (Reserved) |
| Execution Time | INTEGER | 0-10000 | Time (in ms) elapsed before action needs to be taken. If Execution Time = 0 then the action is expected to be taken on receipt of this command. |

7.4.X.1.3 When generated

This primitive is invoked by an MIH_User when it attempts to control the behavior of a local or remote lower layer link.

7.4.X.1.4 Effect on receipt

If the destination of the request is the local MIHF itself, the local MIHF shall issue a Link_Action request to the lower layer link to control the link.

If the destination of the request is a remote MIHF, the local MIHF shall generate and send an MIH_Action_Link request message to the remote MIHF. Upon the receipt of the message, the remote MIHF shall then issue a Link_Action request to the lower layer link to control the link.

# In Section 7.4.16.1 MIH_Configure_Link.confirm (#5282)

## Proposal

[This change effects Section 8, Section 6, consistency is needed.]

Split MIH_Configure_Link.confirm to

MIH_Link_Configure_Thresholds.confirm and MIH_Link_Action.confirm

7.4.16.2 MIH_Link_Configure_Thresholds.confirm

This primitive is issued by an MIHF to report the result of an MIH_Link_Configure_Thresholds.request.

MIH_Link_Configure_Thresholds.confirm (

SourceIdentifier,

LinkIdentifier,

LinkConfigureParametersStatusList,

Status

)

7.4.16.2.3 When generated

This primitive returns the result of an MIH_Link_Configure_Thresholds.request to the requesting MIH User.

7.4.X.2 MIH_Link_Action.confirm

7.4.X.2.1 Funcion

This primitive is issued by an MIHF to report the result of an MIH_Link_Action request.

7.4.X.2.2 Semantics of the service primitive

The parameters of the primitive are as follows:

MIH_Link_Action.confirm (

SourceIdentifier,

LinkIdentifier,

Scan Response Sets

Status

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Source Identifier | MIHF ID | N/A | This identifies the invoker of this primitive which can be either the local MIHF or a remote MIHF |
| Link Identifier | LINK ID | N/A | Identifier of the link configured. |
| Scan Response Sets | LIST | N/A | (Optional) A list of discovered links and related information. See table 45 for detailed definition. |
| Status | ENUMERATED | 0-4 | Result of the actions specified in the MIH_Link_Action.request primitive. 0 Success 1 Unspecified Failure 2 Rejected 3 Resource Unavailable 4 Resource Busy |

7.4.X.2.3 When generated

This primitive returns the result of an MIH_Link_Action.request to the requesting MIH User.

7.4.X.2.4 Effect on receipt

Upon receipt of the result, the MIH User makes appropriate evaluations and takes any suitable actions. However, if Status does not indicate “Success”, the recipient shall ignore any other returned values and, instead, shall perform appropriate error handling.