| Project | IEEE 802.21 MIHO <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | Table 22 |  |
| Date Submitted | April 27, 2007 |  |
| Source(s) | Yoshihiro Ohba |  |
| Re: | IEEE 802.21 Session #20 in May 2007 |  |
| Abstract | This document proposes Table 22 update. |  |
| Purpose | LB#1d comment resolution |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.21. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

# Issues on Table 22

| Issue # | Type | Note |
| --- | --- | --- |
| 1 | BITMAP(size) | “Usually used to represent a list IDs.” Since other data types do not have such example usages, the text should be removed for consistency. |
| 2 | ENUMERATE | - It is incorrect to say that Enumeration of “a list” of allowable values. - Only integer values must be allowed. - It is better to define LINK TYPE as a derived type of ENUMERATE. - It is better to define BOOLEAN as a derived type of ENUMERATE |
| 3 | LIST | The type of data contained in the list needs to be specified, otherwise, the LIST type definition is incomplete. |
| 4 | CONNECTION ID | It is better to define it as a derived type of LINK ID. Also better to use a different name as “connection” can mean a higher-layer protocol connection. |
| 5 | LINK ID | - Optional MAC address of PoA should be included in the definition. - Also, it is better to define two derived types from LINK ID, one is with PoA MAC address (FULL LINK ID) and the other is without PoA MAC address (PARTIAL LINK ID). - For 3GPP2, a PPP link would be a link. What is MAC addresses of MN and PoA for 3GPP2 link? It should be identified as an IP link. In such a case, use of “MAC Address” may not be appropriate. It should use TRANSPORT ADDRESS instead. |
| 6 | Other | To maintain consistency throughout the document, - It is better to define a type for LINK TYPE. - It is better to define INTEGER type and define ENUMERATE and LINK TYPE as derived types of INTEGER. |

# Proposal on Table 22

| Type Name | Definition |
| --- | --- |
| BITMAP(size) | A bitmap of the specified size in number of bits. Usually used to represent a list of IDs. |
| INTEGER(size) | An integer of the specifized size in number of octets. |
| ENUMERATED | Enumeration of a list of allowable INTEGER(1) values. |
| BOOLEAN | A type derived from ENUMERATED where allowable values are 0 (False) and 1 (True). |
| LINK TYPE | A type derived from INTEGER(4) where values are defined in Table 9. |
| LIST(DATATYPE) | A list of values of type DATATYPE. |
| TRANSPORT ADDRESS | A protocol layer specific transport address. It consists of an address type and an address value. |
| MAC ADDRESS | A type derived from TRANPORT ADDRESS where the address type contains the one used for a specific link layer. |
| IP ADDRESS | A type derived from TRANPORT ADDRESS where the address type contains the one used for IPv4 or IPv6. |
| LINK ID | An identifier of a link as defined by IEEE 802.21. Consisting of: 1) a LINK TYPE value, 2) a TRANSPORT ADDRESS of MN and optionally 3) a TRANPORT ADDRESS of PoA. |
| PARTIAL LINK ID | A type derived from LINK ID in which a TRANSPORT ADDRSS of PoA is always omitted. |
| FULL LINK ID | A type derived from LINK ID in which a TRANSPORT ADDRESS of PoA is always contained. |
| MIHF ID | An MIHF Identifier (see definition in 8.3.1). |