| Project | IEEE 802.21 MIHO <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | Table 22 |  |
| Date Submitted | April 27, 2007 |  |
| Source(s) | Yoshihiro Ohba |  |
| Re: | IEEE 802.21 Session #20 in May 2007 |  |
| Abstract | This document proposes Table 22 update. |  |
| Purpose | LB#1d comment resolution |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.21. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

# Issue on MIH_Get_Information primitives and messages

Issue: Parameters in MIH_Get_Information primitives and in MIH_Get_Information messages do not match, e.g.,

  - MIH_Get_Information.request primitive has (InfoQueryType, InfoQueryParameters,MaxResponseSize)

  - MIH_Get_Information request message has (InfoQueryRDFData*, InfoQueryRDFSchemaURL*, InfoQueryRDFSchema*, MaxResponseSize)

# Proposal

[1] Change Section 7.4.25.1.2 as follows:

The parameters of the primitive are as follows:

MIH_Get_Information.request (

DestinationIdentifier,

InfoQueryBinaryDataList,

InfoQueryRDFDataList,

InfoQueryRDFSchemaURLList,

InfoQueryRDFSchemaList,

MaxResponseSize

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| DestinationIdentifier | MIHF ID | N/A | The identifier of a remote MIHF that is the destination of MIH_Get_Information command. |
| InfoQueryBinaryList (optional) | TBD | N/A | See Section 8.6.4.1.1 |
| InfoQueryRDFDataList (optional) | TBD | N/A | See Section 8.6.4.1.2. |
| InfoQueryRDFSchemaURLList (optional) | TBD | N/A | See Section 8.6.4.1.3. |
| InfoQueryRDFSchemaList (optional) | TBD | N/A | See Section 8.6.4.1.4. |
| MaxResponseSize | INTEGER | 1-65,535 | See Section 8.5.38. |

(Remove other text after the table.)

[2] Change Section 7.4.25.2.2 as follows:

The parameters of the primitive are as follows:

MIH_Get_Information.indication (

SourceIdentifier,

InfoQueryBinaryDataList,

InfoQueryRDFDataList,

InfoQueryRDFSchemaURLList,

InfoQueryRDFSchemaList,

MaxResponseSize

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| SourceIdentifier | MIHF ID | N/A | The identifier of a remote MIHF that is the source of MIH_Get_Information command. |
| InfoQueryBinary (0 to k) | TBD | N/A | See Section 8.6.4.1.1 |
| InfoQueryRDFData (0 to k) | TBD | N/A | See Section 8.6.4.1.2. |
| InfoQueryRDFSchemaURL (0 to k) | TBD | N/A | See Section 8.6.4.1.3. |
| InfoQueryRDFSchema(0 to k) | TBD | N/A | See Section 8.6.4.1.4. |
| MaxResponseSize | INTEGER | 1-65,535 | See Section 8.5.38. |

(Remove other text after the table.)

[3] Change Section 7.4.25.3.2 as follows:

The parameters of the primitive are as follows:

MIH_Get_Information.response (

DestinationIdentifier,

InfoResponseBinaryDataList,

InfoResponseRDFDataList,

InfoResponseRDFSchemaURLList,

InfoResponseRDFSchemaList

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| DestinationIdentifier | MIHF ID | N/A | The identifier of a remote MIHF that is the source of MIH_Get_Information command. |
| InfoResponseBinary (0 to k) | TBD | N/A | See Section 8.6.4.2.1 |
| InfoResponseRDFData (0 to k l) | TBD | N/A | See Section 8.6.4.2.2. |
| InfoResponseRDFSchemaURL (0 to k) | TBD | N/A | See Section 8.6.4.2.3. |
| InfoResponseRDFSchema (0 to k) | TBD | N/A | See Section 8.6.4.2.4. |
| Satus | ENUMERATED | 0-4 | [Same as D04] |

(Remove other text after the table.)

[4] Change Section 7.4.25.4.2 as follows:

The parameters of the primitive are as follows:

MIH_Get_Information.confirm (

SourceIdentifier,

InfoResponseBinaryDataList,

InfoResponseRDFDataList,

InfoResponseRDFSchemaURLList,

InfoResponseRDFSchemaList

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| SourceIdentifier | MIHF ID | N/A | The identifier of a remote MIHF that is the destination of MIH_Get_Information command. |
| InfoResponseBinary (0 to k) | TBD | N/A | See Section 8.6.4.2.1 |
| InfoResponseRDFData (0 to k) | TBD | N/A | See Section 8.6.4.2.2. |
| InfoResponseRDFSchemaURL (0 to k) | TBD | N/A | See Section 8.6.4.2.3. |
| InfoResponseRDFSchema (0 to k) | TBD | N/A | See Section 8.6.4.2.4. |
| Satus | ENUMERATED | 0-4 | [Same as D04] |

(Remove other text after the table.)