| Project | IEEE 802.21 Media Independent Handover Services <http://www.ieee802.org/21/> |  |  |
| --- | --- | --- | --- |
| Title | MIH command service flow to support network selection |  |  |
| Date Submitted | May 14, 2007 |  |  |
| Source(s) | Junghoon Jee, Eunah Kim ETRI 161 Gajeong-Dong, Yuseong-Gu Daejeon, 305-700, KOREA | Voice: +82-42-860-5126 Fax: +82-42-861-5404 {jhjee, eakim}@etri.re.kr |  |
| Re: | IEEE 802.21 Session #20 in Montreal |  |  |
| Abstract | There exist two different forms of command service flow. One is the command service flow to control the lower layers specified in the Section 6.3.4. The other is the command service flow to support network selection. However, the latter is missing from the current P802.21/D05.00. We suggest addition of this command service flow showing how the MIH can help for network selection by utilizing the defined primitives and command messages. |  |  |
| Purpose | Adoption |  |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.21. |  |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |  |

1. Problem Statement

There exist two different forms of command service flow. One is the command service flow to control the lower layers specified in the Section 6.3.4. The other is the command service flow to support network selection. However, the latter is missing from the current P802.21/D05.00. We suggest addition of this command service flow showing how the MIH can help for network selection by utilizing the defined primitives and command messages.

2. Suggested Remedy

We suggest making an amendment for Section 6.3.4 and adding the proposed part as Section 6.3.5.

    - Command service flow model to control lower layers

Figure 19 – Command service flow to control lower layers

Figure 19 shows the flow for a local command and an example of a remote command to control lower layers, respectively. Example handover procedures using the commands defined in this subclause can be found in Annex A. Remote commands may be transported over network layer protocols or link layer protocols.

6.3.5     Command service flow model to support network selection

The 802.21 standard defines media independent commands to support network selection as described in the Section 5.3.2.4.1. The overall command service flow to support network selection is depicted in the Figure 20.

The mobile-initiated and network-initiated command flows are depicted in the Figure 21 and Figure 22. Detailed flow diagrams and description are specified in Annex A.1 and A.2.

Figure 20 – Command service flow to support network selection

Figure 21 – Mobile-initiated command flow to support network selection

Figure 22 – Network-initiated command flow to support network selection