| Project | IEEE 802.21 Media Independent Handover Services <http://www.ieee802.org/21/> |  |  |
| --- | --- | --- | --- |
| Title | MIHF Discovery and MIH Capability Discovery (21-07-0156-00-0000-MIHF_discovery_and_Capability_discovery) |  |  |
| Date Submitted | May, 2007 |  |  |
| Source(s) | Subir Das, Telcordia Technologies Yoshihiro Ohba, Toshiba America Research |  |  |
| Re: | IEEE 802.21 Session #20 in Montreal, Canada |  |  |
| Abstract | This contribution proposes MIHF Function Discovery primitives |  |  |
| Purpose | Adoption |  |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.21. |  |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |  |

- Background

It is proposed that “when MN uses MIH discovery using MIH Capability Discover message, MIHF discovery and capability discovery may be performed at the same time” (section 6.1 line 27, Section 6.1.3 , Line 13, and section 8.6.1.1) in Draft D05.

This is not clear. First of all, what is the meaning of MIHF discovery? Is it discovering the peer MIHF transport address or discovering the MIHF ID of a peer MIH or both?

If it is peer transport address then how MIHF at the source will know the destination MIHF ID? For example, in MIH_Capbility_discover request message, destination MIHF is mentioned but  how it will obtain the destination MIHF ID is not clear.  There are two ways it can obtain the MIHF ID: i) pre-configuration and ii) by using a separate MIHF discovery mechanism e.g., defining new primitives as described below:

- Suggested Remedy

i) Separate MIHF discovery and  capability discovery

ii) Define the MIH function discovery primitive and corresponding protocol messages.

iii)  If MIHF ID is preconfigured there is no need for MIHF discovery. It can simply use the Capability discovery for finding out the peer’s transport address at L2.

7.4.x  MIH_Function_Discover

7.4.x.x  MIH_Function_Discover.request

7.4.x.x.x  Function

This primitive is used by MIH Users to discover the MIH function in the network.

7.4.x.x.x  Semantics of Service primitive

MIH_Function_Discover.request (

Source Identifier

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Source Identifier | Identifier | Any valid individual or group identifier | The identifier of entity where the request is initiated. |

7.4.x.x.x When genrated

This primitive is generated by MIH Users that need to know the MIHF ID in the network

7.4.x.x.x Effect on receipt

MIHF capable entities may respond with MIH Function  Discover. Confirm primitive.

7.4.x.x MIH_Function_Discover.indication

7.4.x.x.x  Function

This primitive is used by MIHF to notify the MIH Users

7.6.x.x.x  Semantics of Service primitive

MIH_Capability_Discover.Indication (

Source Identifier

Destination Identifier

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Source Identifier | Identifier | Any valid individual or group identifier | The identifier of entity where the request is initiated. |
| Destination Identifier | Identifier | Any valid individual or group identifier | The identifier of entity where the request is initiated. |

7.4.x.x.x When genrated

This primitive is used by MIHF to notify the MIH Users when a MIH function Discover.request is received

7.4.x.x.x Effect on receipt

MIHF users capable entities may respond with MIH Function  Discover. Response primitive.

7.4.x.x. MIH_Function_Discover.Response

This primitive is used by MIH Users to convey the MIHF ID of MIH functionality.

7.6.x.x.x Semantics of Service primitive

MIH_Capability_Discover.Response(

Source Identifier,

Destination Identifier,

MIHF Identifier

Status

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Source Identifier | Identifier | Any valid individual or group identifier | The identifier of entity where the request is initiated. This field may be optionally left empty if the command is local. |
| Destination Identifier | Identifier | Valid MIHF identifier | The destination identifier of request or response. This is the identifier of local or peer MIHF. |
| MIHF Identifier | Identifier | Valid MIHF ID | MIHF ID as defined in section 8.3.1 |
| Status | Enumerated | Success Error | Status of operation |

7.4.x.x.x When generated

This primitive is generated when MIH Funtion Discevery..indication primitive is received by MIHF

7.6.x.x.x Effect on receipt

Upon receiving this primitive, the MIHF  generates the corresponding response. However, if the Status indicates an error, the recipient shall ignore any other returned values and , instead, shall perform appropriate error handling..

7.4.x.x. MIH_Function_Discover.Confirm

This primitive is used by MIHF to inform the  MIH Users  about the presence MIH function

7.4.x.x.x Semantics of Service primitive

MIH_Function_Discover.Confirm(

Source Identifier,

MIHF ID

Status

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Source Identifier | Identifier | Any valid individual or group identifier | The identifier of entity where the request is initiated. This field may be optionally left empty if the command is local. |
| MIHF Identifier | MIHF ID | Valid MIHF Identifier | Identifier of the network whose MIH function is discovered as defined in section 8.3.1 |
| Status | Enumerated | Success, Error | Status of operation |

7.4.x.x.x When generated

This primitive is generated when MIH Function Discover.response primitive is received by MIHF

7.4.x.x.x Effect on receipt

Upon receiving  this primitive, the receiving entity becomes aware of MIH function.  However, if the Status indicates an error, the recipient shall ignore any other returned values and , instead, shall perform appropriate error handling..

8.6.1.x MIH_Function_Discover request

8.6.1.x  MIH_Function_Discover response