| Project | IEEE 802.21 Media Independent Handover Services <http://www.ieee802.org/21/> |  |  |
| --- | --- | --- | --- |
| Title | Session ID (21-07-0157-00-0000-Session ID) |  |  |
| Date Submitted | May, 2007 |  |  |
| Source(s) | Subir Das, Telcordia Technologies Yoshihiro Ohba, Toshiba America Research |  |  |
| Re: | IEEE 802.21 Session #20 in Montreal, Canada |  |  |
| Abstract | This contribution suggests to replace Session ID with the Subscription ID for Event Subscription |  |  |
| Purpose | Adoption |  |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.21. |  |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |  |

- Background

Session ID is defined as one of  MIH protocol identifiers but its usage is not clear. None of the messages currently use the Session ID. The definition of Session ID seems to imply that it is used for event service only. However, this is  also not clearly specified in D05.

- Suggested Remedy

i) Do not use Session ID for events. Rename it as Subscription ID and assign it when event subscription happens.

ii) Revise Section 6.1.4 (p37, l47), Figure 13 and Section 6.3.5 (p42,l16) to replace “Session ID” with “Subscription ID”.

iii) Add Subscription ID in MIH messages in Section 8.6.1.x …..

Replace the current 8.3.2 Section with the following:

8.3.2. Subscription ID

Subscription Identifier (Subscription ID) is assigned during a  remote event subscription. Each remote subscription needs to be associated with a Subscription ID that keeps the event subscription information mapping for particular event  service.

| Type | Length | Value |
| --- | --- | --- |
| TYPE_HDR_TLV_SUBCRIPTION_ID | 4 Octets | A unique identifier generated by the source and destination and has a valid life time |

Subscription ID Value:

A Subscription ID is a 32-bit random number that is unique within the pair of MIHFs of the source and destination of the session request. The Subscription ID is created jointly by the source and destination MIHF peers at the time of MIH registration to avoid conflict at both ends. The source MIHF chooses the first 2 octets and the destination MIHF chooses the last 2 octets. The Subscription ID is valid only during the lifetime of  event  subscription. The Subscription ID is used in all MIH messages that are related to the Event Service including event subscription.

8.6.1.7. MIH_Event_Subscribe request

8.6.1.8 MIH_Event_Subscribe response

8.6.1.9 MIH_Event_Unsubscribe request

8.6.1.10 MIH_Event_Unsubscribe response

8.6.2.1 MIH_Link_Up indication

8.6.2.1 MIH_Link_Down Indication

8.6.2.3 MIH_Link_Going_Down Indication

8.6.2.4 MIH_Link_Event_ Rollback Indication

8.6.2.5 MIH_Link_Detected  Indication

8.6.2.6 MIH_Link_Parameters_Report Indication

8.6.2.7 MIH_Link_Handover_Imminent Indication

8.6.2.8 MIH_Link_Handover_Complete Indication