| Project | IEEE 802.21 MIHO <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | Optimize Info Query Binary |  |
| Date Submitted | April 27, 2007 |  |
| Source(s) | Yuu-Heng Alice Cheng |  |
| Re: | IEEE 802.21 Session #20 in May 2007 |  |
| Abstract | . |  |
| Purpose | LB#1d comment resolution |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.21. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

# Issues for the current binary encoding

Issue 1: There is additional assigning of encoding for the Querier Location and Network Inclusion.

Querier Location

| Type=101 |  | Length = Variable |  |  |
| --- | --- | --- | --- | --- |
| Location(201) | Length | TYPE_IE_POA_LOCATION | Length | location |
| MAC Address(202) | Length | See 8.5.8 (MAC_ADDRESS) |  |  |
| Net Rad. (203) | Length | TBD |  |  |

Network Inclusion

| Type=103 |  | Length = Variable |  |  |
| --- | --- | --- | --- | --- |
| Access NID(202) | Length | TYPE_IE_ACCESS_NETWORK_ID | Length | ANID_1 |
| … |  |  |  |  |
| Access NID(202) | Length | TYPE_IE_ACCESS_NETWORK_ID | Length | ANID_k |

The specification is not clear if the Network Inclusion will be formatted as:

| Type = 103 | Length = Variable |  |  |
| --- | --- | --- | --- |
| Access NID(202) |  | Length | ANID_1 |
| … |  |  |  |
| Access NID(202) |  | Length | ANID_k |

# Reduce the unnecessary encoding

The payload of the Querier Location and Network Inclusion can directly be the binary encoding TLV.

Querier Location

| Type = 101 | Length = Variable |  |  |
| --- | --- | --- | --- |
| TYPE_IE_POA_LOCATION |  | Length | payload |
| TYPE_IE_POA_MAC_ADDR (??) |  |  |  |
| TYPE_IE_NETWORK_RADIUS |  |  |  |

Network Inclusion

| Type = 103 | Length = Variable |  |  |
| --- | --- | --- | --- |
| TYPE_IE_ACCESS_NETWORK_ID |  | Length | APNID_1 |
| … |  |  |  |
| TYPE_IE_ACCESS_NETWORK_ID |  | Length | APNID_K |

# Proposal

Change 8.6.4.1 for the Querier Location and Network Inclusion to directly use the Binary TLV encodings.

Querier Location

| Type = 101 | Length = Variable |
| --- | --- |
| Location(Optional, format 6.4.6.3.2) |  |
| MAC Address (Optional, format 6.4.6.3.1) |  |
| Neighborhood Radius (Optional) |  |

Network Inclusion

| Type = 103 | Length = Variable |
| --- | --- |
| Access Network Identifier 1 (format 6.4.6.2.1) |  |
| … |  |
| Access Network Identifier k (format 6.4.6.2.1) |  |