| Project | IEEE 802.21 MIHS <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | MIH_N2N_HO_Commit |  |
| DCN | 21-07-0160-02-0000 |  |
| Date Submitted | May 15, 2007 |  |
| Source(s) | Hongseok Jeon, Eunah Kim, Junghoon Jee, Changmin Park ETRI 161 Gajeong-Dong, Yuseong-Gu, Daejeon, 305-700, KOREA | Voice: +82-42-860-3892 Fax: +82-42-861-5404 {jeonhs, jhjee, eakim, cmpark}@etri.re.kr |
| Re: | IEEE 802.21 Ad-hoc in Daejeon, Korea |  |
| Abstract | IEEE P802.21/D05.01 defines primitives and command messages to provide information of commitment to handover request to a specific link and PoA. However, there are no primitives and command messages to communicate between MIHF on serving network and selected target network to which an MN is about to move about the commitment to perform link handover actions. This contribution suggests additional primitives and messages between MIHF on serving network and selected candidate network for providing information of handover commitment. |  |
| Purpose | Adoption |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that IEEE 802.2 may make this contribution public. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

1. Problem Statement

The IEEE 802.21 WG decided to change the naming of primitives and messages to clearly distinguish their role and direction. Therefore, the previous MIH_Handover_Commit was changed as MIH_MN_HO_Commit and MIH_Net_HO_Commit. However, the part of MIH_N2N_HO_Commit is missing from the current D05.01 However, the previous definition of MIH_HO_Commit was to utilize this between Mobile Node and Serving PoS, also for between Serving PoS and Target PoS.

2. Suggested Remedy

We suggest including Section 7.4.23, which describes “MIH_N2N_HO_Commit” primitives.

We suggest making an amendment of Table 51 in Section 8.4.4 for adding “MIH_N2N_HO_Commit” command message to be exchanged between MIHF on the serving network and MIHF on the candidate network.

We suggest including Section 8.6.3.17 describing “MIH_N2N_HO_Commit  request” command message.

We suggest including Section 8.6.3.18 describing “MIH_N2N_HO_Commit response” command message.

7.4.23 MIH_N2N_HO_Commit

7.4.23.1 MIH_N2N_HO_Commit.request

7.4.23.1.1 Function

This primitive is used by an MIH User on the serving network to inform a selected target network to commit handover to the target network. MIH User on the serving network may use this primitive to expect some behaviors for an incoming mobile node on the target network.

7.4.23.1.2 Semantics of service primitive

The parameters of the primitive are as follows:

MIH_N2N_HO_Commit.request  (

DestinationIdentifier,

MobileNodeIdentifier

)

Local or Remote: Remote

MIHF (Network)<->MIHF (Network)

| Name | Type | Description |
| --- | --- | --- |
| Destination Identifier | MIHF_ID | This identifies a remote MIHF which will be the destination of this request. |
| Mobile Node Identifier | MIHF_ID | This identifies a MIHF of the mobile node which commits to perform handover action. |

7.4.23.1.3 When generated

The MIH User on the serving network invokes this primitive when a single target network has been decided.

7.4.23.1.4 Effect on receipt

Upon receipt of this primitive by the local MIHF, it shall generate and send an MIH_N2N_HO_Commit.request message to the remote MIHF on the selected target network identified by the Destination Identifier.

7.4.23.2 MIH_N2N_HO_Commit.indication

7.4.23.2.1 Function

This primitive is generated by a MIHF to indicate that an MIH_N2N_HO_Commit.request message has been received from a peer MIHF on serving network.

7.4.23.2.2 Semantics of service primitive

The parameters of the primitive are as follows:

MIH_N2N_HO_Commit.indication  (

SourceIdentifier,

MobileNodeIdentifier

)

Local or Remote: Local

| Name | Type | Description |
| --- | --- | --- |
| Source Identifier | MIHF_ID | This identifies the invoker of this primitive which can be a remote MIHF. |
| Mobile Node Identifier | MIHF_ID | This identifies a MIHF of the mobile node which commits to perform handover action. |

7.4.23.2.3 When generated

This primitive is generated by an MIHF on receiving an MIH_N2N_HO_Commit.request message from a peer MIHF on the serving network.

7.4.23.2.4 Effect on receipt

The MIH User receiving this primitive replies with an MIH_N2N_HO_Commit.response primitive.

7.4.23.3 MIH_N2N_HO_Commit.response

7.4.23.3.1 Function

This primitive is used to send a response to an MIH_N2N_HO_Commit.request message.

7.4.23.3.2. Semantics of service primitive

The parameters of the primitive are as follows:

MIH_N2N_HO_Commit.response (

DestinationIdentifier,

MobileNodeIdentifier,

Status

)

Local or Remote: Remote

MIHF (Network)<->MIHF (Network)

| Name | Type | Description |
| --- | --- | --- |
| Destination Identifier | MIHF_ ID | This identifies a remote MIHF which will be the destination of this request. |
| Mobile Node Identifier | MIHF_ID | This identifies a MIHF of the mobile node which commits to perform handover action. |
| Status | STATUS | Status of operation: |

7.4.23.3.3 When generated

This primitive is generated in response to an MIH_N2N_HO_Commit.request primitive.

7.4.23.3.4 Effect on receipt

Upon receipt, the MIHF on the serving network may determine that the handover commitment request was processed successfully. Upon receipt, the MIHF on the serving network may start forwarding packets destined for the MN to new Target PoS in the selected target network.

7.4.23.4 MIH_N2N_HO_Commit.confirm

7.4.23.4.1 Function

This primitive is generated by the MIHF to confirm that an MIH_N2N_HO_Commit.response message is received from a peer MIHF on the selected target network.

7.4.23.4.2 Semantics of service primitive

The parameters of the primitive are as follows:

MIH_Net_HO_Commit.confirm (

SourceIdentifier,

MobileNodeIdentifier,

Status

)

Local or Remote: Local

| Name | Type | Description |
| --- | --- | --- |
| Destination Identifier | MIHF_ ID | This identifies a remote MIHF which will be the destination of this request. |
| Mobile Node Identifier | MIHF_ID | This identifies a MIHF of the mobile node which commits to perform handover action. |
| Status | STATUS | Status of operation: |

7.4.23.4.3 When generated

This primitive is generated by the MIHF on receiving an MIH_N2N_HO_Commit.response message from a peer MIHF on the selected target network.

7.4.23.4.4 Effect on receipt

Upon receipt, the MIHF on the serving network may determine that the handover commitment request was processed successfully. Upon receipt, the MIHF in the serving network may start forwarding packets destined for the MN to new Target PoS in the selected target network. However, if Status does not indicate “Success”, the recipient shall perform appropriate error handling.

8.6.3.17 MIH_N2N_HO_Commit request

This message is used by the MIHF on the serving network to communicate with its peer MIHF on the selected target network. This is used to inform the target network of the intent to commit to a handover request to a specific link and PoA, which the target network provides.

MIH Header Fixed Fields (SID=3, Opcode=1, AID=9)

Source Identifier = sending MIHF ID (Source MIHF ID TLV)

Destination Identifier = receiving MIHF ID (Destination MIHF ID TLV)

MobileNodeIdentifier (Mobile Node MIHF ID TLV)

8.6.3.18 MIH_N2N_HO_Commit response

This message is used by the MIHF on the selected target network to communicate with its peer MIHF on the serving network. This is used to respond to the request to commit to a handover request to a specific link and PoA, which the target network provides.

MIH Header Fixed Fields (SID=3, Opcode=2, AID=9)

Source Identifier = sending MIHF ID (Source MIHF ID TLV)

Destination Identifier = receiving MIHF ID (Destination MIHF ID TLV)

MobileNodeIdentifier (Mobile Node MIHF ID TLV)

Status (Status TLV)