IEEE 802.21
Media Independent Handover Services

Enhancement to Table 24 – MIH_LINK_SAP/802.16 primitives mapping

Date:	May 15, 2007

Authors:	Juan Carlos Zuniga, InterDigital Communications

Jin Lee, LG Electronics Inc.

e-Mail:	 juancarlos.zuniga@interdigital.com

jins978@lge.com

Abstract

Table 24 is incomplete and still contains field that are TBD. This contribution aims at resolving some of these open issues. In table 24, lines 52 and 53 are modified. The following mapping was done:

- Link_Action(LINK_DISCONNECT): C-NEM-REQ/RSP(Deregistration)

- Link_Action(LINK_POWER_DOWN): M-SSM-REQ/RSP(Power Down)

- Link_Action(LINK_POWER_UP): M-SSM-REQ/RSP(Power On)

Old

#### Section 7.2.2.1 (‘Table 24 – MIH_LINK_SAP/802.16 primitives mapping’) – Page 74

7.2.2.1 MIH_LINK_SAP

Table 24—MIH_LINK_SAP/802.16 primitives mapping

| MIH_LINK_SAP Primitive | IEEE 802.16 C_SAP | IEEE 802.16 M_SAP |
| --- | --- | --- |
| Link_Event_Subscribe |  |  |
| Link_Event_Unsubscribe |  |  |
| Link_Configure_Thresholds | C-HO-REQ/RSP(HO-Scan) | N/A |
| Link_Up | C-NEM-RSP(Registration) | N/A |
| Link_Down | N/A | M-MTM-RSP(Deregistration) |
| Link_Going_Down | N/A | N/A |
| Link_Event_Rollback | N/A | N/A |
| Link_Detected | C-HO-RSP(HO-Scan) | N/A |
| Link_Parameters_Report | C-HO-IND(HO-Scan) C-HO-RSP(HO-Scan) C-RRM-RSP C-SFM-RSP | N/A |
| Link_PDU_Transmit_Status | N/A | N/A |
| Link_Handover_Imminent | C-HO-IND(HO-Start) | N/A |
| Link_Handover_Complete | C-HO-IND(HO-CMPLT) | N/A |
| Link_Capability_Discover | TBD | TBD |
| Link_Get_Parameters | C-SFM-REQ/RSP C-HO-REQ/RSP/IND (HO-Scan) C-RRM-REQ/RSP | N/A |
| Link_Action | TBD | TBD |

New

7.2.2.1 MIH_LINK_SAP

Table 24—MIH_LINK_SAP/802.16 primitives mapping

| MIH_LINK_SAP Primitive | IEEE 802.16 C_SAP | IEEE 802.16 M_SAP |
| --- | --- | --- |
| Link_Event_Subscribe |  |  |
| Link_Event_Unsubscribe |  |  |
| Link_Configure_Thresholds | C-HO-REQ/RSP(HO-Scan) | N/A |
| Link_Up | C-NEM-RSP(Registration) | N/A |
| Link_Down | N/A | M-MTM-RSP(Deregistration) |
| Link_Going_Down | N/A | N/A |
| Link_Event_Rollback | N/A | N/A |
| Link_Detected | C-HO-RSP(HO-Scan) | N/A |
| Link_Parameters_Report | C-HO-IND(HO-Scan) C-HO-RSP(HO-Scan) C-RRM-RSP C-SFM-RSP | N/A |
| Link_PDU_Transmit_Status | N/A | N/A |
| Link_Handover_Imminent | C-HO-IND(HO-Start) | N/A |
| Link_Handover_Complete | C-HO-IND(HO-CMPLT) | N/A |
| Link_Capability_Discover | TBD | TBD |
| Link_Get_Parameters | C-SFM-REQ/RSP C-HO-REQ/RSP/IND (HO-Scan) C-RRM-REQ/RSP | N/A |
| Link_Action (LINK_DISCONNECT) | C-NEM-REQ/RSP (Deregistration) | N/A |
| Link_Action (LINK_LOW_POWER) | C-IMM-REQ/RSP (Idle_Mode_Initiation) | N/A |
| Link_Action (LINK_POWER_DOWN) | N/A | M-SSM-REQ/RSP (Power down) |
| Link_Action (LINK_POWER_UP) | N/A | M-SSM-REQ/RSP (Power on) |