IEEE 802.21
Media Independent Handover Services

Resource List Definition

Date:	June 10, 2007

Authors:	Juan Carlos Zuniga, InterDigital Communications

e-Mail:	 juancarlos.zuniga@interdigital.com

Abstract

This document presents a solution to the resource definition description that is currently missing in the D5.1 version of the 802.21 specification.

# Background

In mobile wireless technologies such as 3GPP, an inter-RAT handover would have the following sequence of events:

- Query phase - to find out the status of resources at both source and target networks before taking a handover decision

- Preparation phase - where resources are reserved at the target network once a handover decision has been taken

- Execution phase - where the handover commands are sent and performed

- Completion phase - where the result of the handover is informed and the original resources are released at the source network

Currently, the 802.21 D05.01 specification supports the following handover messages:

- Query

  - MIH_Get_Link_Parameters request

  - MIH_Get_Link_Parameters response

  - MIH_Configure_Link request

  - MIH_Configure_Link response

  - MIH_Scan request

  - MIH_Scan response

  - MIH_Net_HO_Candidate_Query request

  - MIH_Net_HO_Candidate_Query response

  - MIH_MN_HO_Candidate_Query request

  - MIH_MN_HO_Candidate_Query response

  - MIH_N2N_HO_Query_Resources request

  - MIH_N2N_HO_Query_Resources response

- Preparation/Execution

  - MIH_Net_HO_Commit request

  - MIH_Net_HO_Commit response

  - MIH_MN_HO_Commit request

  - MIH_MN_HO_Commit response

  - MIH_N2N_HO_Commit request

  - MIH_N2N_HO_Commit response

- Completion

  - MIH_MN_HO_Complete request

  - MIH_MN_HO_Complete response

  - MIH_N2N_HO_Complete request

  - MIH_N2N_HO_Complete response

# Proposed Solution

Following the logic described in the background section, we propose the following Information Element resource definitions:

- Available Resources

  - This information element is used in the Query phase to find out the status of resources before taking a handover decision

- Requested Resources

  - This information element is used in the Preparation phase to request the reservation of resources in target network once a handover decision has been taken.

- Assigned Resources

  - This information element is used in the Preparation phase to respond to the source network about the (successful) reservation of resources in the target network. Similarly, in a network controlled handover, this same information element is used in the Execution phase to inform the MN about the resource assignation in the new network, for instance when a HO command is issued.

Following the above mentioned principles, this would be the resource Information Element applicability to the current 802.21 messages:

- Query Resource List

  - MIH_Get_Link_Parameters request

  - MIH_Net_HO_Candidate_Query request

  - MIH_MN_HO_Candidate_Query request

  - MIH_N2N_HO_Query_Resources request

- Available Resource List

  - MIH_Get_Link_Parameters response

  - MIH_Configure_Threshold request

  - MIH_Scan request

  - MIH_Net_HO_Candidate_Query response

  - MIH_MN_HO_Candidate_Query response

  - MIH_N2N_HO_Query_Resources response

- Measurements Request Set

  - MIH_Configure_Threshold request

  - MIH_Scan request

  - MIH_Net_HO_Candidate_Query request

- Measurements Response Set

  - MIH_Link_Parameters_Report indication

  - MIH_Scan response

  - MIH_Net_HO_Candidate_Query response

- Requested Resource Set

  - MIH_Net_HO_Commit request

  - MIH_MN_HO_Commit request

  - MIH_N2N_HO_Commit request

- Assigned Resource Set

  - MIH_MN_HO_Complete request

  - MIH_MN_HO_Complete response

  - MIH_N2N_HO_Complete request

  - MIH_N2N_HO_Complete response

- Handover Result

  - MIH_Net_HO_Commit response

  - MIH_MN_HO_Complete request

  - MIH_N2N_HO_Complete request

- Resource Status

  - MIH_Net_HO_Commit response

  - MIH_N2N_HO_Complete response

The following tables provide the resource definition elements

| TLV Type Name | TLV Type Value | Data Type | Description |
| --- | --- | --- | --- |
| Query Resource List |  | QUERY_RESOURCE_LIST | TBD. |
| Available Resource List |  | AVAILABLE_RESOURCE_LIST |  |
| Measurements Request Set |  | SEQUENCE(LIST(CELL_ID), MEASUREMENT_TYPE) |  |
| Measurements Response Set |  | SEQUENCE(LIST(CELL_ID), LIST (MEASUREMENT_VALUE) |  |
| Requested Resource Set |  | REQUESTED_LINK_RESOURCE_SET |  |
| Assigned Resource Set |  | ASSIGNED_LINK_RESOURCE_SET |  |

| Data Type Name | Derived From | Definition | Valid Range |
| --- | --- | --- | --- |
| QUERY_RESOURCE_LIST | LIST (NETWORK_TYPE) |  |  |
| AVAILABLE_RESOURCE_LIST | LIST (CELL_RESOURCE_SET) |  |  |
| CELL_RESOURCE_SET | SEQUENCE (CELL_ID, FREQUENCY_CHANNEL, SCRAMBLING_CODE, MEASUREMENT_TYPE) |  |  |
| CELL_ID | MAC_ADDRESS |  |  |
| FREQUENCY_CHANNEL |  |  |  |
| SCRAMBLING_CODE |  |  |  |
| MEASUREMENT_TYPE |  |  |  |
| MEASUREMENT_VALUE |  |  |  |
| REQUESTED_LINK_RESOURCE_SET | SEQUENCE (MN_ID, CELL_ID, PDP_CONTEXTS, SOURCE2TARGET_TRANSPARENT_CONTAINER, HO_CAUSE, CONNECTION_ID, MN_CAPABILITY) |  |  |
| ASSIGNED_LINK_RESOURCE_SET | SEQUENCE (HO_CAUSE, PDP_CONTEXTS, TUNNEL_ENDPOINT_ID, POA_FA_AR_ADDRESS, TARGET2SOURCE_TRANSPARENT_CONTAINER) |  |  |

# Inter-RAT Handovers in 3GPP (informative)

As explained in [1], the 3GPP Inter-RAT handover is based on a few key principles:

- In multiple RAT scenarios, the UE uses a single RAT for all in-progress services.

- Inter-RAT handover decision is made (and the handover command is sent) by the serving RAN. The target RAN may exercise admission control to UEs that are handed over.

- The serving RAN receives information from the target RAN that can be included in the handover command.

Figure 1 shows the message sequence for an inter-SGSN inter-RAT handover between GERAN and UTRAN:

.

Figure 1: Inter-RAT handover from GERAN to UTRAN

The three principles mentioned above are reflected in the following way in this sequence:

- In multiple RAT scenarios, the UE uses a single RAT for all in-progress services.

    - The UE switches RAT between message (7) and message (8).

- Inter-RAT handover decision is made (and the handover command is sent) by the serving RAN. The target RAN may exercise admission control to UEs that are handed over.

    - The source BSC decides that the handover is required and triggers the “PS Handover Required” message (1).

    - The target RNC can perform admission control when receiving the “Relocation Request” (message (3)).

- The serving RAN receives information from the target RAN that can be included in the handover command.

    - Via messages 4, 5, 6 and 7, the target RNC is able to provide a detailed radio configuration to the UE with the UTRAN RRC message “HANDOVER TO UTRAN COMMAND”.

Above three principles are already also applied for handovers to/from the CDMA-2000 non-3GPP RAT.

# References

[1] MOB-070005 [3GPP TSG-RAN Workshop on Radio mobility: ftp://www.3gpp.org/workshop/2007-05-28_Radio_Mobility_non_3GPP/Docs/