IEEE 802.21
Media Independent Handover Services

Resource List Definition

Date:	June 10, 2007

Authors:	Juan Carlos Zuniga, InterDigital Communications

Jin Lee, LG Electronics Inc.

e-Mail:	 juancarlos.zuniga@interdigital.com

jins978@lge.com

Abstract

This document presents a solution to the resource definition description that is currently missing in the D5.1 version of the 802.21 specification.

# Background

In mobile wireless technologies such as 3GPP, an inter-RAT handover would have the following sequence of events:

- Query phase - to find out the status of resources at both source and target networks before taking a handover decision

- Preparation phase - where resources are reserved at the target network once a handover decision has been taken

- Execution phase - where the handover commands are sent and performed

- Completion phase - where the result of the handover is informed and the original resources are released at the source network

Currently, the 802.21 D05.01 specification supports the following handover messages:

- Query

  - MIH_Get_Link_Parameters request

  - MIH_Get_Link_Parameters response

  - MIH_Configure_Link request

  - MIH_Configure_Link response

  - MIH_Scan request

  - MIH_Scan response

  - MIH_Net_HO_Candidate_Query request

  - MIH_Net_HO_Candidate_Query response

  - MIH_MN_HO_Candidate_Query request

  - MIH_MN_HO_Candidate_Query response

  - MIH_N2N_HO_Query_Resources request

  - MIH_N2N_HO_Query_Resources response

- Preparation/Execution

  - MIH_Net_HO_Commit request

  - MIH_Net_HO_Commit response

  - MIH_MN_HO_Commit request

  - MIH_MN_HO_Commit response

  - MIH_N2N_HO_Commit request

  - MIH_N2N_HO_Commit response

- Completion

  - MIH_MN_HO_Complete request

  - MIH_MN_HO_Complete response

  - MIH_N2N_HO_Complete request

  - MIH_N2N_HO_Complete response

# Proposed Solution

Following the logic described in the background section, we propose the following Information Element resource definitions:

- Query Resources

  - This information element is used in the Query phase and it refers to the network that is going to be queried for resources

- Available Resources

  - This information element is used in the Query phase to find out the status of resources before taking a handover decision

- Requested Resources

  - This information element is used in the Preparation phase to request the reservation of resources in target network once a handover decision has been taken.

- Assigned Resources

  - This information element is used in the Preparation phase to respond to the source network about the (successful) reservation of resources in the target network. Similarly, in a network controlled handover, this same information element is used in the Execution phase to inform the MN about the resource assignation in the new network, for instance when a HO command is issued.

Additionally, the following Information Elements are also complementary:

- Measurements Request List

  - This information element makes use of part of the information provided in the Available Resources information element to request performing measurements

- Measurements Response List

  - This information element contains the result of the different measurements taken

- Handover Result

  - This IE informs about the result of the handover, and it is already defined in D05.01

- Result Status

  - This IE informs about the status of the original resources after the handover result has been communicated back to the source. This IE is already defined in D05.01 and a minor editorial change is proposed.

Following the above mentioned principles, this would be the resource Information Element applicability to the current 802.21 messages:

- Query Resource List

  - MIH_Get_Link_Parameters request

  - MIH_Net_HO_Candidate_Query request

  - MIH_MN_HO_Candidate_Query request

  - MIH_N2N_HO_Query_Resources request

- Available Resource List

  - MIH_Get_Link_Parameters response

  - MIH_Configure_Threshold request

  - MIH_Scan request

  - MIH_Net_HO_Candidate_Query response

  - MIH_MN_HO_Candidate_Query response

  - MIH_N2N_HO_Query_Resources response

- Measurements Request List

  - MIH_Configure_Threshold request

  - MIH_Scan request

  - MIH_Net_HO_Candidate_Query request

- Measurements Response List

  - MIH_Link_Parameters_Report indication

  - MIH_Scan response

  - MIH_Net_HO_Candidate_Query response

- Requested Resource Set

  - MIH_Net_HO_Commit request

  - MIH_MN_HO_Commit request

  - MIH_N2N_HO_Commit request

- Assigned Resource Set

  - MIH_MN_HO_Complete request

  - MIH_MN_HO_Complete response

  - MIH_N2N_HO_Complete request

  - MIH_N2N_HO_Complete response

- Handover Result

  - MIH_Net_HO_Commit response

  - MIH_MN_HO_Complete request

  - MIH_N2N_HO_Complete request

- Resource Status

  - MIH_Net_HO_Commit response

  - MIH_N2N_HO_Complete response

The following tables provide the resource definition elements

| TLV Type Name | TLV Type Value | Data Type |
| --- | --- | --- |
| Query Resource List |  | QUERY_RESOURCE_LIST |
| Available Resource List |  | AVAILABLE_RESOURCE_LIST |
| Measurements Request List |  | MESAUREMENT_REQUEST_LIST |
| Measurements Response List |  | MEASUREMENT_RESPONSE_LIST |
| Requested Resource Set |  | REQUESTED_LINK_RESOURCE_SET |
| Assigned Resource Set |  | ASSIGNED_LINK_RESOURCE_SET |
| Handover Result |  | HANDOVER_RESULT |
| Resource Status |  | LINK_RESOURCE_RETENTION_STATUS |

| Data Type Name | Derived From | Definition | Valid Range |
| --- | --- | --- | --- |
| QUERY_RESOURCE_LIST | LIST (NETWORK_TYPE) | List of networks that are queried for possible handovers | N/A |
| AVAILABLE_RESOURCE_LIST | LIST (SEQUENCE (NETWORK_TYPE, LIST (CELL_RESOURCE_SET))) | List of available cells per network type | N/A |
| MESAUREMENT_REQUEST_LIST | LIST (SEQUENCE (NETWORK_TYPE, LIST (CELL_RESOURCE_SET), MEASUREMENT_TYPES)) | List of measurement types to be taken and list of cells to be measured, per network type | N/A |
| MEASUREMENT_RESPONSE_LIST | LIST (SEQUENCE (NETWORK_TYPE, MEASUREMENT_VALUES)) | List of results of measurements taken for each cell, per network type | N/A |
| REQUESTED_LINK_RESOURCE_SET | SEQUENCE (NETWORK_TYPE, MN_ID, CELL_ID, QOS_PARAM_LIST, SOURCE2TARGET_TRANSPARENT_CONTAINER, HANDOVER_CAUSE, CONNECTION_ID, MN_CAPABILITIES) | Set of parameters required for performing admission control at the target network | N/A |
| ASSIGNED_LINK_RESOURCE_SET | SEQUENCE (NETWORK_TYPE, HANDOVER_STATUS, QOS_PARAM_LIST, TUNNEL_ENDPOINT_ID, POA_FA_AR_ADDRESS, TARGET2SOURCE_TRANSPARENT_CONTAINER, LINK_AC_EXEC_TIME) | Set of parameters assigned by the target network | N/A |
| CELL_RESOURCE_SET | SEQUENCE (CELL_ID, CHANNEL_ID, FREQUENCY_CODE, LIST(PHY_MODE)) | Set of parameters required to identify and monitor a wireless cell (i.e. PoA or BS) | N/A |
| MN_ID | LINK_ID | Mobile Node ID | N/A |
| CELL_ID | LINK_ID | PoA Base Station Cell ID | N/A |
| PHY_MODE | SEQUENCE (RANGING_CDMA_CODES, BANDWIDTH, FFT_SIZE, CYCLIC_PREFIX, FRAME_DURATION, TTG, RTG, DL_BURST_PROFILE, UL_BURST_PRIFILE) | PHY mode information | N/A |
| FREQUENCY_CODE | INTEGER(2) | Frequency code number (i.e. UMTS scrambling code, cdma2000 Walsh code, etc.) | 0-65535 |
| MEASUREMENT_TYPES | LIST (LINK_PARAM_TYPE) | List of measurement types | N/A |
| MEASUREMENT_VALUES | LIST (SEQUENCE (CELL_ID, LINK_PARAM)) | List of measurement types and their associated reported values | N/A |
| QOS_PARAM_LIST | LIST (SEQUENCE (QOS_TYPE, QOS_PARAM)) | List of QoS parameters | N/A |
| HANDOVER_CAUSE | ENUMERATED | Represents the reason for performing a handover | See Table (TBD) |
| CONNECTION_ID | INTEGER(1) | Data connection identifier | 0-255 |
| TUNNEL_ENDPOINT_ID | INTEGER(1) | Tunnel endpoint identifier | 0-255 |
| MN_CAPABILITIES | SEQUENCE (MN_CAPABILITIES_TYPE, MN_CAPABILITIES_VALUE) | Mobile node capabilities | N/A |
| MN_CAPABILITIES_TYPE | ENUMERATED | Mobile node capabilities for establishing a new connection | 0: Parameter not available 1: GERAN Capabilities 2: 3GPP Capabilities 3 : IEEE 802.16 Capabilities |
| MN_CAPABILITIES_VALUE | BITMAP(64) | Bitmap representing the MN capabilities depending on the access technology | See Table (TBD) |
| SOURCE2TARGET_TRANSPARENT_CONTAINER | SEQUENCE (S2T_CONTAINER_TYPE, S2T_CONTAINER_VALUE) | Source to target transparent container including the required configuration for admitting the new connection at the target network | N/A |
| S2T_CONTAINER_TYPE | ENUMERATED | Type of source to target container | 0:Parameter not available 1:Pre-defined configuration 2-255: Reserved |
| S2T_CONTAINER_VALUE | CHOICE (PREDEFINED_CONFIG_ID) | A value associated to the source to target container type | N/A |
| PREDEFINED_CONFIG_ID | INTEGER(1) | Pre-defined configuration identifier | 0-255 |
| TARGET2SOURCE_TRANSPARENT_CONTAINER | SEQUENCE (T2S_CONTAINER_TYPE, T2S_CONTAINER_VALUE) | Target to source transparent container including the required configuration for admitting the new connection at the target network | N/A |
| T2S_CONTAINER_TYPE | ENUMERATED | Type of target to source container | 0:Parameter not available 1:Pre-defined configuration 2-255: Reserved |
| T2S_CONTAINER_VALUE | CHOICE (PREDEFINED_CONFIG_ID) | A value associated to the source to target container type | N/A |
| HANDOVER_RESULT | [Same as D05.01] | [Same as D05.01] | [Same as D05.01] |
| LINK_RESOURCE_RETENTION_STATUS | BOOLEAN | Represents the status of the resources | True: Retaining resources False: Releasing resources |

| Definition | Valid Range |
| --- | --- |
| GERAN Capabilities | Bit 0: Multiband supported Bit 1: Associated Radio Capability 2 Bit 2: Associated Radio Capability 1 Bit 3: R Support Bit 4: HSCSD Multi Slot Capability Bit 5: UCS2 treatment Bit 6: Extended Measurement Capability Bit 7: MS measurement capability Bit 8: MS Positioning Method Capability Bit 9: ECSD Multi Slot Capability Bit 10: 8-PSK Struct Bit 11: GSM 400 Bands Supported Bit 12: GSM 400 Associated Radio Capability Bit 13: GSM 850 Associated Radio Capability Bit 14: GSM 1900 Associated Radio Capability Bit 15: UMTS FDD Radio Access Technology Capability Bit 16: UMTS 3.84 Mcps TDD Radio Access Technology Capability Bit 17: CDMA 2000 Radio Access Technology Capability Bit 18: DTM GPRS Multi Slot Class Bit 19: Single Slot DTM Bit 20: DTM EGPRS Multi Slot Class Bit 21: Single Band Support Bit 22: GSM 750 Associated Radio Capability Bit 23: UMTS 1.28 Mcps TDD Radio Access Technology Capability Bit 24: GERAN Feature Package 1 Bit 25: Extended DTM GPRS Multi Slot Class Bit 26: Extended DTM EGPRS Multi Slot Class Bit 27: High Multislot Capability Bit 28: GERAN Iu Mode Capabilities Bit 29: GERAN Feature Package 2 Bit 30: GMSK Multislot Power Profile Bit 30: 8-PSK Multislot Power Profile Bit 31: T-GSM 400 Bands Supported Bit 32: T-GSM 400 Associated Radio Capability Bit 33: T-GSM 900 Associated Radio Capability Bit 34: Downlink Advanced Receiver Performance Bit 35: DTM Enhancements Capability Bit 36: DTM GPRS High Multi Slot Class Bit 37: Offset required Bit 38: DTM EGPRS High Multi Slot Class Bit 39: Repeated ACCH Capability Bit 40: GSM 710 Associated Radio Capability Bit 41: T-GSM 810 Associated Radio Capability Bit 42: Ciphering Mode Setting Capability Bit 43: Multislot Capability Reduction for Downlink Dual Carrier Bit 44: R-GSM band Associated Radio Capability Bit 45: HSCSD Multi Slot Capability Bit 46: MS Measurement Bit 47: MS Positioning Method Capability Bit 48: ECSD Multi Slot Capability Bit 49: 8-PSK RF Power Capability 1 Bit 50: 8-PSK RF Power Capability 2 Bit 51: FLO Iu Capability Bit 52-64: Reserved |
| UMTS Capabilities | Bit 0: Predefined configuration status information (and compressed) - UE security information - UE capability container - UE radio access capability (and extenstion) - Release indicator - PDCP, RLC, transport channel, physical channel, RF, UE positioning, Measurement capabilities - Device type - Support for SIB 11bis Support Bit 4: HSCSD Multi Slot Capability Bit 5: UCS2 treatment Bit 6: Extended Measurement Capability Bit 7: MS measurement capability Bit 8: MS Positioning Method Capability Bit 9: ECSD Multi Slot Capability Bit 10: 8-PSK Struct Bit 11: GSM 400 Bands Supported Bit 12: GSM 400 Associated Radio Capability Bit 13: GSM 850 Associated Radio Capability Bit 14: GSM 1900 Associated Radio Capability Bit 15: UMTS FDD Radio Access Technology Capability Bit 16: UMTS 3.84 Mcps TDD Radio Access Technology Capability Bit 17: CDMA 2000 Radio Access Technology Capability Bit 18: DTM GPRS Multi Slot Class Bit 19: Single Slot DTM Bit 20: DTM EGPRS Multi Slot Class Bit 21: Single Band Support Bit 22: GSM 750 Associated Radio Capability Bit 23: UMTS 1.28 Mcps TDD Radio Access Technology Capability Bit 24: GERAN Feature Package 1 Bit 25: Extended DTM GPRS Multi Slot Class Bit 26: Extended DTM EGPRS Multi Slot Class Bit 27: High Multislot Capability Bit 28: GERAN Iu Mode Capabilities Bit 29: GERAN Feature Package 2 Bit 30: GMSK Multislot Power Profile Bit 30: 8-PSK Multislot Power Profile Bit 31: T-GSM 400 Bands Supported Bit 32: T-GSM 400 Associated Radio Capability Bit 33: T-GSM 900 Associated Radio Capability Bit 34: Downlink Advanced Receiver Performance Bit 35: DTM Enhancements Capability Bit 36: DTM GPRS High Multi Slot Class Bit 37: Offset required Bit 38: DTM EGPRS High Multi Slot Class Bit 39: Repeated ACCH Capability Bit 40: GSM 710 Associated Radio Capability Bit 41: T-GSM 810 Associated Radio Capability Bit 42: Ciphering Mode Setting Capability Bit 43: Multislot Capability Reduction for Downlink Dual Carrier Bit 44: R-GSM band Associated Radio Capability Bit 45: HSCSD Multi Slot Capability Bit 46: MS Measurement Bit 47: MS Positioning Method Capability Bit 48: ECSD Multi Slot Capability Bit 49: 8-PSK RF Power Capability 1 Bit 50: 8-PSK RF Power Capability 2 Bit 51: FLO Iu Capability Bit 52-64: Reserved |

# Inter-RAT Handovers in 3GPP (informative)

As explained in [1], the 3GPP Inter-RAT handover is based on a few key principles:

- In multiple RAT scenarios, the UE uses a single RAT for all in-progress services.

- Inter-RAT handover decision is made (and the handover command is sent) by the serving RAN. The target RAN may exercise admission control to UEs that are handed over.

- The serving RAN receives information from the target RAN that can be included in the handover command.

Figure 1 shows the message sequence for an inter-SGSN inter-RAT handover between GERAN and UTRAN:

.

Figure 1: Inter-RAT handover from GERAN to UTRAN

The three principles mentioned above are reflected in the following way in this sequence:

- In multiple RAT scenarios, the UE uses a single RAT for all in-progress services.

    - The UE switches RAT between message (7) and message (8).

- Inter-RAT handover decision is made (and the handover command is sent) by the serving RAN. The target RAN may exercise admission control to UEs that are handed over.

    - The source BSC decides that the handover is required and triggers the “PS Handover Required” message (1).

    - The target RNC can perform admission control when receiving the “Relocation Request” (message (3)).

- The serving RAN receives information from the target RAN that can be included in the handover command.

    - Via messages 4, 5, 6 and 7, the target RNC is able to provide a detailed radio configuration to the UE with the UTRAN RRC message “HANDOVER TO UTRAN COMMAND”.

Above three principles are already also applied for handovers to/from the CDMA-2000 non-3GPP RAT.

# References

[1] MOB-070005 [3GPP TSG-RAN Workshop on Radio mobility: ftp://www.3gpp.org/workshop/2007-05-28_Radio_Mobility_non_3GPP/Docs/