IEEE 802.21
Media Independent Handover Services

Resource List Definition

Date:	June 10, 2007

Authors:	Juan Carlos Zuniga, InterDigital Communications

Jin Lee, LG Electronics Inc.

e-Mail:	 juancarlos.zuniga@interdigital.com

jins978@lge.com

Abstract

This document presents a solution to the resource definition description that is currently missing in the D5.1 version of the 802.21 specification.

# Background

In mobile wireless technologies such as 3GPP, an inter-RAT handover would have the following sequence of events:

- Query phase - to find out the status of resources at both source and target networks before taking a handover decision

- Preparation phase - where resources are reserved at the target network once a handover decision has been taken

- Execution phase - where the handover commands are sent and performed

- Completion phase - where the result of the handover is informed and the original resources are released at the source network

Currently, the 802.21 D05.01 specification supports the following handover messages:

- Query

  - MIH_Get_Link_Parameters request

  - MIH_Get_Link_Parameters response

  - MIH_Configure_Link request

  - MIH_Configure_Link response

  - MIH_Scan request

  - MIH_Scan response

  - MIH_Net_HO_Candidate_Query request

  - MIH_Net_HO_Candidate_Query response

  - MIH_MN_HO_Candidate_Query request

  - MIH_MN_HO_Candidate_Query response

  - MIH_N2N_HO_Query_Resources request

  - MIH_N2N_HO_Query_Resources response

- Preparation/Execution

  - MIH_Net_HO_Commit request

  - MIH_Net_HO_Commit response

  - MIH_MN_HO_Commit request

  - MIH_MN_HO_Commit response

  - MIH_N2N_HO_Commit request

  - MIH_N2N_HO_Commit response

- Completion

  - MIH_MN_HO_Complete request

  - MIH_MN_HO_Complete response

  - MIH_N2N_HO_Complete request

  - MIH_N2N_HO_Complete response

# Proposed Solution

Following the logic described in the background section, we propose the following Information Element resource definitions:

- Query Resources

  - This information element is used in the Query phase and it refers to the network that is going to be queried for resources

- Available Resources

  - This information element is used in the Query phase to find out the status of resources before taking a handover decision

- Requested Resources

  - This information element is used in the Preparation phase to request the reservation of resources in target network once a handover decision has been taken.

- Assigned Resources

  - This information element is used in the Preparation phase to respond to the source network about the (successful) reservation of resources in the target network. Similarly, in a network controlled handover, this same information element is used in the Execution phase to inform the MN about the resource assignation in the new network, for instance when a HO command is issued.

Additionally, the following Information Elements are also complementary:

- Measurements Request List

  - This information element makes use of part of the information provided in the Available Resources information element to request performing measurements

- Measurements Response List

  - This information element contains the result of the different measurements taken

- Handover Result

  - This IE informs about the result of the handover, and it is already defined in D05.01

- Result Status

  - This IE informs about the status of the original resources after the handover result has been communicated back to the source. This IE is already defined in D05.01 and a minor editorial change is proposed.

Following the above mentioned principles, we propose using a new resource Information Element definition in the current 802.21 messages, as described below.

Update the information to include the following Information Elements in the Messages (section 8.6) and Primitives (section 7.4) as follows:

- Query Resource List

  - MIH_Get_Link_Parameters request

  - MIH_Net_HO_Candidate_Query request

  - MIH_MN_HO_Candidate_Query request

  - MIH_N2N_HO_Query_Resources request

- Available Resource List

  - MIH_Get_Link_Parameters response

  - MIH_Net_HO_Candidate_Query response

  - MIH_MN_HO_Candidate_Query response

  - MIH_N2N_HO_Query_Resources response

- Measurements Request List

  - MIH_Configure_Threshold request

  - MIH_Scan request

  - MIH_Net_HO_Candidate_Query request

- Measurements Response List

  - MIH_Link_Parameters_Report indication

  - MIH_Scan response

- Requested Resource Set

  - MIH_N2N_HO_Commit request

- Assigned Resource Set

  - MIH_Net_HO_Commit request

  - MIH_N2N_HO_Commit response

- Handover Result

  - MIH_Net_HO_Commit response

  - MIH_MN_HO_Complete request

  - MIH_N2N_HO_Complete request

- Resource Status

  - MIH_MN_HO_Complete response

  - MIH_N2N_HO_Complete response

The following tables provide the resource definition elements

Include the following tables in Annex D: Type values for TLV encoding

| TLV Type Name | TLV Type Value | Data Type |
| --- | --- | --- |
| Query Resource List | (assigned by Editor) | QUERY_RESOURCE_LIST |
| Available Resource List | (assigned by Editor) | AVAILABLE_RESOURCE_LIST |
| Measurements Request List | (assigned by Editor) | MESAUREMENT_REQUEST_LIST |
| Measurements Response List | (assigned by Editor) | MEASUREMENT_RESPONSE_LIST |
| Requested Resource Set | (assigned by Editor) | REQUESTED_LINK_RESOURCE_SET |
| Assigned Resource Set | (assigned by Editor) | ASSIGNED_LINK_RESOURCE_SET |
| Handover Result | (assigned by Editor) | HANDOVER_RESULT |
| Resource Status | (assigned by Editor) | LINK_RESOURCE_RETENTION_STATUS |

Include the following tables in Annex C: Data type definition:

| Data Type Name | Derived From | Definition | Valid Range |
| --- | --- | --- | --- |
| QUERY_RESOURCE_LIST | SEQUENCE (LIST (NETWORK_TYPE), QOS_PARAM_LIST) | List of networks that are queried for possible handovers and QoS of possible HO candidate connection | N/A |
| AVAILABLE_RESOURCE_LIST | LIST (SEQUENCE (NETWORK_TYPE, LIST (CELL_RESOURCE_SET))) | List of available cells per network type | N/A |
| MESAUREMENT_REQUEST_LIST | LIST (SEQUENCE (NETWORK_TYPE, LIST (CELL_RESOURCE_SET), MEASUREMENT_TYPES)) | List of measurement types to be taken and list of cells to be measured, per network type | N/A |
| MEASUREMENT_RESPONSE_LIST | LIST (SEQUENCE (NETWORK_TYPE, MEASUREMENT_VALUES)) | List of results of measurements taken for each cell, per network type | N/A |
| REQUESTED_LINK_RESOURCE_SET | SEQUENCE (NETWORK_TYPE, MN_ID, CELL_ID, QOS_PARAM_LIST, SOURCE2TARGET_TRANSPARENT_CONTAINER, HANDOVER_CAUSE, CONNECTION_ID, MN_CAPABILITIES) | Set of parameters required for performing admission control at the target network | N/A |
| ASSIGNED_LINK_RESOURCE_SET | SEQUENCE (NETWORK_TYPE, HANDOVER_STATUS, QOS_PARAM_LIST, TUNNEL_ENDPOINT_ID, POA_FA_AR_ADDRESS, TARGET2SOURCE_TRANSPARENT_CONTAINER, LINK_AC_EXEC_TIME) | Set of parameters assigned by the target network | N/A |
| CELL_RESOURCE_SET | SEQUENCE (CELL_ID, CHANNEL_ID, FREQUENCY_CODE, LIST(PHY_MODE)) | Set of parameters required to identify and monitor a wireless cell (i.e. PoA or BS) | N/A |
| MN_ID | LINK_ID | Mobile Node ID | N/A |
| CELL_ID | LINK_ID | PoA/Base Station Cell ID | N/A |
| PHY_MODE | SEQUENCE (RANGING_CDMA_CODES, BANDWIDTH, FFT_SIZE, CYCLIC_PREFIX, FRAME_DURATION, TTG, RTG, DL_BURST_PROFILE, UL_BURST_PRIFILE) | PHY mode information | N/A |
| FREQUENCY_CODE | INTEGER(2) | Frequency code number (i.e. UMTS scrambling code, cdma2000 Walsh code, etc.) | 0-65535 |
| MEASUREMENT_TYPES | LIST (LINK_PARAM_TYPE) | List of measurement types | N/A |
| MEASUREMENT_VALUES | LIST (SEQUENCE (CELL_ID, LINK_PARAM)) | List of measurement types and their associated reported values | N/A |
| LINK_PARAM_GSMGERAN | ENUMERATED | A type to represent a link parameter for GSM GERAN. | 0: BLER 1: RxLevNCell 2: RSSI 3-255: (reserved) |
| LINK_PARAM_GPRS |  |  | N/A |
| LINK_PARAM_EDGE |  |  | N/A |
| LINK_PARAM_UMTS | ENUMERATED | A type to represent a link parameter for UMTS. | 0: UL TrCH BLER 1: PhCH_BER 2: TrCH_BER 3: RSCP 4: RTWP 5: Ec/No 6-255: (reserved) |
| QOS_PARAM_LIST | LIST (SEQUENCE (QOS_TYPE, QOS_PARAM)) | List of QoS parameters | N/A |
| HANDOVER_CAUSE | ENUMERATED | Represents the reason for performing a handover | See Table XXX |
| CONNECTION_ID | INTEGER(1) | Data connection identifier | 0-255 |
| TUNNEL_ENDPOINT_ID | INTEGER(1) | Tunnel endpoint identifier | 0-255 |
| MN_CAPABILITIES | SEQUENCE (MN_CAPABILITIES_TYPE, MN_CAPABILITIES_VALUE) | Mobile node capabilities | N/A |
| MN_CAPABILITIES_TYPE | ENUMERATED | Mobile node capabilities for establishing a new connection | 0: Parameter not available 1: GERAN Capabilities 2: 3GPP Capabilities 3 : IEEE 802.16 Capabilities 4-255: Reserved |
| MN_CAPABILITIES_VALUE | BITMAP(128) | Bitmap representing the MN capabilities depending on the access technology | See Table YYY |
| SOURCE2TARGET_TRANSPARENT_CONTAINER | SEQUENCE (S2T_CONTAINER_TYPE, S2T_CONTAINER_VALUE) | Source to target transparent container including the required configuration for admitting the new connection at the target network | N/A |
| S2T_CONTAINER_TYPE | ENUMERATED | Type of source to target container | 0:Parameter not available 1:Pre-defined configuration 2-255: Reserved |
| S2T_CONTAINER_VALUE | CHOICE (PREDEFINED_CONFIG_ID) | A value associated to the source to target container type | N/A |
| PREDEFINED_CONFIG_ID | INTEGER(1) | Pre-defined configuration identifier | 0-255 |
| TARGET2SOURCE_TRANSPARENT_CONTAINER | SEQUENCE (T2S_CONTAINER_TYPE, T2S_CONTAINER_VALUE) | Target to source transparent container including the required configuration for admitting the new connection at the target network | N/A |
| T2S_CONTAINER_TYPE | ENUMERATED | Type of target to source container | 0:Parameter not available 1:Pre-defined configuration 2-255: Reserved |
| T2S_CONTAINER_VALUE | CHOICE (PREDEFINED_CONFIG_ID) | A value associated to the source to target container type | N/A |
| HANDOVER_RESULT | [Same as D05.01] | [Same as D05.01] | [Same as D05.01] |
| LINK_RESOURCE_RETENTION_STATUS | BOOLEAN | Represents the status of the resources | True: Retaining resources False: Releasing resources |

# Table XXX: MN Capabilities

| Definition | Valid Range |
| --- | --- |
| GERAN Capabilities | Bit 0: Multiband supported Bit 1: Associated Radio Capability 2 Bit 2: Associated Radio Capability 1 Bit 3: R Support Bit 4: HSCSD Multi Slot Capability Bit 5: UCS2 treatment Bit 6: Extended Measurement Capability Bit 7: MS measurement capability Bit 8: MS Positioning Method Capability Bit 9: ECSD Multi Slot Capability Bit 10: 8-PSK Struct Bit 11: GSM 400 Bands Supported Bit 12: GSM 400 Associated Radio Capability Bit 13: GSM 850 Associated Radio Capability Bit 14: GSM 1900 Associated Radio Capability Bit 15: UMTS FDD Radio Access Technology Capability Bit 16: UMTS 3.84 Mcps TDD Radio Access Technology Capability Bit 17: CDMA 2000 Radio Access Technology Capability Bit 18: DTM GPRS Multi Slot Class Bit 19: Single Slot DTM Bit 20: DTM EGPRS Multi Slot Class Bit 21: Single Band Support Bit 22: GSM 750 Associated Radio Capability Bit 23: UMTS 1.28 Mcps TDD Radio Access Technology Capability Bit 24: GERAN Feature Package 1 Bit 25: Extended DTM GPRS Multi Slot Class Bit 26: Extended DTM EGPRS Multi Slot Class Bit 27: High Multislot Capability Bit 28: GERAN Iu Mode Capabilities Bit 29: GERAN Feature Package 2 Bit 30: GMSK Multislot Power Profile Bit 30: 8-PSK Multislot Power Profile Bit 31: T-GSM 400 Bands Supported Bit 32: T-GSM 400 Associated Radio Capability Bit 33: T-GSM 900 Associated Radio Capability Bit 34: Downlink Advanced Receiver Performance Bit 35: DTM Enhancements Capability Bit 36: DTM GPRS High Multi Slot Class Bit 37: Offset required Bit 38: DTM EGPRS High Multi Slot Class Bit 39: Repeated ACCH Capability Bit 40: GSM 710 Associated Radio Capability Bit 41: T-GSM 810 Associated Radio Capability Bit 42: Ciphering Mode Setting Capability Bit 43: Multislot Capability Reduction for Downlink Dual Carrier Bit 44: R-GSM band Associated Radio Capability Bit 45: HSCSD Multi Slot Capability Bit 46: MS Measurement Bit 47: MS Positioning Method Capability Bit 48: ECSD Multi Slot Capability Bit 49: 8-PSK RF Power Capability 1 Bit 50: 8-PSK RF Power Capability 2 Bit 51: FLO Iu Capability Bit 52-128: Reserved |
| UMTS Compressed Capabilities | Access Stratum Release (Bit 0 ¦ Bit 1 ¦ Bit 2) 000: REL-99 001: REL-4 010: REL-5 011: REL-6 100: REL-7 Bit 3: Total AM RLC buffer size exceeds 10 kByte (True: Yes, False: No) Bit 4: Support for System Information Block type 11bis (True: Yes, False: No) FDD physical channel capability for HS-PDSCH/E-DCH DL capability with simultaneous HS-DSCH configuration (Bit 5 ¦ Bit 6 ¦ Bit 7) 000: 32 kb/s 001: 64 kb/s 010: 128 kb/s 011: 384 kb/s HS-DSCH physical layer category Bits 8-13: Integer (1..64) As defined in [3GPP TS 25.306 "UE Radio Access Capabilities"] Bit 14: Support of E-DCH (True: Yes, False: No) E-DCH physical layer category Bits 15-18: Integer (1..16) As defined in [3GPP TS 25.306 "UE Radio Access Capabilities"] Frequency band Bit 19: Band I Bit 20: Band II Bit 21: Band III Bit 22: Band VI Bit 23: Band IV Bit 24: Band V Bit 25: Band VII Bit 26: Band VIII Bit 27: Band IX Bit 28: Band X Bits 29-128: Reserved |
| UMTS Capabilities | Access Stratum Release (Bit 0 ¦ Bit 1 ¦ Bit 2) 000: REL-99 001: REL-4 010: REL-5 011: REL-6 100: REL-7 DL capability with simultaneous HS-DSCH configuration (Bit 3 ¦ Bit 4 ¦ Bit 6) 000: 32 kb/s 001: 64 kb/s 010: 128 kb/s 011: 384 kb/s PDCP capability Bit 7: Support for lossless SRNS relocation Bit 8: Support for lossless DL RLC Bit 11: PDU size change Bit 9: Support for RFC2507 Bit 10: Support for RFC3095 Bit 11: Support for RFC3095 context relocation RLC capability: Total RLC AM buffer size (Bit 12 ¦ Bit 13 ¦ Bit 14) 000: 10 001: 50 010: 100 011: 150 100: 500 101: 1000 Bit 15: Maximum RLC AM Window Size (False: 2047, True: 4095) Maximum number of AM entities (Bit 16 ¦ Bit 17 ¦ Bit 18) 000: 4 001: 5 011: 6 100: 8 101: 16 110: 30 DL Transport channel capability Max no of bits received (Bit 19 ¦ Bit 20 ¦ Bit 21 ¦ Bit 22) 0000: 640 0001: 1280 0010: 2560 0011: 3840 0100: 5120 0101: 6400 0110: 7680 1000: 8960 1001: 10240 1010: 20480 1011: 40960 1100: 81920 1101: 163840 Max convolutionally coded bits received (Bit 23 ¦ Bit 24 ¦ Bit 25 ¦ Bit 26) 0000: 640 0001: 1280 0010: 2560 0011: 3840 0100: 5120 0101: 6400 0110: 7680 1000: 8960 1001: 10240 1010: 20480 1011: 40960 1100: 81920 1101: 163840 Max turbo coded bits received (Bit 27 ¦ Bit 28 ¦ Bit 29 ¦ Bit 30) 0000: 640 0001: 1280 0010: 2560 0011: 3840 0100: 5120 0101: 6400 0110: 7680 1000: 8960 1001: 10240 1010: 20480 1011: 40960 1100: 81920 1101: 163840 Maximum number of simultaneous transport channels (Bit 31 ¦ Bit 32 ¦ Bit 33 ¦ Bit 34) 0001: 4 0010: 8 0011: 16 0100: 32 Maximum number of simultaneous CCTrCH (Bit 35 ¦ Bit 36 ¦ Bit 37 ¦ Bit 38) 0000: 1 0001: 2 0010: 3 0011: 4 0100: 5 0101: 6 0110: 7 0111: 8 Max no of received transport blocks (Bit 39 ¦ Bit 40 ¦ Bit 41 ¦ Bit 42) 0000: 4 0001: 8 0010: 16 0100: 32 0101: 48 0110: 64 0111: 96 1000: 128 1001: 256 1010: 512 Maximum number of TFC (Bit 43 ¦ Bit 44 ¦ Bit 45 ¦ Bit 46) 0000: 16 0001: 32 0010: 48 0011: 64 0100: 96 0101: 128 0110: 256 0111: 512 1000: 1024 Maximum number of TF (Bit 47 ¦ Bit 48 ¦ Bit 49 ¦ Bit 50) 0000: 32 0001: 64 0010: 128 0011: 256 0100: 512 0101: 1024 Bit 51: Support for turbo decoding (False: not supported, True: supported) Bit 52-64: Reserved UL Transport channel capability Max no of bits transmitted (Bit 65 ¦ Bit 66 ¦ Bit 67 ¦ Bit 68) 0000: 640 0001: 1280 0010: 2560 0011: 3840 0100: 5120 0101: 6400 0110: 7680 1000: 8960 1001: 10240 1010: 20480 1011: 40960 1100: 81920 1101: 163840 Max convolutionally coded bits transmitted (Bit 69 ¦ Bit 70 ¦ Bit 71 ¦ Bit 72) 0000: 640 0001: 1280 0010: 2560 0011: 3840 0100: 5120 0101: 6400 0110: 7680 1000: 8960 1001: 10240 1010: 20480 1011: 40960 1100: 81920 1101: 163840 Max num of simultaneous transport channels (Bit 73 ¦ Bit 74 ¦ Bit 75) 000: 4 001: 8 010: 16 011: 32 Maximum number of simultaneous CCTrCH of DCH type (Bit 76 ¦ Bit 77 ¦ Bit 78 ¦ Bit 79) 0000: 1 0001: 2 0010: 3 0011: 4 0100: 5 0101: 6 0110: 7 0111: 8 Max no of transmitted transport blocks (Bit 80 ¦ Bit 81 ¦ Bit 82 ¦ Bit 83) 0000: 4 0001: 8 0010: 16 0100: 32 0101: 48 0110: 64 0111: 96 1000: 128 1001: 256 1010: 512 Maximum number of TFC (Bit 84 ¦ Bit 85 ¦ Bit 86 ¦ Bit 87) 0000: 16 0001: 32 0010: 48 0011: 64 0100: 96 0101: 128 0110: 256 0111: 512 1000: 1024 Maximum number of TF (Bit 88 ¦ Bit 89 ¦ Bit 90 ¦ Bit 91) 0000: 32 0001: 64 0010: 128 0011: 256 0100: 512 0101: 1024 Bit 92: Support for turbo decoding (False: not supported, True: supported) Bit 93-128: Reserved |
| 802.16 Capabilities | Bit 0: 802.16 2004 Bit 1: 802.16e and Cor-2005 Bit 2-128 Reserved |

# Table YYY: Handover Cause

| Reason Code | Name | Description |
| --- | --- | --- |
| 0 | N/A | Reason not available |
| 1 | LINK_GOING_DOWN | Link connection being lost |
| 2 | UNSUFFICIENT_RESOURCES | Not enough resources in source network to support the service |
| 3 - 255 | Reserved | Reserved |

# Inter-RAT Handovers in 3GPP (informative)

As explained in [1], the 3GPP Inter-RAT handover is based on a few key principles:

- In multiple RAT scenarios, the UE uses a single RAT for all in-progress services.

- Inter-RAT handover decision is made (and the handover command is sent) by the serving RAN. The target RAN may exercise admission control to UEs that are handed over.

- The serving RAN receives information from the target RAN that can be included in the handover command.

Figure 1 shows the message sequence for an inter-SGSN inter-RAT handover between GERAN and UTRAN:

.

Figure 1: Inter-RAT handover from GERAN to UTRAN

The three principles mentioned above are reflected in the following way in this sequence:

- In multiple RAT scenarios, the UE uses a single RAT for all in-progress services.

    - The UE switches RAT between message (7) and message (8).

- Inter-RAT handover decision is made (and the handover command is sent) by the serving RAN. The target RAN may exercise admission control to UEs that are handed over.

    - The source BSC decides that the handover is required and triggers the “PS Handover Required” message (1).

    - The target RNC can perform admission control when receiving the “Relocation Request” (message (3)).

- The serving RAN receives information from the target RAN that can be included in the handover command.

    - Via messages 4, 5, 6 and 7, the target RNC is able to provide a detailed radio configuration to the UE with the UTRAN RRC message “HANDOVER TO UTRAN COMMAND”.

Above three principles are already also applied for handovers to/from the CDMA-2000 non-3GPP RAT.

# References

[1] MOB-070005 [3GPP TSG-RAN Workshop on Radio mobility: ftp://www.3gpp.org/workshop/2007-05-28_Radio_Mobility_non_3GPP/Docs/