| Project | IEEE: 802.21 MIHO <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | Issue: Missing Resource List Definitions |  |
| Date Submitted | June 2007 |  |
| Source(s) | Qiaobing Xie, Juan Carlos Zuniga |  |
| Re: | IEEE 802.21 Ad-hoc in June 2007 |  |
| Abstract | This provides the missing resource list definition |  |
| Purpose | Providing missing resource list definition |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.21. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

Remedy 1:

- Semantics of service primitive

MIH_Net_HO_Candidate_Query.request

(
			DestinationIdentifier,
			SuggestedNewLinkList,
			QueryResourceReportFlag
			)

Parameters:

| Name | Type | Description |
| --- | --- | --- |
| Destination Identifier | MIHF_ID | This identifies a remote MIHF which will be the destination of this request. |
| Suggested New Link List | LIST(LINK_POA_LIST) | A list of PoAs for each link, suggesting the new access networks to which handover initiation should be considered. The access networks towards the top of the list are more preferable than those towards the bottom of the list. |
| Query Resource Report Flag | BOOLEAN | TRUE - Not required to report resource list FALSE - required to report resource list |

- When generated

This primitive is invoked by an MIH User to communicate with a remote MIH User about its intent of handover initiation. Serving PoS requests MN to provide information about resources required to initiate a handover by setting the QueryResourceReportFlag parameter.

Remedy 2:

- Semantics of service primitive

MIH_Net_HO_Candidate_Query.indication (
			SourceIdentifier,
			SuggestedNewLinkList,
			QueryResourceReportFlag
			)

Parameters:

| Name | Type | Description |
| --- | --- | --- |
| Source Identifier | MIHF_ID | This identifies the invoker of this primitive which is a remote MIHF. |
| Suggested New Link List | LIST(LINK_POA_LIST) | A list of PoAs for each link, suggesting the new access networks to which handover initiation should be considered. The access networks towards the top of the list are more preferable than those towards the bottom of the list. |
| Query Resource Report Flag | BOOLEAN | 0 - Not required to report resource list 1 - required to report resource list |

Remedy 3:

1) Replace all occurrences of LINK_RESOURCE_SET, QUERY_RESOURCE_SET QUERY_RESOURCE_LIST with LIST(QOS_PARAM)

2) remove the Editor’s notes related to the resource list in the corresponding section.