| Project | IEEE 802.21 MIHS <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | Notify Mobility Management Protocol supported before handover |  |
| DCN | 21-07-0xxx-00-0000 |  |
| Date Submitted | May 10, 2007 |  |
| Source(s) | Yan PENG, Jing LIU, Junxiang GUO |  |
| Re: | IEEE 802.21 Session #20 in Montreal |  |
| Abstract | This contribution suggests notifying the MN or network about the Mobility Management Protocol supported, which would help handover decision, during the handover preparation phase instead of Link_Up.indication primitive. |  |
| Purpose |  |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that IEEE 802.2 may make this contribution public. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

# Introduction

In Link_Up.indication, the parameter Mobility Management Support is carried indicating the type of Mobility Management Protocol (MMP) supported by the new PoA.

Currently only four values are defined in this parameter: Mobile IPv4 with FA, Mobile IPv4 without FA, Mobile IPv6 and Mobile IPv6 with DHCPv6.

# Problem Statement

When receiving this Link_Up.indication primitive, the MN has established the L2 connection on the specified link interface. To achieve total connectivity of both L2 and L3, Mobility Management Protocol should be used to perform L3 handover, thus achieve L3 connectivity. However, if the MN and candidate network can’t support same L3 Mobility Management Protocol, the MN has to choose another candidate network to attach or rollback to the previous network, which results in increased handover failure ratio, and handover latency.

Supposing the following scenario:

There are three network operators: WiMAX, WLAN and UMTS. The WiMAX operator gets contract with the other two. Between WiMAX and WLAN, any MN which supports Simple IP or PMIP or MIP (v4 or v6) could enjoy seamless mobility service. However, only the MN which supports MIPv4 could enjoy seamless mobility service between WiMAX and UMTS.

Now a MN is served by WiMAX operator. Because it's relative old, this MN can only support Simple IP and PMIP. Although the WiMAX operator have contract with the two networks, while handover decision making, this MN has to choose WLAN to achieve seamless handover.

In addition, there are some other mobility management protocols defined by IETF and widely used in different networks. Four values are not enough for MMP parameter.

# Suggested Remendy

In order to reduce the risk of the handover failure due to the mismatch of MMPs supported by the MN and target network, we propose to notify the MN about the supported MMPs before the MN attaches to the target network, (i.e. before Link_Up.indication). This info of candidate networks would help the MN to choose the best target network. Using MIIS to notify the MMPs supported in candidate network is a good method. MIH handover message is another good alternative.

In the MN-initiated case, the MN can get MMP information of candidate networks through MIH_MN_HO_Candidate_Query response message. Similarly, the serving PoS can notify the MN about MMP information if candidate networks through MIN_Net_HO_Candicate_Query response message.

Also, another three values are added to the definition of MMP parameter: FMIP (fast Mobile IP), HMIP (Hierarchical Mobile IP), and PMIP (Proxy Mobile IP).

[Modify the text as suggested below: Page 131, line 28]

7.4.19.3 MIH_MN_HO_Candidate_Query.response

7.4.19.3.1 Function

7.4.19.3.2 Semantics of service primitive

MIH_MN_HO_Candidate_Query.response	(

DestinationIdentifier,

CurrentLinkIdentifier,

PreferredCandidateLinkList,

AvailableResourceSet,

IPConfigurationMethod,

DHCPServerAddress,

FAAddress,

AccessRouterAddress,

IPAddressInformationStatus,

MobilityManagementSupported,

Status

)

Local or Remote: Remote

MIHF (Mobile Node) <- MIHF (Network)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Destination Identifier | MIHF ID | N/A | This identifies a remote MIHF which will be the destination of this response. |
| Current Link Identifier | LINK ID | N/A | This identifies the current access network over which the command needs to be sent. This is valid only for remote commands which need to be sent to remote MIHF. |
| Preferred Candidate Link List | LIST | N/A | A list of {LINK ID, PoA List} pairs, as defined in Table46, identifying candidate networks to which handover needs to be initiated. The list is sorted from most preferred first to least preferred last. |
| Available Resource Set | LIST | N/A | Containing a set of LinkIdentifier parameters and their corresponding available resource on candidate network(s): LinkIdentifier AvailableResourceList |
| IP Configuration Method | BITMAP(32) | 0-31 | IP configuration Method on PreferredCandidatePoA List Bit 0: IPv4 static configuration Bit 1: IPv4 dynamic configuration (DHCPv4) Bit 2. Mobile IPv4 with FA (FA-CoA) Bit 3: Mobile IPv4 without FA (Co-located CoA) Bit 4~10: (Reserved for IPv4 address configuration) Bit 11: IPv6 stateless address configuration Bit 12: IPv6 stateful address configuration (DHCPv6) Bit 13: IPv6 manual configuration Bit 14: No configuration allowed Bit 15~31: (Reserved) |
| DHCP Server Address | IP ADDRESS | N/A | IP address of candidate DHCP Server. It is only included when dynamic address configuration is supported |
| FA Address | IP ADDRESS | N/A | IP address of current Foreign Agent. It is only included when mobile node is using Mobile IPv4 |
| Access Router Address | IP ADDRESS | N/A | IP address of current Access Router. It is only included when mobile node is using IPv6 |
| IPAddressInformationStatus | BITMAP(8) | 0-8 | Bit 0: IP configuration Method is not available Bit 1: DHCP Server address is not available Bit 2: FA address is not available Bit 3: Access Router Address is not available Bit 4: No information is provided due to accessibility of same entity (FA, Access Router, DHCP Server, etc.) Bit 5-7: Reserved |
| Mobility Management Supported | BITMAP(16) | N/A | This indicates the type of Mobility Management Protocol supported by candidate PoA: Bit 0: Mobile IPv4 with FA(FA-CoA) Bit 1: Mobile Ipv4 without FA(Co-located CoA) Bit 2: Mobile IPv6 Bit 3: Mobile IPv6 with DHCPv6 Bit 4: FMIP Bit 5: HMIP Bit 6: PMIP Bit 7-15: (Reserved) |
| SStatus | ENUMERATED | 0-4 | Status of operation: 0 Success 1 Unspecified Failure 2 Rejected 3 Authorization Failure 4 Network Error |

[Modify the text as suggested below: Page 136, line 1]

7.4.19.4 MIH_MN_HO_Candidate_Query.confirm

7.4.19.4.1 Function

7.4.19.4.2 Semantics of service primitive

MIH_MN_HO_Candidate_Query.confirm	(

SourceIdentifier,,

CurrentLinkIdentifier,

PreferredCandidateLinkList,

AvailableResourceSet,

IPConfigurationMethod,

DHCPServerAddress,

FAAddress,

AccessRouterAddress,

IPAddressInformationStatus,

MobilityManagementSupported,

Status

)

Local or Remote: Both

MIHF (Network) <> MIHF (Network)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Source Identifier | MIHF ID | N/A | This identifies the invoker of this primitive which can be a remote MIHF. |
| Current Link Identifier | LINK ID | N/A | This identifies the current access network over which the command needs to be sent. This is valid only for remote commands which need to be sent to remote MIHF. |
| Preferred Candidate Link List | LIST | N/A | A list of {LINK ID, PoA List} pairs, as defined in Table46, identifying candidate networks to which handover needs to be initiated. The list is sorted from most preferred first to least preferred last. |
| Available Resource Set | LIST | N/A | Containing a set of LinkIdentifier parameters and their corresponding available resource on candidate network(s): LinkIdentifier AvailableResourceList |
| IP Configuration Method | BITMAP(32) | 0-31 | IP configuration Method on PreferredCandidatePoA List Bit 0: IPv4 static configuration Bit 1: IPv4 dynamic configuration (DHCPv4) Bit 2. Mobile IPv4 with FA (FA-CoA) Bit 3: Mobile IPv4 without FA (Co-located CoA) Bit 4~10: (Reserved for IPv4 address configuration) Bit 11: IPv6 stateless address configuration Bit 12: IPv6 stateful address configuration (DHCPv6) Bit 13: IPv6 manual configuration Bit 14: No configuration allowed Bit 15~31: (Reserved) |
| DHCP Server Address | IP ADDRESS | N/A | IP address of candidate DHCP Server. It is only included when dynamic address configuration is supported |
| FA Address | IP ADDRESS | N/A | IP address of current Foreign Agent. It is only included when mobile node is using Mobile IPv4 |
| Access Router Address | IP ADDRESS | N/A | IP address of current Access Router. It is only included when mobile node is using IPv6 |
| IPAddressInformationStatus | BITMAP(8) | 0-8 | Bit 0: IP configuration Method is not available Bit 1: DHCP Server address is not available Bit 2: FA address is not available Bit 3: Access Router Address is not available Bit 4: No information is provided due to accessibility of same entity (FA, Access Router, DHCP Server, etc.) Bit 5-7: Reserved |
| Mobility Management Supported | BITMAP(16) | N/A | This indicates the type of Mobility Management Protocol supported by candidate PoA: Bit 0: Mobile IPv4 with FA(FA-CoA) Bit 1: Mobile Ipv4 without FA(Co-located CoA) Bit 2: Mobile IPv6 Bit 3: Mobile IPv6 with DHCPv6 Bit 4: FMIP Bit 5: HMIP Bit 6: PMIP Bit 7-15: (Reserved) |
| SStatus | ENUMERATED | 0-4 | Status of operation: 0 Success 1 Unspecified Failure 2 Rejected 3 Authorization Failure 4 Network Error |

[Modify the text as suggested below: Page 125, line 56]

7.4.18.1 MIH_Net_HO_Candidate_Query.request

7.4.18.1.1 Function

7.4.18.1.2 Semantics of service primitive

MIH_Net_HO_Candidate_Query.request 				(

DestinationIdentifier,

SuggestedNewLinkList,

HandoverMode,

OldLinkAction,

QueryResourceList

MobilityManagementSupported

)

Local or Remote: Remote

MIHF (Network) -> MIHF (Mobile Node)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Destination Identifier | MIHF ID | N/A | This identifies a remote MIHF which will be the destination of this request. |
| Suggested New Link List | LIST | N/A | A list of {Link Identifier, PoA List} pairs, as defined in Table , suggesting the new access networks to which handover initiation should be considered. The access networks towards the top of the list are more preferable than those towards the bottom of the list. |
| Handover Mode | ENUMERATED | 0-1 | The handover mode may influence the manner in which links are prepared for handover. 0 Make-before-Break 1 Break-before-Make |
| Old Link Actions | BITMAP (32) | N/A | Specifies the suggested action on link during handover. Combination of the following choices are allowed. Bit 0: LINK_DISCONNECT Bit 1: LINK_LOW_POWER Bit 2: LINK_POWER_DOWN Bit 3: LINK_NO_ACTION Bit 4: LINK_RESOURCE_RETAIN Bit 5: DATA_FORWARDING_REQUEST Bit 6: LINK_POWER_UP Bit 7-31: (Reserved) |
| Query Resource List | LIST | N/A | List of resources to be queried at the new can didate network. (Editor’s note: need input) |
| Mobility Management Supported | BITMAP(16) | N/A | This indicates the type of Mobility Management Protocol supported by candidate PoA: Bit 0: Mobile IPv4 with FA(FA-CoA) Bit 1: Mobile Ipv4 without FA(Co-located CoA) Bit 2: Mobile IPv6 Bit 3: Mobile IPv6 with DHCPv6 Bit 4: FMIP Bit 5: HMIP Bit 6: PMIP Bit 7-15: (Reserved) |

[Modify the text as suggested below: Page 125, line 56]

MIH_Net_HO_Candidate_Query.indication

Function

This primitive is generated by an MIHF to indicate to an upper layer entity that an MIH_Net_HO_Candidate_Query request message is received from a peer MIHF.

Semantics of service primitive

The parameters of the primitive are as follows:

MIH_Net_HO_Candidate_Query.indication (

SourceIdentifier,

SuggestedNewLinkList,

HandoverMode,

OldLinkAction,

QueryResourceList

MobilityManagementSupported

)

| Name | Type | Valid range | Description |  |
| --- | --- | --- | --- | --- |
| Source Identifier | MIHF ID | N/A | This identifies the invoker of this primitive which can be a remote MIHF. |  |
| Suggested New Link List | LIST | N/A | A list of {LINK ID, PoA List} pairs, as defined in Table , suggesting the new access networks to which handover initiation should be considered. The access networks towards the top of the list are more preferable than those towards the bottom of the list. |  |
| Handover Mode | ENUMER ATED | 0-1 | The handover mode may influence the manner in which links are prepared for handover. 0 Make-before-Break 1 Break-before-Make |  |
| Old Link Actions | BITMAP (32) | N/A | Specifies the suggested action on link during handover. Combination of the following choices are allowed. Bit 0: LINK_DISCONNECT Bit 1: LINK_LOW_POWER Bit 2: LINK_POWER_DOWN Bit 3: LINK_NO_ACTION Bit 4: LINK_RESOURCE_RETAIN Bit 5: DATA_FORWARDING_REQUEST Bit 6: LINK_POWER_UP Bit 7-31: (Reserved) |  |
| Query Resource List | LIST | N/A | List of resources to be queried at the new can didate network. |  |
| Mobility Management Supported | BITMAP(16) | N/A | This indicates the type of Mobility Management Protocol supported by candidate PoA: Bit 0: Mobile IPv4 with FA(FA-CoA) Bit 1: Mobile Ipv4 without FA(Co-located CoA) Bit 2: Mobile IPv6 Bit 3: Mobile IPv6 with DHCPv6 Bit 4: FMIP Bit 5: HMIP Bit 6: PMIP Bit 7-15: (Reserved) |  |

[Modify the text as suggested below: Page 85, line 1]

7.3.4 Link_Up indication

7.3.4.1 Function

7.3.4.2 Semantics of service primitive

The primitive parameters are as follows:

Link_Up.indication(

LinkIdentifier,

MacOldAccessRouter,

MacNewAccessRouter,

IPRenewalFlag,

Mobility Management Support

)

Parameters:

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Link Identifier | LINK ID | N/A | Identifier of the link associated with the event |
| MacOldAccessRouter | MAC ADDRESS | N/A | (Optional)MAC Address of old Access Router |
| MacNewAccessRouter | MAC ADDRESS | N/A | (Optional)MAC Address of new Access Router |
| IPRenewalFlag | BOOLEAN | N/A | (Optional)Indicates whether the MN shall change IP Address in the new PoA 0: Change Required 1: Change Not required |
| Mobility Management Support | BITMAP(816) | N/A | (Optional) Indicates the type of Mobility Management Protocol supported by the new PoA Bit 0: Mobile IPv4 with FA (FA-CoA) Bit 1: Mobile IPv4 without FA (Co-located CoA) Bit 2: Mobile IPv6 Bit 3: Mobile IPv6 with DHCPv6 Bit 4: FMIP Bit 5: HMIP Bit 6: PMIP Bit 7-15: (Reserved) |

[Modify the text as suggested below: Page 50 Line 48]

| Access Network Specific Information Elements |  |  |  |
| --- | --- | --- | --- |
| 2.1 | TYPE_IE_ACCESS_NETWORK_IDENTIFIER | Identifier for the access network. | 6.4.6.2.1 |
| 2.2 | TYPE_IE_NETWORK_SYSTEM_ID | SSID to be used for this network. Only applicable when the network type is IEEE 802.11. | 6.4.6.2.2 |
| 2.3 | TYPE_IE_ROAMING_PARTNERS | Roaming Partners. Network Operators with which the current network operator has direct roaming agreements. | 6.4.6.2.3 |
| 2.4 | TYPE_IE_COST | Cost. Indication of cost for service or network usage. | 6.4.6.2.4 |
| 2.5 | TYPE_IE_NETWORK_SECURITY | Security characteristics of the link layer. | 6.4.6.2.5 |
| 2.6 | TYPE_IE_NETWORK_QOS | QoS characteristics of the link layer. | 6.4.6.2.6 |
| 2.7 | TYPE_IE_NETWORK_DATA_RATE | Data Rate. The minimum and maximum value of data rate supported by the link layer of the access network. | 6.4.6.2.7 |
| 2.8 | TYPE_IE_NETWORK_IP_CONFIG_METHODS | IP Configuration Methods supported by the access network. | 6.4.6.2.8 |
| 2.9 | TYPE_IE_NETWORK_CAPABILITIES | Bitmap of access network capabilities | 6.4.6.2.9 |
| 2.10 | TYPE_IE_NETWORK_MOBILITY_MANAGEMENT | The type of Mobility Management Protocol supported by the new PoA | 6.4.6.2.10 |

[Add the text as suggested below: Page 59, line 64]

6.4.6.2.10  Network mobility management

This indicates the type of Mobility Management Protocol supported by the new PoA.

| Type= TYPE_IE_NETWORK_MOBILITY_MANAGEMENT | Length= 2 octets |
| --- | --- |
| Network Mobility Management Supported Bitmap[2 octets] (see Table XX) |  |

Table XX - Network mobility management supported bitmap encoding

| Syntax | Length (octets) | Notes |
| --- | --- | --- |
| Network Mobility Management Supported Bitmap | 2 | This indicates the type of Mobility Management Protocol supported by the new PoA: Bit 0: Mobile IPv4 with FA(FA-CoA) Bit 1: Mobile Ipv4 without FA(Co-located CoA) Bit 2: Mobile IPv6 Bit 3: Mobile IPv6 with DHCPv6 Bit 4: FMIP Bit 5: HMIP Bit 6: PMIP Bit 7-15: (Reserved) |