| Project | IEEE 802.21 MIHS <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | Proposal for a new IE |  |
| DCN | 21-07-0xxx-00-0000 |  |
| Date Submitted | May 13, 2007 |  |
| Source(s) | Gabor Bajko Sreemanthula Srinivas Nokia |  |
| Abstract | The contribution proposes a new IE, Location Configuration Protocol (LCP) IE. The IE would allow the STA to query the network and find out the supported LCPs in the given network. |  |
| Purpose | approval |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that IEEE 802.2 may make this contribution public. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

# Introduction

Precise location information knowledge of the STA is required for placing an Emergency Call. A STA without a location determination technique natively supported (eg GPS) needs to query the network to find out its location.

# Problem Statement

Currently there are a number of different Location Configuration Protocols supported in different networks.

Example of (network based) LCPs:

- Link layer specific ones: LLDP[-MED], U-TDoA, D-TDoA

- Link layer agnostic ones: DHCP, OMA SUPL, RELO, HELD (HTTP based)

Currently the STA has no means to find out which Location Configuration Protocol is supported in a specific network.

# Suggested Remendy

[Modify the text as suggested below: Page 50, line 49]

Add a new row into Table 7:

- No: 2.10

- Name of Information Element: TYPE_IE_LIST_SUPPORTED_LCP

- Description: List of Location Configuration Protocols supported by the access network

- Reference: 6.4.6.2.10

6.4.6.2.10 List of Supported LCP

| TYPE = TYPE_IE_LIST_SUPPORTED_LCP | Length = variable |
| --- | --- |
| Supported LCP #1 (table 15) |  |
| Supported LCP #2 (optional) |  |
| …………… |  |
| Supported LCP #k (optional) |  |

| Syntax | Length | note |
| --- | --- | --- |
| Supported LCP | 1 octet | Values represent LCPs: 0: NULL 1: LLDP 2: LbyR with LLDP 3-10: Reserved 11: LLDP-MED 12: LbyR with LLDP-MED 13-20: Reserved 21: U-TDoA 22: D-TDoA 23-30: Reserved 31: DHCP 32: LbyR with DHCP 33-40: Reserved 41: OMA SUPL 42-50: Reserved 51: HELD 52: LbyR with HELD 53-255: Reserved |

All the supported LCPs of a network are returned in a form of  a number of 1 octet concatenated string.

4. Abbreviations and Acronyms

[Modify the text as suggested below: Page 11, line 47]

Insert into the abbreviations list a new line:

LbyR	Location by reference

LCP	Location Configuration Protocol