| Project | IEEE 802.21 MIHO <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | MIH_Get_Information Editorial Fix |  |
| Date Submitted | June 10, 2007 |  |
| Source(s) | Yoshihiro Ohba, Yuu-Heng Alice Cheng, Eunah Kim and Junghoon Jee |  |
| Re: | IEEE 802.21 Session #20.5 in June 2007 |  |
| Abstract | This document describes editorial fix around MIH_Get_Information primitives and messages, as part of comment #5412. This contribution is harmonized with 21-07-0232. |  |
| Purpose | LB#1d comment resolution |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.21. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

To be consistent, the following editorial changes are needed in MIH_Get_Information primitives and messages.

[1] Revise table in Section 7.4.25.1.2 as follows:

| Name | Type | Description |
| --- | --- | --- |
| Destination Identifier | MIHF_ID | The local MIHF or a remote MIHF which will be the destination of this request. |
| Info Query Binary Data List | LIST(INFO_QUERY_BINA RY_DATA) | (Optional) A list of binary queries. The order of the queries in the list identifies the priority of the query. The first query has the highest priority to be processed by MIIS. |
| Info Query RDF Data List | LIST(INFO_QUERY_RDF_ DATA) | (Optional) A list of RDF queries. The order of the queries in the list identifies the priority of the query. The first query has the highest priority to be processed by MIIS. |
| Info Query RDF Schema URL | NULL | (Optional) An RDF Schema URL query. |
| Info Query RDF Schema List | LIST(INFO_QUERY_RDF_ SCHEMA) | (Optional) A list of RDF schema queries. The order of the queries in the list identifies the priority of the query. The first query has the highest priority to be processed by MIIS. |
| Max Response Size | INTEGER(2) | (Optional) This field specifies the maximum size of InfoQueryParameters Info Response parameters (i.e., Info Response Binary Data List, Info Response RDF Data List, Info Response RDF Schema URL and Info Response RDF Schema List) in MIH_Get_Information response primitive in octets. If this field is not specified, the maximum size is set to 65,535. The actual maximum size forced by the IS server may be smaller than that specified by the IS client. |

[2] Revise Section 7.4.25.1.3 as follows:

7.4.25.1.3 When generated

This primitive is generated by an MIH User that is seeking to retrieve information.

At least one of the following parameters should be specified:

- Info Query Binary DataList

- Info Query RDF Data List

- Info Query RDFSchemaURL

- Info Query RDF Schema List

The order of the queries in each of Info Query Binary Data List, Info Query RDF Data List and Info Query RDF Schema List parameters identifies the priority of the query. The first query has the highest priority to be processed by MIIS.

[3] Revise table in Section 7.4.25.2.2 as follows:

| Name | Type | Description |
| --- | --- | --- |
| Source Identifier | MIHF_ID | Shall contain the MIHF ID of the node that invoked MIH_GET_Information.request. |
| Info Query Binary Data List | LIST(INFO_QUERY_BINA RY_DATA) | (Optional) A list of binary queries. The order of the queries in the list identifies the priority of the query. The first query has the highest priority to be processed by MIIS. |
| Info Query RDF Data List | LIST(INFO_QUERY_RDF_ DATA) | (Optional) A list of RDF queries. The order of the queries in the list identifies the priority of the query. The first query has the highest priority to be processed by MIIS. |
| Info Query RDF Schema URL | NULL | (Optional) An RDF Schema URL query. |
| Info Query RDF Schema List | LIST(INFO_QUERY_RDF_ SCHEMA) | (Optional) A list of RDF schema queries. The order of the queries in the list identifies the priority of the query. The first query has the highest priority to be processed by MIIS. |
| Max Response Size | INTEGER(2) | (Optional) This field specifies the maximum size of InfoQueryParameters Info Response parameters (i.e., Info Response Binary Data List, Info Response RDF Data List, Info Response RDF Schema URL and Info Response RDF Schema List) in MIH_Get_Information response primitive in octets. If this field is not specified, the maximum size is set to 65,535. The actual maximum size forced by the IS server may be smaller than that specified by the IS client. |

[4] Revise Section 7.4.25.2.3 as follows:

7.4.25.2.3 When generated

This primitive is generated by the MIHF on receiving an MIH_Get_Information request message from a

peer MIHF.

The order of the queries in each of Info Query Binary Data List, Info Query RDF Data List and Info Query RDF Schema List parameters identifies the priority of the query. The first query has the highest priority to be processed by MIIS. Thus the order of the queries must be maintained as indicated by the request message.

[5] Revise table in Section 7.4.25.3.2 as follows:

| Name | Type | Description |
| --- | --- | --- |
| Destination Identifier | MIHF_ID | The local MIHF or a remote MIHF which will be the destination of this response. |
| Info Response Binary Data List | LIST(INFO_RSP_BINARY_ DATA) | (Optional) A list of binary query responses. The list may be sorted from most preferred first to least preferred last. |
| Info Response RDF Data List | LIST(INFO_RSP_RDF_DAT A) | (Optional) A list of RDF query responses. The list may be sorted from most preferred first to least preferred last. |
| Info Response RDF Schema URL | LIST(INFO_RSP_RDF_URL ) | (Optional) A list of RDF Schema URL. The list may be sorted from most preferred first to least preferred last. |
| Info Response RDF Schema List | LIST(INFO_RSP_RDF_SCH EMA) | (Optional) A list of RDF schema query responses. The list may be sorted from most preferred first to least preferred last. |
| Status | STATUS | Status of operation. The response lists contains meaningful data if and only if the status is ‘0’ |

[6] Revise Section 7.4.25.3.3 as follows:

7.4.25.3.3 When generated

This primitive is generated by an MIH User in response to an received MIH_Get_Information.indication primitive.

When the size of the Info Response parameters exceeds the maximum size specified in the Max Response Size parameter from MIH_Get_Information.indication primitive, one or more of the lower order list elements in Info Response parameters (i.e., Info Response Binary Data List, Info Response RDF Data List, Info Response RDF Schema URL and Info Response RDF Schema List) may be omitted.

[7] Revise Section 7.4.25.3.4 as follows:

7.4.25.3.4 Effect on receipt

The MIH User that requested the information tries to interpret the InfoQueryResponse and take suitable

action. However, if Status does not indicate “Success”, the recipient shall ignore any other returned values

and, instead, shall perform appropriate error handling.

The recipient will return an MIH_Get_Information response message to the designated MIIS client.

[8] Revise table in Section 7.4.25.4.2 as follows:

| Name | Type | Description |
| --- | --- | --- |
| Source Identifier | MIHF_ID | Shall contain the MIHF ID of the node that invoked MIH_GET_Information.response. |
| Info Response Binary Data List | LIST(INFO_RSP_BINARY_ DATA) | (Optional) A list of binary query responses. The list may be sorted from most preferred first to least preferred last. |
| Info Response RDF Data List | LIST(INFO_RSP_RDF_DAT A) | (Optional) A list of RDF query responses. The list may be sorted from most preferred first to least preferred last. |
| Info Response RDF Schema URL | LIST(INFO_RSP_RDF_URL ) | (Optional) A list of RDF Schema URL. The list may be sorted from most preferred first to least preferred last. |
| Info Response RDF Schema List | LIST(INFO_RSP_RDF_SCH EMA) | (Optional) A list of RDF schema query responses. The list may be sorted from most preferred first to least preferred last. |
| Status | STATUS | Status of operation. The response lists contains meaningful data if and only if the status is ‘0’ |

[9] Revise Section 7.4.25.4.4 as follows:

7.4.25.4.4 Effect on receipt

The MIH User that requested the information tries to interpret the InfoQueryResponse Info Response parameters (i.e., Info Response Binary Data List, Info Response RDF Data List, Info Response RDF Schema URL and Info Response RDF Schema List) and take suitable action. However, if Status does not indicate “Success”, the recipient shall ignore any other returned values

and, instead, shall perform appropriate error handling.

When the size of the Info Response parameters exceeds the maximum size specified in the Max Response Size parameter from MIH_Get_Information.request primitive, one or more of the lower order list elements in Info Response parameters may be omitted.

[10] Revise Section 8.6.4.1 as follows:

8.6.4.1 MIH_Get_Information request

This message is used by an MIHF to retrieve a set of Information Elements provided by the information service. A single MIH_Get_Information request message can carry multiple queries of a combination of

Binary, RDF_Data, RDF_Schema_URL and RDF_Schema types. However, there should be at least one

query specified. The order of the queries identifies the priority of the query. The first query has the highest

priority to be processed by MIIS.

MIH Header Fixed Fields (SID=4, Opcode=1, AID=1)

Source Identifier = sending MIHF ID (Source MIHF ID TLV)

Destination Identifier = receiving MIHF ID (Destination MIHF ID TLV)

InfoQueryBinaryDataList (optional) (Info Query Binary Data List TLV)

InfoQueryRDFDataList (optional) (Info Query RDF Data List TLV)

InfoQueryRDFSchemaURL (optional) (Info Query RDF Schema URL TLV)

InfoQueryRDFSchemaList (optional) (Info Query RDF Schema List TLV)

MaxResponseSize (optional) (Max Response Size TLV)

[11] Revise Section 8.6.4.2 as follows:

8.6.4.2 MIH_Get_Information response

This is used as a response to the MIH_Get_Information request message. In one response message, one or

more results can be returned. The order of the returned results corresponds to the order of the multiple queries provided in the MIH_Get_Information request message. When the result in MIH_Get_Information

response message size exceeds the maximum size specified in the Max Response Size TLV from

MIH_Get_Information request message, one or more of the lower order response TLVs may be omitted.

MIH Header Fixed Fields (SID=4, Opcode=2, AID=1)

Source Identifier = sending MIHF ID (Source MIHF ID TLV)

Destination Identifier = receiving MIHF ID (Destination MIHF ID TLV)

InfoResponseBinaryDataList (optional) (Info Response Binary List TLV)

InfoResponseRDFDataList (optional) (Info Response RDF Data List TLV)

InfoResponseRDFSchemaURLList (optional) (Info Response RDF Schema URL List TLV)

InfoResponseRDFSchemaList (optional) (Info Response RDF Schema List TLV)

Status (Status TLV)