| Project | IEEE 802.21 MIHO <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | MIH_Registration Nits |  |
| Date Submitted | September 16, 2007 |  |
| Source(s) | Yoshihiro Ohba, Victor Fajardo and Yuu-Heng Alice Cheng |  |
| Re: | IEEE 802.21 Session #22 in September 2007 |  |
| Abstract | This document proposes a new parameter to MIH_Registration for optimization purpose. |  |
| Purpose | Sponsor Ballot comment resolution |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.21. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

# Problem Description

MIH_Register command is defined for an MN to register with a PoS so that the PoS can subscribe to the MN for event notification as well as send commands to the MN.

The current MIH_Register.request primitive carries the following parameters:

- DestinationIdentifier (The MIHF_ID of the recipient)

- RequestCode (a BOOLEAN value to indicate whether this is registration or re-registration)

On the other hand, in order for the PoS to subscribe to the MN for event notifications, the PoS needs to know about the MN’s LINK_ID on which it wants to obtain event notifications. As a result, once MIH_Register command is completed, the PoS needs to issue an MIH_Get_Link_Paramers command with an empty list of LinkIdentifeirList to know the list of LINK_IDs of the MN to be able to issue an MIH_Event_Subscribe command.

# Approach

Add one parameter to MIH_Register.request primitive and MIH_Register request message:

- LinkIdentifierList  (type: LIST(LINK_ID), carrying a list of link identifiers of the MN)

With this parameter, the PoS obtains the list of link identifiers of the MN during MIH registration. Therefore, after the registration, the PoS can immediately issue an MIH_Event_Subscribe command to the MN without issuing an MIH_Get_Link_Paramers command.

# Proposed Change

[1] Change Clause 7.4.2.1.2 as follows:

MIH_Register.request (

DestinationIdentifier,

LinkIdentifierList,

RequestCode

)

| Name | Type | Description |
| --- | --- | --- |
| Destination Identifier | MIHF_ID | This identifies a remote MIHF which will be the destination of this request. |
| Link Identifier List | LIST(LINK_ID) | List of local link identifiers. |
| Request Code | REG_REQUEST_CODE | Registration request code. Depending on the request code, the MIH User can choose to either register or re-register with the local or remote MIHF. |

[2] Change Clause 7.4.2.2.2 as follows:

MIH_Register.indication (

SourceIdentifier,

LinkIdentifierList,

RequestCode

)

| Name | Type | Description |
| --- | --- | --- |
| Source Identifier | MIHF_ID | This identifies the invoker of this primitive which is a remote MIHF. |
| Link Identifier List | LIST(LINK_ID) | List of link identifiers of the remote MIHF. |
| Request Code | REG_REQUEST_CODE | Registration request code. Depending on the request code, the MIH User can choose to either register or re-register with the local or remote MIHF. |

[3] Add the following TLV in MIH_Register request message format in Clause 8.6.1.3:

LinkIdentifierList (Link Identifier List TLV)