| Project | IEEE 802.21 MIHS <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | IS Response Updates |  |
| DCN | 21-07-0232-00-0000 |  |
| Date Submitted | 06-09-2007 |  |
| Source(s) | Srinivas Sreemanthula, Mika Forssell, Janne Tervonen |  |
| Re: | IEEE 802.21 Adhoc Session #NN in Korea |  |
| Abstract | IS Response updates |  |
| Purpose |  |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that IEEE 802.21 may make this contribution public. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as stated in Section 6 of the IEEE-SA Standards Board bylaws <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and in Understanding Patent Issues During IEEE Standards Development http://standards.ieee.org/board/pat/faq.pdf |  |

# Problem Statement

Currently, the information provides in the network operator or roaming partners in information services have no order by which the terminal or network node needs to interpret the information from a preferential view. It would be beneficial for the network and the terminal to sort the list in the order of preference from the information server point of view.

Summary of the Proposed Solution

When network receives MIH_Get_Information.request message from mobile node, network may send information included into MIH_Get_Information.response message in prioritized order to make preference for access networks the mobile node uses. In case prioritized list is given, mobile node shall consider the list when selecting access network to use.

# Proposed Changes to the Current Specification

7.4.25.3.2 Semantics of service primitive

MIH_Get_Information.response (

DestinationIdentifier,

InfoResponseBinaryDataList,

InfoResponseRDFDataList,

InfoResponseRDFSchemaURL,

InfoResponseRDFSchemaList,

Status

)

Parameters:

| Name | Type | Description |
| --- | --- | --- |
| Destination Identifier | MIHF_ID | The local MIHF or a remote MIHF which will be the destination of this response |
| Info Response Binary Data List | LIST(INFO_RSP_BINARY_ DATA) | A list of binary query responses. The list may be sorted from most preferred first to least preferred last. |
| Info Response RDF Data List | LIST(INFO_RSP_RDF_DAT A) | A list of RDF query responses. The list may be sorted from most preferred first to least preferred last. |
| Info Response RDF Schema URL | LIST(INFO_RSP_RDF_URL ) | A list of RDF Schema URL. The list may be sorted from most preferred first to least preferred last. |
| Info Response RDF Schema List | LIST(INFO_RSP_RDF_SCH EMA) | A list of RDF schema query responses. The list may be sorted from most preferred first to least preferred last. |
| Status | STATUS | Status of operation. The response lists contains meaningful data if and only if the status is ‘0’. |

7.4.25.3.4 Effect on receipt

The MIH User that requested the information shall interpret the InfoQueryResponse and take suitable

action. In case Info Response is sorted, the MIH User that requested the information shall consider the order of information when using the information. However, if Status does not indicate “Success”, the recipient shall ignore any other returned values

and, instead, shall perform appropriate error handling.

7.4.25.4.4 Effect on receipt

The MIH User that requested the information shall interpret the InfoQueryResponse and take suitable action. In case Info Response is sorted, the MIH User that requested the information shall consider the order of information when using the information. However, if Status does not indicate “Success”, the recipient shall ignore any other returned values and, instead, shall perform appropriate error handling.

C.2.9.1 Info Response Binary

Rules for generating returned IEs:

Upon receipt of a binary query, the information server will:

1) create the list of neighboring access network information for the given location;

— If a NETWORK_TYPE_INCLUSION is provided in the query, include only the information of the neighboring access networks of the network type(s) indicated in the NETWORK_TYPE_INCLUSION. Otherwise, include information of all available neighboring

access networks for the given location.

— If a NETWORK_INCLUSION is provided in the query, include only the information of the neighboring access network(s) indicated in the NETWORK_INCLUSION. Otherwise, include information of all available neighboring access networks for the given location.

2) if no REPORTING_TEMPLATE is given in the query, send the list of neighboring access network information in a List of Neighboring Access Networks Container in an Information Response.

3) if a REPORTING_TEMPLATE is given in the query, extract the requested IE(s)/Containers from the list of neighboring access network information using the rules described for

REPORTING_TEMPLATE and send them in an Information Response.

Upon receipt of a Response Information containing sorted information, the mobile node will consider the order of the information when using the information.