| Project | IEEE 802.21 MIHS <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | Loose Ends HO Commands |  |
| DCN | 21-07-0234-01-0000 |  |
| Date Submitted | 06-14-2007 |  |
| Source(s) | Jari Mustajarvi, Mika Forssell, Janne Tervonen, Srinivas Sreemanthula Yoshihiro Ohba |  |
| Re: | IEEE 802.21 Adhoc Session #NN in Korea |  |
| Abstract | Loose ends in ho commands |  |
| Purpose |  |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that IEEE 802.21 may make this contribution public. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as stated in Section 6 of the IEEE-SA Standards Board bylaws <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and in Understanding Patent Issues During IEEE Standards Development http://standards.ieee.org/board/pat/faq.pdf |  |

Open Issues

- Issue: PoA address is defined as a MAC address which is very 802 specific. Need to include other link types as well. Editor’s notes: issue 17 and 18.

REMEDY:

[1] Add the following data type to Table 16:

| Type Name | Derived From | Definition | Valid Range |
| --- | --- | --- | --- |
| LINK_ADDRESS | CHOICE( MAC_ADDRESS, OTHER_L2_ADDRESS) | A data type to represent an address of any link-layer. | N/A |
| OTHER_L2_ADDRESS | OCTET_STRING | A data type to present a link-layer address other than an MAC address, for example, SSID, PLMN_ID, 3GPP_CELL (CELL_ID + (2G: ARFCN+BSIC, Band / 3G: ARFCN + PSC)). | N/A |

[2] Change the LINK_ID and LINK_TUPLE_ID definition in Annex C.2.2 to:

| Type Name | Derived From | Definition | Valid Range |
| --- | --- | --- | --- |
| LINK_ID | SEQUENCE( LINK_TYPE LINK_ADDRESS) | The identifier of a link that is not associated with any PoA. The LINK_ADDRESS contains a link-layer address of MN. This may be used for the current link being used by MN or the link that was used by MN before handover. | N/A |
| LINK_TUPLE_ID | SEQUENCE( LINK_ID, CHOICE( LINK_ADDRESS, NULL)) | The identifier of a link that may be associated with a PoA. The optional LINK_ADDRESS contains a link-layer address of PoA. |  |

- Parameters:

- Parameters: