| Project | IEEE: 802.21 MIHO <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | Text clarification to Link_PDU_Transmit_Status primitive description (Assignment 12) |  |
| Date Submitted | June 2007 |  |
| Source(s) | Qiaobing Xie |  |
| Re: | IEEE 802.21 Ad-hoc in June 2007 |  |
| Abstract | Text clarification to Link_PDU_Transmit_Status primitive description (Assignment 12) |  |
| Purpose | This proposes text clarification to Link_PDU_Transmit_Status primitive description (Assignment 12) |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.21. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

Assignment #12 (5203, 5260): Link PDU - Need the text to be clearer. Also need to understand how the interoperability will be achieved.

===== Proposed text change to 7.3.10.1 (D5.1, page 70, line 22) ======

Link_PDU_Transmit_Status indicates the transmission status of a higher layer PDU by the link layer. A success status indicates that a higher layer PDU has been successfully delivered from the ARQ module of the link layer in the local node to the ARQ module in the link layer in the peer node. A higher layer intermediate buffer management entity could use this indication to flush the delivered  PDU from its buffer. A failure status indicates that  the higher layer PDU identified in the indication was not delivered successfully from the ARQ module of the link layer in the current node to the ARQ module in the link layer in the peer node.

A Packet Identifier is expected to be passed alongside when each higher layer PDU is sent from the higher layer to the link for transmission, and the link shall include this Packet Identifier in its Link_PDU_Transmit_Status indication to identify the PDU being reported. This standard does not define the semantics of the Packet Identifier but it is expected that the MIH User(s) which subscribes for receiving Link_PDU_Transmit_Status indications from a particular link understand the semantics of the Packet Identifier.

===End proposed text change===