| Project | IEEE 802.21 MIHS <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | Data Types for Mobility Management and Fast Handover Protocol |  |
| DCN | 21-07-0255-00-0000 |  |
| Date Submitted | July 9, 2007 |  |
| Source(s) | Junghoon Jee, Eunah Kim, Changmin Park, Srinivas.Sreemanthula | Voice: +82-42-860-5126 Fax: +82-42-861-5404 {jhjee, eakim, cmpark}@etri.re.kr Srinivas.Sreemanthula@nokia.com |
| Re: | IEEE 802.21 Session #21 in San Francisco |  |
| Abstract | This contribution proposes an amendment for data types of mobility management protocol and a new data type for fast handover protocol. |  |
| Purpose | Adoption |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that IEEE 802.21 may make this contribution public. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as stated in Section 6 of the IEEE-SA Standards Board bylaws <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and in Understanding Patent Issues During IEEE Standards Development http://standards.ieee.org/board/pat/faq.pdf |  |

1. Overview

The IEEE P802.21/D06.00 specifies the data types for IP configuration in the B.2.6. That existing data types needs to be amended concerning the following problems and suggestion.

The IP_MOBILITY_MGMT data type contains redundant information of configuring the IP address which is already defined through the IP_CONFIG_METHODS data type.

The IP_MOBILITY_MGMT data type needs to allocate bits for Proxy Mobile IPv4, Proxy Mobile IPv6, Mobile IPv4 Regional Registration and Hierarchical Mobile IPv6.

There needs a new data type for fast handover protocols like low latency handoffs and fast handover for Mobile IP.

2. Suggested Remedy

We suggest applying the following amendment for the Data types for IP configuration which is specified in the B.2.6.

- Data types for IP configuration

| Type Name | Derived From | Definition | Valid Range |
| --- | --- | --- | --- |
| IP_CONFIG_METHODS | BITMAP(32) | A set of IP configuration methods. | Bit 0: IPv4 static configuration Bit 1: IPv4 dynamic configuration (DHCPv4) Bit 2: Mobile IPv4 with foreign agent (FA) care of address (CoA) (FA-CoA) Bit 3: Mobile IPv4 without FA (Co-located CoA) Bits 4-10: reserved for IPv4 address configurations Bit 11: IPv6 stateless address configuration Bit 12: IPv6 stateful address configuration (DHCPv6) Bit 13: IPv6 manual configuration Bits 14-31: Reserved. |
| IP_MOBILITY_MGMT | BITMAP(8) | Indicates the supported mobility management protocols. | 0: Mobile IPv4 with FA (FA-CoA) 1: Mobile IPv4 without FA (Co-located CoA) 2: Mobile IPv6 3: Mobile IPv6 with DHCPv6 4-7: (Reserved) 0: Mobile IPv4 1: Mobile IPv4 Regional Registration 2: Mobile IPv6 3: Hierarchical Mobile IPv6 4: Low Latency Handoffs 5: Fast Handovers for Mobile IPv6 6: IKEv2 Mobility and Multihoming Protocol 7 (Reserved) |
| IP_CONFIG_STATUS | BITMAP(8) | Status of the IP configuration methods. | Bit 0: IP configuration Method is not available Bit 1: DHCP Server address is not available Bit 2: FA address is not available Bit 3: Access Router Address is not available Bit 4: No information is provided due to accessibility of same entity (FA, Access Router, DHCP Server, etc.) Bit 5-7: Reserved |
| IP_PREFIX_LEN | UNSIGNED_INT(1) | The length of an IP subnet prefix. | 0 - 32 for IPv4 subnet. 0 - 128 for IPv6 subnet. |
| IP_SUBNET_INFO | SEQUENCE( IP_PREFIX_LEN, IP_ADDRESS) | Represent an IP subnet. The IP_PREFIX_LEN contains the bit length of the prefix of the subnet to which the IP_ADDRESS belongs. | N/A |
| IP_RENEWAL_FLAG | BOOLEAN | Indicates whether MN’s IP address needs to changed or not. | True: Change required. False: Change not required. |

RFC number lists

[1] Mobile IPv4, RFC3344

[2]  Mobile IPv4 Regional Registration , RFC4857

[3]  Mobile IPv6, RFC3375

[4]  Hierarchical Mobile IPv6, RFC4140

[5]  Low Latency Handoffs, RFC4881

[6]  Fast Handovers for Mobile  IPv6, RFC4068

[7] IKEv2 Mobility and Multihoming Protocol, RFC4555