| Project | IEEE 802.21 MIHS <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | Parameters for Link_Get_Parameters primitive |  |
| DCN | 21-07-0303-00-0000 |  |
| Date Submitted | 2007-09-14 |  |
| Source(s) | Donald Baker | Yuu-Heng Alice Cheng |
| Re: | IEEE 802.21 Session #22 in Big Island |  |
| Abstract | Update parameters of primitive Link Get Parameters to be coherent with MIH Link Get Parameters. |  |
| Purpose | Update parameters on Link_API. |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that IEEE 802.21 may make this contribution public. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as stated in Section 6 of the IEEE-SA Standards Board bylaws <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and in Understanding Patent Issues During IEEE Standards Development http://standards.ieee.org/board/pat/faq.pdf |  |

# Introduction

Link_Get_Parameters primitive  uses  LINK_PARAM_TYPE that returns LINK_PARAM_STAUTS. [Bottom part of page 75]

MIH_Get_Link_Parameters primitive uses the LINK_STATUS_REQ for retrieving link information that is provided in the LIST(LINK_ID)  [top part of page 99]. The data structure for LINK_STATUS_REQ is as following:

SEQUENCE(

CHOICE(LINK_STATES_REQ,NULL),

CHOICE(LIST(LINK_PARAM_TYPE),NULL),

CHOICE(LINK_DESCRIPTORS_REQ,NULL)

)

# Problem

Where should MIHF query for the LINK_STATES and LINK_DESCRIPTORS if not through the Link_Get_Parameters primitive?

# Proposed Change

Add the LINK_STATES_REQ and LINK_DESCRIPTOR_REQ to parameters of Link_Get_Parameters.request primitive.

Add the LINK_STATES_RSP and LINK_DESCRIPTORS_RSP to parameters of Link_Get_Parameters.confirm primitive.